# Introduction

The SGS survey package is designed for use with the instrument communication packages by Mike Morris for Topcon and Sokkia instruments which can be found at the website http://www.hpcalc.org/hp48/utils/comms/

They were written while I was studing at college and have not had ideal field testing.  If there is a bug that you find, please try to fix it yourself, I’m a very busy man, but if you cannot find the error, email me at nixon202@yahoo.com.

The programs are free to use and distribute. I was annoyed that these sort of things weren’t available on the net when I first started out, so I wouldn’t think of charging money for them. I can take no responsibility for the use of these programs or any subsequent claims arising from their use. The software is provided “as-is” without warranty of any kind. Should the software prove to be defective you assume the full cost of any repairs or corrections necessary.  If you really feel it is necessary to pay me, however, I like beer.

# Installing SGS

1.Start kemit from sgs location

2.Connect the calculator to computer

3. Type in conncetion settings (set po 1 [enter] set ba 9 [enter])

4. Type in send QCLOSE

If you are using Richard Stanaways Quickclose program, the Directory QCLOSE will already exist on your home menu.  Instead load the SGS.txt and SGS files directly into the QCLOSE folder.

To run press QCLOSE button and then SGS button

Ok, so we will start a new job

Scroll through the commands until you find NEW JOB

Hit enter and type the name of your new job (I am using SMG)

# STORE POINTS

1.Press SGS and go to the menu STORE POINTS

2.Type in the easting and press down

3. Type in the northing and press down

4. Type in the point number and press enter

5. continue to do the above to store more points, pressing enter with no data exits.

Note: the program will error and return to the main menu if the point already exists.  Points cannot be overwritten.

# GRIDLINES

OK now lets use the main feature which is the Gridlines.  Gridlines if you have never heard of them before are generally used in construction of large buildings.  They are similar to alignments, but I do not use chainages and offsets.

1.Lets say this is our job and we need to set out the two corners dimensioned.  Firstly we need to add some grids to our job.  So scroll down to NEW GRIDS, press ENTER then type in the easting and northing (note: not northing and easting), these they refer to the bottom left corner of the grid system (14-AH).  We will assume the co-ords 5030,1030.

Now it asks for the bearing running up the page, could be 0° but in this case, 244°45’29”.  Type it in as 244.4529.

Now we need the grids running off the origin, up down being 14, left right being AH

Note: the grids cannot have the names A, D, PLG, GBRG or PN, these are permanent program variables and odd things might happen if they are used

Next we put the grids in from left to right, distance 8.5 name 15, distance 8 name 16, and so on and so forth for all the grids

Hit enter with nothing in typed in to move to the next section.

Next we put the grids from bottom to top, distance 8 name AG, and so on and so forth for all the grids

And all grids are inputed.

2. Now we need to put in the two points.

Select STORE PT O/S

Type in the Grid offset.

We want AH and an offset of 2.635, type the number and then press enter then move the box to the section you next want to add a number or grid to and start typing again.

We also want 15 and an offset of 2.4, but because the point is to the left of the gridline we need to input it as a minus number, ie –2.4

So we have entered

GRID       AH

OFFSET  2.635

GRID       15

OFFSET –2.4

Pressing enter twice will go to the next section which lists the offsets and co-ordinates

Press any key to store the point.  If the point already exists you will have to try again.

We will store the point as point 2.

For the second point we only need to change one offset which is –2.4, -2.75 (added 0.35) so we simply scroll down to that offset and type in the changed number. And press enter twice again.  You can continue to do this for as long as you want.  Hitting the cancel key exits.

There are quite a number of other functions available in this program.

I will give an outline on how to use them in the order that they appear in the menu.

# SET OUT POINTS

This function sets out points previously stored.

1.Press SGS and go to the menu SET OUT POINTS

2. Type in the station you are set up on

3.Type in the point you wish to set out

4. Turn to the show angle

4.Depending on what set out instrument you are using either press enter or cancel

5.If you press enter it will get some information from the connected jigger if not enter a angle (azimith) and a distance (probably the ones on your jigger)

6. The hp will show a distance to or away and an angle error left or right if you are not on the right angle.

7. Press enter or cancel to get another shot until you are happy with the marks position.

8. Press cancel on the to/away screen to display field co-ords and your setout error the n press enter.

9. Press enter again to choose another point to setout.

10. To exit keep pressing cancel until you return to the main menu

Note: the program records the setout and field co-ordinates and their errors in a file called PLG in the jobs folder.  This is for the purpose of quality control, it also mean you know exactly where each co-ordinate is in the field, which could be handy when trying to re-establish yourself when your control is lost.

# SET LINE O/S

This function is used for setting out an offset to a grid line.  See above for gridline.  It is handy for putting marks on line on a wall

- Press SGS and go to the menu SET LINE O/S

- Type in the station you are set up on

- Type in the grid and it’s offset that you want to mark then enter

- In this format, it differs from the point setout due to the fact the A, B and C keys control what you are doing. Press enter to get info from the jigger, or C or the ME button to enter the distance manually.

- The next screen display the distance from the offset line to the offset point.  It is left or right of the line due to the minus or plus and the left and right is taken from the origin of the line, probably the bottom or left.

- Press A to enter a new grid and offset or,

- Press B to store the point for later use, or

- Press C or Enter to take another shot to the mark.

- Pressing stop exits and returns you to the main menu

# STORE POINTS

# CHECK BACKSIGHT

This tool is simply to get check your backsight.

- Press SGS and go to the menu BACSIGHT

- Type in the station you are set up on and press enter

- Type in the backsight point and press enter

- Press enter or cancel to get shot info

- The page wil display backsight errors and the per original co-ordinate. Press enter

- The next page displays the azimith to the backsight.  Press enter to set this bearing on the jigger or cancel to exit.

Note: the backsight errors will be recorded in the PLG file for quality control purposes.

# 3 PT RESECTION

This is a 3 pt resection function, which uses standard 3 point resection calcs.  Note: this does not use least squares to calculate a position.

- Press SGS and go to the menu 3 PT RESECTION

- Line up the first target and press enter or cancel for manual entry

- Enter the point sighting to and if cancel was pressed the azimuth and distance

- Line up second and third targets repeating steps 2 and 3

- You will then get a distance difference to the points, press enter

- Then azimiths to the points

- Then you can store the point

# RADIATE POINTS

This function is for picking up points for whatever reason, to add to you co-ordinate file.  It also writes them to the PLG file with heights if you require to do a primitive detail.

- Press SGS and go to the menu RADIATE PTS

- Type in the station you are set up on, and if required the station RL and instrument height. Press enter

- Press enter or cancel to get jigger information or manually enter

- Type in the point number, and if required the pole height (PLHT) and code.

Note: If you are stringing code the number must go after the code not before ie. POINT01 not 01POINT, due to the processing of this in the hp.

If you are using manual entry the height is entered in the PLG as N/A.

# TRAVERSE

This is to do a traverse without using a jigger, so that you can draw in a boundary or something similar.

- Press SGS and go to the menu TRAVERSE

- Type in the start point

- Type in a bearing and distance

- Pressing the LBRG button in the bearing input will use the last bearing, the same applies for the last distance, and LB90 and LB-90 will use the last bearing plus or minus 90.

# ENQUIRE OS GRID

This is getting offset from a grid to a point, in particular the WAE’s on lifts and stairs.

- Press SGS and go to the menu ENQUIRE OS GRID

- Type in the two grids you wish to offset from, press enter

- Type in a list of numbers you wish to see offsets from, press enter

- The offsets will be display, press enter to scroll through the points.

# NEW DAY

This function add daily info to the PLG file

- Press SGS and go to the menu NEW DAY

- Type in the job, instrument, surveyor, desc, then if you scroll down, you can set the time and date.  This is automatically got from the calculator, so provide it’s set right this should be correct.

# LSQ RESECTION

This function does a least squared resection, using only angles.  It does not check distances like the 3 point and is really designed for trig work, but it does the job all the same.  It is a bit long winded, but I’m to lazy to make it work better.

- You need to have a approximate co-ordinate already stored, you may be able to use the normal 3 pt resection to get this point

- Press SGS and go to the menu LSQ RESECTION

- Type in your setup point, the approximate co-ordinate point

- Type in the target you are sighting to and the bearing you read to it and press enter

- The program will display the A,B, and C.  The C is important, it is the variance of the bearing in seconds, if it gets over around 100,000 there is something very worng with you data or my programming.

- Type in the next target and the bearing to it, and the A,B, and C’s  are displayed.

- When you are finished press enter on the next target screen.

- The program will then give you the adjustments and co-ordinates of the adjusted point. It will also display SUGGEST RERUN if the adjustments are more than 0.1, this means run the program again with the new adjusted co-ordinates.

- Press enter  and store the resected point as a new number or overwrite the existing on.

- The program will then give you a matrix with the residuals, and probably the word matrix written underneath it.  This was so I could read it with an advance matrix writer, which are available at the HPCALC website, and I strongly recommend using.

# CHOOSE JOB

This function change the current job.  Just press the button and type in the job you wish to change to.

# APPENDIX

When changing from Sokkia to Topcon instruments press the 2 and go into the respective folder, then press setup.  Don’t delete the INSTP file from the home directory as this is what the program runs off.