XLINE  HP48 PROGRAM

XLINE  (LINE with EXtended Capabilities)  ver 1.6  by  Victor Maletic

This program calculates a straight line between two points a specified distance from each other and of specified heights or elevations.

By specifying a ± mid ordinate value the line will take on a parabolic curvature.  The program extends the straight or parabolically curved line beyond its originally defined end points (P1 and P2) and allows you to computed elevations or heights at locations outside the portion bounded by P1 and P2.

If actual P2 (asbuilt) is displaced from its intended position, a linear or a reversing curvilinear correction can be incorporated into the computed elevations by inputting a ± max correction value and selecting a type (RCC-reversing curve correction or SLC-straight line correction).

If an intended line between P1 and P2 is intercepted by an obstruction - such as an existing utility line in the path of a proposed drainage line - a bell shaped correction (BCC-bell curve correction) can be included in the computed elevations to provide a smooth detouring path over or under the utility.

Applied corrections can be cleared at any time or over written with new values.  Pressing the (DATA) key will give complete listing of all active parameters.  Press the (RESET) key on the last menu to clear all inputs to zero to work on another line problem unrelated the previous one.

Legend:	(   )  Redefinable keys (top row) - labeled by program.

[   ]  Dedicated keys - normal built in functions.

[<┐]  [┌>]  Left/Right Shift keys

" Soft key text "  To see press [<┐][REVIEW] - To return press [ATTN]

Press [NXT] or [<┐][NXT] to change menus.

Menu layout

──────────────────────────────────────────────────────────────────────────────────

Menu 1: ( P1 )   ( P2 )   (  L )    (  C )    (QUIT )    (  X  )

Menu 2: (  K )   (SLC )   (RCC )    (BCC )    (CLCOR)    ( DATA)

Menu 3: (RESET)

Key Definitions

──────────────────────────────────────────────────────────────────────────────────

Menu 1:

( P1 )	"Elev1 or H1"  Inputs elevation or height at point 1.

( P2 )	"Elev2 or H2"  Inputs elevation or height at point 2.

(  L )	"between P1 P2"  Inputs distance between P1 and P2.

(  C )	"±vert curv ord"  Inputs mid-ordinate value for vertical curvature.  At start up 'C' defaults to zero.  Flag 1 annunciator turns on if 'C' ≠ 0.

(QUIT)	"QUIT program"  Removes local vars, pointers and frees memory.

(  X  )	"along line"  Key in distance along the line then press this key for computed elevation or height at X.

Menu 2:

(  K  )	"±Constant"  Inputs constant value.  Subsequent elevation or height values will incorporate the constant.  Flag 2 annunciator 		will 		turn on at top of display.  A zero 'K' value will clear Flag 2.

(SLC )	"±Str Line Corr"  Inputs a straight-line-correction to be applied linearly to calculated elevations/heights between P1 and P2.  Flag 		3 annunciator will turn on if correction value ≠ 0.

( RCC )	"±Rev Curv Corr"  Inputs a reverse-curve-correction.  A reversing curve correction will be applied to calculated elevations/heights between P1 and P2.  Flag 4 annunciator will turn on if correction value ≠ 0.

( BCC)	"±Bell Curv Corr"  Inputs a bell-curve-correction.  A bell shaped curve correction will be applied to calculated elevations/heights between P1 and P2.  Flag 5 annunciator will turn on if correction value ≠ 0.

(CLCOR)	"CLcorrection"  Press this key to cancel application of correction to computed values between P1 and P2.  Flags 3, 4 or 5 will turn off.

(DATA)	"Data summary"  Pressing this key will list the following information depending upon which parameters have been activated.

P1:     Input P1 elevation/height

P2:     Input P2 elevation/height

L:      Input L length

c:      Input vertical curve ordinate

k:      Input additive constant

Slope:  From P1 to P2

[0.00 n2 n3 ...0.00]  A 9pt array of correction values at L/8 intervals

Max correction value and type (SLC, RCC, BCC)

Menu 3:

(RESET)	"loc vars to zero"  This key clears all inputs to zero so that you may work on a new line problem without exiting the program.

Conventions / Notes

──────────────────────────────────────────────────────────────────────────────────

Distance are positive if directed from P1 towards P2.  P1 is the zero point.  Negative distances are directed from P1 away from P2.

Line correction types SLC and BCC assume P1 value is correct and correction values accrue to a ± maximum at P2.

To review accumulated data (assuming the stack was not cleared), enter interactive stack mode by pressing [▴].  Press [ATTN] to exit the interactive stack.

ATTENTION! If you inadvertently press [<┐][LIBRARY] or another key combination that takes you out of the program immediately press [┌>][MENU] to get back to the program menu.  If you go too far astray and can't get back, press [<┐][CONT] to clear the HALT annunciator and restore memory.   Then press [VAR] and try XLINE again.

If erroneous data was entered, key in correct data and press the proper key.  The new data will replace the old.  Internal variables affected by the corrected data will be recomputed.

If you get insufficient memory type messages while using this program, look up "LowMemory" in the owner's manual for mitigation measures.

Example Applications

──────────────────────────────────────────────────────────────────────────────────

1.)	The end post lengths for the falsework bent shown below were derived by subtracting field elevations from plan elevations and deducting various structural thicknesses.  Calculate intermediate post lengths at the locations shown.

Input and/or Press		Results and /or Remarks

(XLINE) if you quit program or (RESET) to reset for this example

12.92( P1  )			input P1

12   ( P2  )			input P2

45.5 (  L  )			input L

0	(  X  )     		12.92 check

45.5	(  X  )			12.00 check

6.75	(  X  )			12.78 first interior post length

14.75(  X  )			12.62 next post length

22.75(  X  )			12.46 next post length

30.75(  K  )			12.30 next post length

38.75(  X  )			12.14 next post length

2.)	The soffit forms for a bridge stem and soffit pour are to be adjusted to plan elevation.  The large crosses represent the points on the soffit that require adjustment.  The cross locations are located to be accessible at all times during construction of the bridge and are not directly over the jack post.  Convert the soffit CUTS/FILLS to equivalent jack post CUTS/FILLS.

Input and/or Press		Results and /or Remarks

(XLINE) if you quit program or (RESET) to reset for this example

.09[±]( P1  )			input P1

.04 ( P2  )			input P2

14.4	(  L  )			input L

0	(  X  )     		-0.09  check

14.4	(  X  )			 0.04  check

6.7[±](  X  )			-0.15 Cut .15' at jacking post E

7	(  X  )			-0.03 Cut .03' at jacking post C

23	(  X  )			 0.12 Fill .12' at jacking post A

3.)	A concrete slab is to be constructed upon which a precast post-tensioned inverted trapezoidal girder is to be formed and cast.  Planned girder length is 80' with a total parabolic rise at center of 0.35'. (This includes span vertical curve and girder camber.)  The field where the first casting slab is to be constructed is not level and cannot be made so because of restrictions.  Also a rise or bump is noted at the 50 ft location of the form with an elevation of 7.31'.

Input and/or Press		Results and /or Remarks

(XLINE) if you quit program or (RESET) to reset for this example

6.17	( P1  )			input P1

7.39 ( P2  )			input P2

80	(  L  )			input L

.35  (  C  ) 			input C

50	(  X  )			7.26 required but exist elev is 7.31 therefore a

K value of .05 (say .06) to raise whole profile

[NXT]		to clear bump and not require any excavation.

.06	(  K  )[<┐][NXT]	input K to raise forms and avoid excavating bump.

0	(  X  )			6.230 .06 - => 6.17  check

80	(  X  )			7.450 .06 - => 7.39  check

5[±]	(  X  )			6.061  top form elev

10	(  X  )			6.536  top form elev

●	   ●				   ●

85	(  X  )			7.433  top from elev

Flag 1 annunciator lit:	For 0.35 parabolic curvature applied.

Flag 2 annunciator lit:	For 0.06 constant applied.

4.)	Camber strips for uniformly loaded falsework beams spanning 38' have a maximum mid-span ordinate of 1.75".  The contractor intends to assemble the camber strips from 8' 2x4's symmetrically with 3' end lengths.  Calculate end heights for the individual lengths of 2x4's comprising a camber strip.  Calculate the end heights, but include a 1.5" sleeper thickness.  For practical purposes camber strips up to 3" high can be considered parabolic ( less than 1/8" error at the quarter span).

Input and/or Press		Results and /or Remarks

(XLINE) if you quit program or (RESET) to reset for this example

0	( P1  )			input P1

0  ( P2  )			input P2

38	(  L  )			input L

1.75 (  C  ) 			input C

0	(  X  )     		0.00  check

38	(  X  )			0.00  check

38 [SPC] 2 [÷](  X  )	1.75  mid-point check

3	(  X  )			0.51  say 0.00" to 0.50" for first section

11	(  X  )			1.44  say 0.50" to 1.50" for second section

19	(  X  )			1.75  say 1.50" to 1.75" for third section

Use same end heights for last half of strip

[NXT] 		move to next menu

1.5	(  K  )[<┐][NXT]	input sleeper thickness, back to previous menu

0	(  X  )			1.50  check

3 	(  X  )			2.01  say 1.50" to 2.00" for first section

11	(  X  )			2.94  say 2.00" to 3.00" for second section

19	(  X  )			3.25  say 3.00" to 3.25" for third section

Use same end heights for last half of strip

Flag 1 annunciator lit:	Parabolic curvature applied.

Flag 2 annunciator lit:	If constant 1.5 was applied.

5.)	A final 72' section of retaining wall is to be constructed between two previously built sections.  Plan elevations for the beginning of final section is 73.15' and 70.99 at it end.  However, because of error caused mostly as a result of total station trigonometric leveling, the end of the last wall section must be lowered 0.36' to meet and tie in with the existing wall.  Apply a reverse curve correction to the top of the wall elevations and compute the revised elevations at 8 foot intervals (to coincide with form panel joints).

Input and/or Press		Results and/or Remarks

(XLINE)			start program

73.15 ( P1  )			input P1

70.99 ( P2  )			input P2

72    (  L  )[NXT]		input L,  move to next menu

.36[±]( RCC )[<┐][NXT]	input negative Rev-Curv-Corr value,  Flag 4 on,  to prev menu

0	(  X  )			73.15  top wall elev check

8	(  X  )			72.90  top wall elev

16	(  X  )			72.63  top wall elev

●	   ●				  ●	  top wall elevs

64	(  X  )			70.88  top wall elev

72	(  X  )			70.63   0.36 + => 70.99 check

(QUIT )			To quit program and free memory

or	(RESET)			To set all vars and flags to zero and do another example.

Flag 4 annunciator lit:	RCC correction applied.

To review accumulated data (assuming the stack was not cleared), enter interactive stack mode by pressing [▴].  Press [ATTN] to exit the interactive stack.

6.)	While excavating for a storm drain, the contractor encountered a utility line.  The utility line was intercepted at sta. 115+32±.  Adjust the profile of the storm drain flow line to pass under the utility line.  Start at 200' before the intercept point, and end the adjustment 200' after it.  At sta. 113+32 the plan flow line elevation is 136.14 and at sta 117+32 it is 112.14.  Flow line to top of pipe is 1.75'.  The flow line must be lowered 3.45' at the intercept point to adequately pass under the utility line.  Calculate revised flow line grades for the excavation and top of pipe at 20' intervals.  Apply negative bell curve correction (BCC) to the pipe profile to affect a smooth flow profile.

Input and/or Press		Results and /or Remarks

(XLINE) if you quit program or (RESET) to reset for this example

136.14 ( P1  )			input P1

112.14 ( P2  )			input P2

400	(  L  )[NXT]		input L,  move to menu 2

3.45[±]( BCC )[<┐][[NXT]	input negative Bell-Curv-Corr value, on to menu 1

0	(  X  )			136.14	check

20	(  X  )			134.79 	new fl elev

40	(  X  )			133.35 	new fl elev

●	   ●				   ●

380	(  X  )			113.19 	new fl elev

400	(  X  )			112.14 	check

For top of pipe elevations press [NXT] to move to menu 2.

1.75	(  K  )[<┐][NXT]	input pos K,  Flag 2 on,  move to prev menu

0	(  X  )			137.89 	1.75 - => 136.14 check

20	(  X  )			136.54	top of pipe elev

●	   ●				   ●

380	(  X  )			114.94	top of pipe elev

400	(  X  )			113.89	1.75 - => 112.14 check

(QUIT )			To quit program and free memory

or	(RESET)			To set all vars and flags to zero and do another example.

Flag 5 annunciator lit:	BCC correction applied.

Flag 2 annunciator lit:	For 1.75 applied constant.

To review accumulated data (assuming the stack was not cleared), enter interactive stack mode by pressing [▴].  Press [ATTN] to exit the interactive stack.