Partial Documentation	PLINESX or (GX)  HP48 PROGRAM

PLINESX (GX) (Profile LINE for SX or GX)  ver 1.2  by  Victor Maletic  4-14-98

This program calculates elevations along three types of profile lines.  The profiles are inputted and saved as data files that can be retrieved anytime elevations are required from it.  After profile data has been entered and editing option is available so that the input my be checked and corrected if necessary before it is filed.  Additional features:

● Addition of a positive or negative constant value to computed elevation.

● Calculate elevations at specified intervals from a specified starting station.

● Station range checking.  Will not calculate elevations beyond profile limits.

● Profile data files are stored in port 0 to reduce accidental erasure.

Type 1 Profiles: Any combination of tangents and vertical curves.  Typically used for highway and top of retaining wall profiles.  The vertical curves are parabolic.  This type is also used for tops/bottoms of stepped walls/footings profiles.

Type 2 Profiles:  A series of elevations positioned at equal intervals (horizontal).  Often used for flow lines, ground lines, rail profiles, etc. that cannot be described with a simple formulae.

Type 3 Profiles:  A series of elevations at un-equal intervals (horizontal).  Often used to described top of retaining wall profiles.

CONVENTIONS / NOTES

──────────────────────────────────────────────────────────────────────────────────

The distinction between program keys and built-in is described in the legend below.

Legend:	(   )  Soft keys (top row) - labeled by program.

[   ]  Dedicated keys - normal HP built-in functions.

[<┐]  [┌>]  Left/Right Shift keys

"Soft key long label"  To review on:

HP48SX press [<┐][REVIEW] - To return press [ATTN]

HP48GX press [┌>][VIEW]   - To return press [CANCEL]

The PLINE program follows HP's convention of entering values first and then pressing the key for the operation to be done on them with three exceptions.  An example:  To compute an elevation for a point on the profile, key in the decimal station for that point and press (STA) key and see the labeled results.

Exceptions:

(?K)  A question mark placed in front of the K signifies that you press this key to see the current value of K.  To return to the Profile Menu press [ENTER].  To revise K, key in its new value then press [ENTER].

(?INTE)  A question mark placed in front of the INTE signifies that you press this key to see the current setup for interval calculations.  To return to the Profile Menu press [ENTER]. To revise the interval data, key in its new starting station value then press [▾], key in the interval at which elevations will be computed.  Press [ENTER] when you are done to return to the Profile Menu.

(→NEX)  Calculates the elevations at the intervals specified from the starting station specified with the (?INTE) key.  Each time you press (→NEX) the next interval station and elevation are computed and displayed.

To review accumulated data (assuming the stack was not cleared), enter interactive stack mode by pressing [▴].  Press [ATTN] on the 48SX or [CANCEL] on the 48GX to exit the interactive stack.

ATTENTION! If you inadvertently press [<┐][LIBRARY] or another key combination that takes you out of the program, immediately press [┌>][MENU] to get back to the program menu.  If you go too far astray and can't get back, press [<┐][CONT] to clear the HALT annunciator and restore memory.   Then press [VAR] and try PLINE again.

If you get insufficient memory type messages while using this program, look up "LowMemory" in the owner's manual for mitigation measures.

MENU KEY DESCRIPTIONS

──────────────────────────────────────────────────────────────────────────────────

File Menu:

( GET ) "Get Profile File"  Used to choose a profile to work with.

(INPUT) "Input a Profile"  Used to input data describing a profile.

INPUT Sub Menu:

(SAVE) "Save Profile"  Press this key to save the profile date under a file   name.

(EDIT) "Edit Profile"  Use this key to review/correct data before saving.

(EXIT) "Exit to Prev menu"  Press to return to previous menu.

(PURG) "Purge File/s"  To purge file/s no longer needed press soft key for each file to be purged then press [ENTER].

(QUIT) "QUIT"  Press to quit program and return to VAR menu.

Profile Menu:

( RND) "RND Elevs n Decimals" Key in the number of decimal places (1-4) then press this key.  For 3 or less, the least significant places are padded with zeros.

( ?K ) "±Const Rev/Revise"  Inputs constant value.  Subsequent elevation or height values will incorporate the constant.  Flag 1 annunciator 		will 		turn on at top of display to remind you a constant is being applied.  A zero 'K' value will clear Flag 1.  The ? mark indicated you are to press the key first, review or update then press [ENTER] to continue.

(EXIT) "Exit to Prev menu"  Press to return to previous menu.

(?INTE) "Interval elev setup"  Press this key, then key in the interval starting station and interval distance (feet).  Press [▾] to move to lower line.

( →NEX ) "Next interval Elev"  Press this key repeatedly to obtain first and subsequent interval elevations along profile.  This can be interrupted at any time to use the ( STA ) key.

( STA ) "Sta along Profile"  Key in decimal station (Sta 123+15.5 is 123.155).

Press this key to obtain corresponding elevation.

Selecting a Tan/VC profile:

When starting Pline you are presented with the File Menu and a prompt to pick an option.  Press (INPUT).

You are then prompted to choose a profile type:  Press [1]

Note: Ignore the GET INPUT PURG and QUIT soft key labels at the bottom of the display.  Follow the prompts at the top of the display.

Inputting Tan/VC Profile data:

You are prompted to enter the number of segments for the profile.  Each distinct portion of tangent or vertical curve is a segment.  I recommend that you pencil in the number of each segment as you count them off.  In the example below you would key in 4 and press [ENTER].

You are then presented with a fill in template for segment number 1 with the cursor next to begin station.

Input Seg No. 1

:BegSta:   Key in a decimal station then press [▾]

:BegEl:    Key in the elevation for this station then press [▾]

:G1 G2:    Key in G1 as a percent [SPC] G2 as percent then press [ENTER]

Note:  Before pressing [ENTER] you may use the arrow keys to move the cursor around and correct any errors made.

Fill in the data for the rest of the segments as prompted for sequentially.  For tangent sections G1 and G2 are both required.  They will have the same value and sign.  After the last segment is entered the end station prompt will be displayed.

End Sta Seg No. X

Key in a decimal station then press [ENTER]

Next the data file for the profile will be shown in the display

1:	{ "H" data

data data

data data

data data

The Input submenu is activated and relabels the soft keys with the following selections:

(SAVE)    (EDIT)    (EXIT)

If you are sure that all of the data for the profile is correct press (SAVE) and proceed with the naming and saving of the data file.

I recommend that you press (EDIT) to sequentially review all of the input data.  The first screen appears with the cursor at the station value.  Review the data for correctness.  Use the arrow keys to move to the incorrect information and rekey in the correct information.  Press [ENTER] when you are done.  If no corrections were needed just press [ENTER] to proceed to the next screen.

Edit Seg 1 then ENTER

:Sta: data

:Elev: data

:G1 G2: ±data ±data

Check the data for the rest of the segments as prompted for sequentially.  After the last segment is displayed for editing, the ending station will be displayed.

:End Sta:    Edit if needed then press [ENTER]

Note:  You must edit the whole file.  You cannot stop short and go to (SAVE).

The screen redisplays the data file and the Input submenu is active allowing the data file to be saved.

1:	{ "H" data

data data

data data

data data

Press (SAVE)   The calculator prompts you as follows with the cursor next to the port 0 designator.  Also the alpha annunciator is lit indicating that the apha keys are active.

Enter File Name

:0: Type in a name  then press [ENTER].

The display briefly displays the message FILED and then returns you to the File Menu.

I recommend short easy to recognize names.

Note:  You may type in a twenty character name but only the first four or five of the letters will be visible in the soft label.  To view the full name you must exit Pline then follow the instructions below:

HP48SX press [<┐][REVIEW] - To return press [ATTN]

HP48GX press [┌>][VIEW]   - To return press [CANCEL]

Selecting an Equal Chords profile:

When starting Pline you are presented with the File Menu and a prompt to pick an option.  Press (INPUT).

At the prompt to choose a profile type press [2]

Inputting Equal Chords Profile data:

Prior to inputting an equal chords profile I recommend that you pencil in the nodes as shown below to aid in keeping track of the input:

You are prompted with a fill in template for the data:

Input All: then ENTER

:BegSta:   Key in a decimal station then press [▾]

:EndSta:   Key in a decimal station then press [▾]

:InvSta:   Key in an interval in decimal stations (50' interval would be input as 			.5) then press [ENTER].

Note:  Before pressing [ENTER] you may use the arrow keys to move the cursor around and correct any errors made.

You will be prompted sequentially for the elevation at each station:

Sta X

Elev   Key in the elevation corresponding to the above station then press [ENTER].

Fill in the data for the rest of the stations as prompted for sequentially.

Next the data file for the profile will be shown in the display

1:	{ "E" data

data data

data data

data data

The Input submenu is activated and relabels the soft keys with the following selections:

(SAVE)    (EDIT)    (EXIT)

If you are sure that all of the data for the profile is correct press (SAVE) and proceed with the naming and saving of the data file.

I recommend that you press (EDIT) to sequentially review all of the input data.  The first screen appears with the first station and with the cursor at the elevation value.  Review the data for correctness.  Use the left/right arrow keys to move to the incorrect information and rekey in the correct information.  Press [ENTER] when you are done.  If no corrections were needed just press [ENTER] to proceed to the next screen.

Edit El @ Sta X

then ENTER

:Elev: data     Edit if needed then press [ENTER]

Check the data for the rest of the stations as prompted for sequentially.

Note:  You must edit the whole file.  You cannot stop short and go to (SAVE).

The screen redisplays the data file and the Input submenu is active allowing the data file to be saved.

1:	{ "E" data

data data

data data

data data

Press (SAVE)   The calculator prompts you as follows with the cursor next to the port 0 designator.  Also the alpha annunciator is lit indicating that the apha keys are active.

Enter File Name

:0: Type in a name  then press [ENTER].

The display briefly displays the message FILED and then returns you to the File Menu.

I recommend short easy to recognize names.

Note:  You may type in a twenty character name but only the first four or five of the letters will be visible in the soft label.  To view the full name you must exit Pline and follow the instructions below:

HP48SX press [<┐][REVIEW] - To return press [ATTN]

HP48GX press [┌>][VIEW]   - To return press [CANCEL]

Selecting a Not Equal Chords profile:

When starting Pline you are presented with the File Menu and a prompt to pick an option.  Press (Input).

At the prompt to choose a profile type press [3]

Inputting a Not Equal Chords Profile:

Prior to inputting an equal chords profile I recommend that you pencil in the nodes as shown below to aid in keeping track of the data:

You are prompted with a fill in template for the data:

Enter No of Nodes

X	Key in the number of nodes.  Each node is a point on the profile defined 		with	a station and its elevation.

Note:  Before pressing [ENTER] you may use the arrow keys to move the cursor left/right to correct any errors made.

You will be prompted sequentially for the station and elevation at each node:

Input Node No. 1

:Sta:	Key in a decimal station then press [▾]

:Elev:	Key in the elevation corresponding to the above station then press

[ENTER].

Fill in the data for the rest of the nodes as prompted for sequentially.

Next the data file for the profile will be shown in the display

1:	{ "N" data

data data

data data

data data

Inputting a Non Equal Chords profile:  continued

The Input submenu is activated and relabels the soft keys with the following selections:

(SAVE)    (EDIT)    (EXIT)

If you are sure that all of the data for the profile is correct press (SAVE) and proceed with the naming and saving of the data file.

I recommend that you press (EDIT) to sequentially review all of the input data.  The first screen appears with the first node and with the cursor at the station value.  Review the data for correctness.  Use the arrow keys to move to the incorrect information and rekey in the correct information.  Press [ENTER] when you are done.  If no corrections were needed just press [ENTER] to proceed to the next screen.

Edit Node 1 then ENTER

:Sta:	data     Edit if needed

:Elev:	data     Edit if needed   Press [ENTER] when done

Check the data for the rest of the nodes as prompted for sequentially.

Note:  You must edit the whole file.  You cannot stop short and go to (SAVE).

The screen redisplays the data file and the Input submenu is active allowing the data file to be saved.

1:	{ "N" data

data data

data data

data data

Press (SAVE)   The calculator prompts you as follows with the cursor next to the port 0 designator.  Also the alpha annunciator is lit indicating that the apha keys are active.

Enter File Name

:0: Type in a name  then press [ENTER].

The display briefly displays the message FILED and then returns you to the File Menu.

I recommend short easy to recognize names.

Note:  You may type in a twenty character name but only the first four or five of the letters will be visible in the soft label.  To view the full name you must  Pline then follow the instructions below:

HP48SX press [<┐][REVIEW] - To return press [ATTN]

HP48GX press [┌>][VIEW]   - To return press [CANCEL]