VERTICAL CURVE PROGRAM

 David O’Sullivan 1999

INSTALLATION

Copy the lib file to the HP-48 Calculator using one of the various PC compatible kits available. Install as you would any other Library.

OPTIONAL : Add the command VC to your CST variable.

USE

Start the Program by either entering VC onto the stack or alternatively selecting it from the CST menu.

VC

This is the main menu.

The following options are available

FILE     -  Data file management utilities. (ie. New, Load, Save, Edit, Delete)

STAT   -  Displays the vertical curve statistics of the current file.

CHR -  For a given chainage the program will return the level at that point and also the

instantaneous grade at the specified chainage.

RLC  -  For a given level the program will find all occasions of that  level along the

current vertical alignment and return a list of chainages.

EXIT     -  Leaves the program and displays the CST custom menu.

This is the file menu.

The following options are available

NEW        -  Allows the user to Enter a new file which will become current.

LOAD      -  Load a previously saved file.

SAVE       -  Save the current file under a specified name to the data directory.

EDIT        -  Edit the current file.

DELETE  -  Delete a previously saved file from the data directory.

EXIT        -  Returns the user to the main menu.

NEW

Upon selecting NEW the following input form is displayed.

Enter the total number of Intersection Points for the alignment. (Minimum of 3)

The program then displays an input form for each intersection point. When entry is complete, the user is returned to the file menu. The entered file is now the current file until either edited or another file loaded.

LOAD

Upon selecting LOAD a choose list is displayed showing all previously saved alignment files.

Highlight the required data file then press either ENTER or OK. The selected file will then become the current file. If CANCL is pressed the existing current file will remain unchanged.

SAVE

Upon selecting SAVE the following input form is displayed.

Enter any valid HP48 variable name inside “” (eg. “GUMTREE.AVE” ) and press OK. This will save the current file under that name in the VC data directory.

Note: 1. For valid variable names refer to the HP48 Series User’s Guide.

2. Using an existing variable name will be treated as invalid. To overwrite an

existing filename, delete it first and then save under that name.

EDIT

Upon selecting EDIT, the data for the 1st I.P. is displayed along with the edit menu.

This is the edit menu.

    - Steps the user backwards and forwards through the I.P.s of the current file.

INS      - Inserts a new I.P. into the alignment.

DEL     - Deletes the currently displayed I.P. from the alignment.

EDIT   - Edits the currently displayed I.P.

EXIT   - Returns the user to the file menu.

To insert a new I.P. press INS.

Input the new I.P. and press OK. The new I.P. will be added to the current file and the chainages sorted to ensure they are in the correct order.

To delete the currently displayed I.P. press DEL.

Caution should be exercised when deleting I.P.s as the program does not ask yes or no, but deletes it immediately. If only 3 I.P.s are left the program will not let you delete any of them.

To edit the currently displayed I.P. press EDIT.

An input form showing the current I.P. values will be displayed for editing. OK saves the changes and CANCL abandons the changes.

Editing Notes:  1. The I.P. identification numbers will change as I.P.s are added and

deleted.

2. The edited changes affect only the current file and will not affect saved

files. If you wish to keep the changes you should save it again.

3. Regardless of what V.C. length is used for the first and last I.P.s, they

will be regarded by the program as having zero length.

4. The program does not check for overlapping Vertical Curves. If two or

more V.C.s overlap each other, incorrect results will be produced.

DELETE

Upon selecting DEL a choose list is displayed showing all previously saved alignment files.

Highlight the data file you wish to delete, then press either ENTER or OK. The selected file will then be erased from the data directory. If CANCL is pressed no the command is aborted and no files will be erased.

DATA FILES

All the data files are kept in a sub-directory called VCDATDIR. This sub-directory is created by the program automatically. You can use the HP-48’s standard variable commands to copy, move etc. the files contained in this directory. However it is recommended that you don’t save other variables (non-VC data files) in  VCDATDIR as these will be detected by the File Load and Delete commands and may lead to confusion.

EXAMPLE

Here is an example showing how to enter a Vertical Curve alignment, save it for later use and how to obtain required information.

Consider the following alignment.

The alignment above is defined by the following I.P.s.

| CHAINAGE | R.L. | V.C. LENGTH |
| --- | --- | --- |
| 0.000 | 32.569 | 0.000 |
| 10.000 | 32.269 | 10.000 |
| 60.000 | 36.200 | 40.000 |
| 110.000 | 33.700 | 0.000 |
| 170.000 | 31.300 | 50.000 |
| 250.000 | 33.980 | 40.000 |
| 315.000 | 39.830 | 0.000 |

First start the Vertical Curve program

VC

Select the file submenu.

FILE

Select the new file option to enter the data

NEW

Fill out the input forms as shown.

Select EXIT to return to the main menu.

EXIT

To view the statistics for each individual curve press STAT.

Information displayed includes  - Length and radius of curve (if applicable)

- Entry and exit grades

- Curve tangent points

- High and low points (if applicable)

Use the arrows     to step through the information.

STAT

To find the level and instantaneous grade at a certain chainage, select CHR from the main menu and then type in the required chainage (say 151.587) and hit ENTER.

CHR

You can continue entering chainages as required, or just hit enter to return to the main menu.

To find all occurances of a particular level, select RLC from the main menu and then type in the required level (say 34.000) and hit ENTER.

RLC

You can continue entering levels as required, or just hit enter to return to the main menu.

Note: Searching for chainages for a particular level will take longer depending on

the number of I.P.s.

So far this example has been using the current file. If you wish to save it, then select File from the main menu followed by save. All the file utilities have been covered earlier.

DISCLAIMER

Considerable  effort has been undertaken to ensure that this program is error free. However if there is fault with the program, the author assumes no responsibility whatsoever.

 David O’Sullivan 1999

dmosull@powerup.com.au