Instructions for Vector Offset Program

- “QUADO” and “DIST” need to be downloaded into the same directory as the o.set program to run properly.

- Enter in the easting, northing and RL for the start of the design vector in the first window and the end of vector coordinate data in the second.

- Setup data in the third window

- Take a reading to a chosen point and enter in a SLOPE distance, bearing and zenith angle to the final window.

- A misclose message box will appear.  A negative misclose in the horizontal indicates that the reading taken was too far left of the design vector – in a perpendicular direction.  Left and right is assumed that you are facing in the direction of the vector.  A negative misclose in the vertical indicates the reading was too low.

- The readings window repeats itself until you arrive at an acceptable misclose.