PROGRAMING INFORMATION

Programs like COX and XLINE were written for the SX but will also run on the GX.

The more complicated programs using nested menue call have have GX and SX versions.  The underlying programming structures for SX and GX are the same, but because of different internal menu numbers the program code has to be tailored to the differences.  Also error message displays are tailored to the differences between the SX and GX key boards.

Programs are optimized for speed:

● Only local variables are used for internal operations and calculations.  Local    variables are accessed by programs much faster than global variables.

● All subroutines are generated at start up and become local variables.

● Wherever possible calculations utilize the stack rather than repeated local    variable calls.  Fetching values from the stack is even faster than            recalling local variables.  Algebraic expressions are not used - too slow.

● The HP's built-in STO+ and INCR functions are utilized where needed.

● Complex number manipulation is used in most the computations used in the TRAV program.  This eliminates the programming overhead of tracking and dealing with angular quantities.

The only global variable used by these programs are the names associated with a saved or retrieved data file.  The data file are automatically placed in PORT :0:.  Files saved to PORT :0: are not readily deleted by accident.

Some of the programs are too large to review or edit on the stack.  It takes approximately 20 minutes to decompile the TRAVGX program on the stack.  I used an IBM clone computer with Hewlett Packard's PDL program to create, edit and download this program to the HP48's.  To get around PDL's 400 line limit, I place as many commands

on a line as is practical.  The Windows based CalcWare is even more limited than PDL.  I also use the text editor found in WordPerfect or Word to write my programs.  These word processors are not limited to 400 lines but the special charactors used by the HP calculator require you to use the ASCII Charactor Translations.  These translations are discussed section 27 of the HP 48GX Users Guide.

For example:  « HMS→ » as seen on the HP display would be typed  \<< HMS\-> \>>  in WORD.

Instructions or manuals are available for most of these programs.  In some cases the ASCII code for the program includes a description of it various functions.

PROGRAMS:

ADDLAST (Add the Last level 1 obj to new level 1 obj) 31.5 bytes by V.A. Maletic

31.5 bytes

To add a positive or negative constant to a succession of numbers.

Example: To subtract .65 from 13.45, 13.49, 13.53, 13.56 etc.

Do this:

13.45 [ENTER] .65[±] [+] -> 12.80

13.49 |ADDLA|            -> 12.84

13.53 |ADDLA|            -> 12.88

13.56 |ADDLA|            -> 12.91

etc.

BSEAT (Bearing SEAT) by V. A. Maletic

3211.5 bytes

This program calculates corner grades for girder bearing seat areas.

The corner grades define a surface that is level along a normal to the

girder in the bearing area and simultaniously parallel to the gradient

of the deck surface.  It requires a contour map of the bridge deck.

Girder seats that are parallel to the deck gradient longitudinaly and

level along a perpendicular to the girders axis provide lateral sta-

bility and less restraint to longitudinal movement of the bridge.

Please note on curved bridges with skewed bents each bearing seat will

have its own skew angle value.  Using the plan skew is not sufficient.

Execute PIC in menu for geometry notations.

COX (Curve Offset at X distance) ver 1.5  by V.A. Maletic 11-08-92

4112.5 bytes

This program is a curve layout program with special utilities for

field use.

Curve layout can be calculated by the chord offset or tangent offset

method.  Also offset layout lines and crossfall corrections can be

be applied as needed.  User flag 1 indicates and offset line is in

use and being applied to results.  User flag 2 indicates a crossfall

correction is being applied to results.

KEY     USE

------------------------------------------------------------------------

CHORD    Input a keyed in or calculated chord

RAD      Input a keyed in or calculated radius

QUIT     Exit program, delete pointers and restore memory

XC       Input a distance from 0 along the chord line

XT       Input a distance from BC along the tangent line

For Chord Offset layout there are three utility keys:

CV/2     Eliminates working with large unwieldy offset distances

by dividing a curve into two parts with corresponding chords

and much smaller offset values for each.  To use the new chord

press the "CHORD" key on menu 1.  Offset values calculated for

the first half section of curve can be used for the last half.

MO->R     Calculates a radius that fits a given chord and midordinate.

To use the calculated radius press the "RAD" key on menu 1.

PIC      Provides a picture and sign conventions for chord layout of

curves.

For Tangent Offset layout there are three utility keys:

X,OFF    Calculates a radius that fits a given tanget

and offset distance.  To use this radius in subsequent

input it by pressing the "RAD" key on menu 1.

T->CD    Calculates a chord that corresponds to a given tangent

distance and a previously inputted radius.  To use the

the new chord press the "CHORD" key on menu 1.

PIC      Provides a picture and sign conventions for tangent layout

of curves.

Two utility keys apply to both Chord and Tangent layout methods:

OFFSE    Incorporates an offset line into calculated curve offsets.

When active USER FLAG 1 annunciater is lit.  To cancel,

key in 0 and press "OFFSE".  Flag 1 will clear.

XFALL    Incorporates a slope correction in the calculated curve

offsets.  When active USER FLAG 2 annunciator is lit.

To cancel, key in 0 and press "XFALL".  Flag 2 will clear.

To see more menu key descriptions press the REVIEW key(LS 3,5).

Use the interactive stack key by pressing the UP key(2,5) to review

all previous inputs and results.  Press ATTN to return.

This program uses local variables and the stack for all calculations.

Use the "QUIT" key on menu 1 to exit the program and restore memory.

NNP - Nine Point Plot  by V.A. Maletic

1005.5 bytes

This program is used to spot irregularities or blunders in nine-point

elevations taken of existing bridge spans for widening purposes.

Elevations can be verified in the field before moving or taking down

the level.

It also outputs a residual list and a moving residual list.  These list

can be used to stringline in profile elevations at 9 pt dowels.

This program takes a nine element list { E1 E2 E3 .. E9 } of uniformly

spaced elevations and generates a screen R-Plot of them.  If a bridge

span or approach slab is so short that elevations taken at 1/8th inter-

vals (nine points) are too close, you may place on the stack a five

element list { E1 .. E5 } of elevations taken at quarter points of the

span or approach slab.  Five element lists of elevations are expaned

into nine element list by insertion of intermediate average elevations.

Thus { E1 E2 E3 E4 E5 } becomes -> { E1 E12 E2 E23 E3 E34 E4 E45 E5 }.

If you begin with an empty stack this program will sequentialy prompt

for the nine elements required.

The R-Plot is labled along the bottom with point numbers and the maximum

positive or negative residual.  Eighth span tick marks are provided.

An R-Plot of the elevations removes the gradient between E1 and E9 and

plots the interior elevation's deviations (residuals) from the E1->E9

gradient.  The resulting R-Plot is displayed on the graphics screen and

irregularities (or blunders) in the nine point elevations are usually

very obvious.  The magnitude of irregular elevations (blunders) can be

estimated by inspection.

SPANPROFILE ver 1.5 by V.A. Maletic  2-28-93

3872.5 bytes

This program can be used to develop span profiles.

SPANPROFILE begins with a straight or vert curved line through two

given points of known elevation or height and whose distance between

them is known.  A vert curve is applied by inputting a +/- 'c' value.

A span camber can be applied by inputting a max +/- value and

pressing a camber type key (Sgl span, 1st span, Inter span Last span).

A correction adjustment can be applied by inputting a max +/- value

and pressing a type key (Rev curve corr or Straight line corr).

The correction will be zero at P1 and max +/- value at P2.

Cambers and corrections apply only to distances between P1 and P2.

Outside of P1 and P2 only vertical curve applies.

Direction convention:  Layout stationing is directed from P1 to P2.

First span is of lower station than last span.  Interior spans have

stations larger than first span and less than last span.

The DATA key copies the following to the stack:

P1, P2, L, c, K, slope and if applied:

Camber type, value 9pt[list];  Correction type, value, 9pt[list]

PLINESX or PLINEGX (Profile LINE for SX or GX) ver 1.2 by Victor Maletic 4-14-98

5837.5 bytes

This program calculates elevations along three types of profile lines.  The profiles are inputted and saved as data files that can be retrieved anytime elevations are required from it.  After profile data has been entered and editing option is available so that the input my be checked and corrected if necessary before it is filed.  Additional features:

● Addition of a positive or negative constant value to computed elevation.

● Calculate elevations at specified intervals from a specified starting station.

● Station range checking.  Will not calculate elevations beyond profile limits.

● Profile data files are stored in port 0 to reduce accidental erasure.

XLINE ver 1.6 by V.A. Maletic  3-27-93

2539.0 bytes

This program is an extended LINE program hence the name XLINE.

XLINE will generate a straight or vert curved line through two given

points of known elevation or height and whose distance between

them is known.  A vertical curve (parabolic) is applied by inputting

a +/- 'c' value - positive 'c' for upward curvature - negative 'c'

for sag curvature.

A correction adjustment can be applied by inputting a max +/- value

and pressing a correction type key (Rev curve corr, Straight line corr

or Bell curve correction).

The correction will be zero at P1 and max +/- value at P2.

Corrections apply only to distances between P1 and P2.

Outside of P1 and P2 only straight or vertical curve line is available.

Direction convention:  Stationing direction is P1 to P2 and P1 = 0.

The DATA key copies the following to the stack:

P1, P2, L, c, K, slope and if applied, correction type, max/min value,

and 9pt[array] of values.

TRAVGXv6 or TRAVSXv6 ver 6 by V.A. Maletic  3-30-98

16901.5 bytes

The TRAV program is used to calculate coordinates for structure layout and checking reference staking.  This program makes it easier to coordinate key structural elements and 'RP' staking for cross-tie checks, transit angle checks and other checks.

Commercial computer surveying programs have more capability than TRAV does, but access to them and familiarity with them is often limited.  TRAV is optimized for structure layout and 'RP' checks.

With TRAV you are encouraged to work directly on the project plans generating coordinates for structural features and 'RP' points.  You can write the coordinate values directly on the plans as they are computed or better yet - just write down the point number automatically assigned to each calculated coordinate.  To inverse between any two points, enter their assigned numbers on the stack and press the (INV) key.  To compute the coordinates of two intersecting lines, input a point number and associated azimuth for each line and press the (INTER) key. You can also define a line to be intersected with two point numbers.  Line-Distance and Distance-Distance intersections are also available.  Each line or distance input can have an offset value that will be used in the intersection.  See page 9 for a description of key conventions used above and below.

Traverse data generated can be saved to a file name of choice and later retrieved for additional traversing, inversing or line intersections.

Some other features of TRAV:

● Repeatedly traverse up the current azimuth specified distances without having to key in the azimuth each time.

● Reverse azimuth direction.  A reversed azimuth will be displayed along with accumulated distance since last angle change.

● Move to any previously calculated point number and begin a branch traverse.

● Incorporate a non-traverse point.  This point will become part of the traverse file and can be used like any other calculated point.

● Traverse along a curve to point of interest as defined by stations or arc distances from the BC.  Azimuths to the radius point, radius coordinates, POC stations and coordinates and current azimuth ahead on tangent line are displayed.

● Convert bearing and quadrant numbers to azimuths.

● Convert ±skews to ±deflection angles.

● Show an azimuth 90° left or right or 180° of current azimuth.

● Convert feet, inches and sixteenths to decimal feet and the inverse.

● Calculate areas bounded by straight lines connecting specified coordinate numbers.

● Utilities for working with triangles, polar to rectangular, rectangular to polar, radians to dd.mmss, dd.mmss to radians, HMS→, →HMS, HMS+ and HMS-.