Copyright ã 1994, Rick Grevelle and Jeoff Krontz

All rights reserved.  No part of this book may be reproduced, transmitted, or stored in a retrieval system in any form or by any means, electronic, mechanical, photocopying, recording or otherwise, without the prior written permission of the authors.

The software, this manual, and any examples contained herein are provided "as is" and are subject to change without notice.  The authors make no warranty of any kind with regard to this software or manual, including, but not limited to, the implied warranties of merchantability and fitness for any purpose.  The authors shall not be liable for any error or for incidental or consequential damages in connection with the furnishing, performance or use of this software, keyboard overlay, manual or examples presented herein.

The owner of this manual and software may not distribute or transfer the software under any circumstances without specific prior written permission of the authors.

Edition 1 - April, 1994  coinciding with software version 1.0

Thermodynamics Library Registration Card

In order to be notified of program updates, documentation updates, or new products, please fill out the information below. Tear out this sheet and mail to:

Jeoff Krontz

401 Southwest Parkway #803

College Station, TX  77840

USA

| Product: | Thermodynamics Library for the HP48 |
| --- | --- |
| Date Purchased: |  |
| Purchased From: |  |
| Purchase Price: |  |
| Name & Address: |  |
| Comments: |  |

Thermodynamics Library for the HP48

by:

Rick Grevelle

Jeoff Krontz

| # Table of Contents |  |
| --- | --- |

µTable of Contents	1

Notes and Acknowledgements	2

Introduction	3

Getting Started	4

Installing Thermo-Pro	4

Removing Thermo-Pro	4

Entering Thermo-Pro	4

Thermodynamic Properties	5

Allowable Combinations of Properties	5

Available Databases	5

Allowable Ranges	6

Available Units	6

Programmable Commands	8

Arguments	8

Results	8

Plotting	8

Table of Programmable Commands	9

The Interactive Environment	10

Examples	11

Thermodynamic Cycles	15

| # Notes and Acknowledgements |  |
| --- | --- |

During the Spring of 1993, I took a thermodynamics class at Texas A&M University.  The class was very labor intensive, in that every problem involved looking up values on one of twenty tables in the back of my thermo book.  Many of these lookups involved much interpolation to arrive at the right value of a property at a given temperature or pressure.  I used my HP48 extensively to perform the necessary interpolations for me and often marveled at how those who had to do the interpolations by hand even managed to finish the test.

It was at this time that I hit upon the idea of creating a thermodynamics package for the HP48.  Such a package would simplify every aspect of the menial table lookups and leave a lot of extra time during test in which to actually work the problems themselves.  However, due to a lack of time, resources, and programming experience I put the idea aside.  It was not until I was approached by a friend, Rick Grevelle, with the same idea, that this package began to form.

From that point on, this package grew steadily.  It involved lots of code writing, scanning in thermodynamic tables, late night discussions, and tutoring to get the project finished.  Rick programmed the entire package, quite a feat considering he has never taken a thermodynamics class.  Neither one of us could have finished this project without the other, as we both had areas of expertise in which the other lacked.

Thank everyone and tell about the resources that we used.

Jeoff Krontz

April, 1994

| # Introduction |  |
| --- | --- |

The Thermo-Pro is a library written for the HP 48 calculator which contains an extensive thermodynamics database of the properties of water in saturated, superheated vapor, and compressed liquid states.  Additionally, the properties for refrigerants 12 and 134a in saturated and superheated states appear along with data for the ideal gases N2, O2, H2O, CO, CO2, H2, and air.  To effectively deal with such a massive data base with only the 48's architecture, this software was written in machine code.  This accounts for the impressive performance that enables the user to quickly look up properties of any of the above substances as well as use the programmable commands to quickly plot graphs of any region within the range of the database.  For direct hands on number crunching a machine code user interface allows the user to process large amounts of data in record time.  Both English and S.I. units are available and unit conversion between these systems is supported.  Those who are already familiar with the details of thermodynamics, and who regularly use a database of thermodynamic properties, will find this software to be enormously beneficial.  Precision demands of the professional engineer can now be obtained in a handheld device using this software.  Students of thermodynamics will no longer have to endure the brutality of exams traditionally given in thermodynamics courses.

| # Getting Started | 1 |
| --- | --- |

The Thermo-Pro Package is resident upon the 128k ROM card supplied in this package.  This package is compatible with any version HP48 S/SX/G/GX.

## Installing Thermo-Pro

To install Thermo-Pro on your HP48 calculator, do the following:

·	Turn off the calculator.

·	Insert the 128k card into either port of the calculator.

·	Turn on the calculator.

Thermo-Pro should now be installed in your calculator.  To verify this, press <-\ LIBRARY and make sure that one of the soft-key menu labels says "THER", short for "THERMO".

## Removing Thermo-Pro

To remove Thermo-Pro from your calculator, do the following:

·	Turn off the calculator.

·	Remove the 128k card from the calculator.

## Entering Thermo-Pro

To begin using Thermo-Pro, press <-\ LIBRARY and press the menu key which says "THER".  You will see the following menu labels which correspond to the different substances included in the databases included.

MENU LABEL GROB HERE WHEN RICK FINISHES...

| # Thermodynamic Properties | 2 |
| --- | --- |

## Allowable Combinations of Properties

Only certain combinations of properties may be entered when using Thermo-Pro.  This is due to the fact that certain combinations may appear at more than one position in the tables and would be impossible or unfeasible to use.

When using the routines for H2O, R-12, and R-134a, a temperature or pressure must be given before Thermo-Pro will attempt to perform a lookup.  Both temperature and pressure can be used simultaneously, but at least one must be given when v,h,u,s, or X is given.  An error will occur if this condition is not met.

Example:  Incorrect parameters entered.

An error message occurs.

When using the ideal gas routines, there are no restrictions on what you may enter.  Only one property is necessary and then Thermo-Pro will be able to look up the rest, no matter which one is given.

## Available Databases

Thermo-Pro comes with databases containing useful thermodynamic properties of many different substances.  Databases are available for water, refrigerant 12, and refrigerant 134a as well as the ideal gases N2, O2, H2O, CO, CO2, H2, and air.

The properties available for lookup in the databases for water and the two refrigerants are temperature, pressure, specific volume, internal energy, enthalphy, and entropy.

The properties available for lookup for the ideal gases are temperature, internal energy, and specific entropy.  Relative pressure and relative volume may also be looked up for the ideal gas air.  The specific entropy for the ideal gases is actually the entropy for the gas at a given temperature and a pressure of 1 atm.

## Allowable Ranges

The databases within Thermo-Pro are finite.  Sometimes Thermo-Pro will not be able to find a given combination of properties within the tables.  This will be due either to the fact that the given combination does not physically exist or to a limitation of the tables.  The tables contained in Thermo-Pro have maximum temperatures and pressures.  These limits are listed in the following table.

|  | Temperature (°C) |  | Pressure (bars) |  |
| --- | --- | --- | --- | --- |
| Substance | Min | Max | Min | Max |
| Water Two-Phase Mixture | .01 | 374.14 | .00611 | 220.9 |
| Superheated Vapor | saturation | 500 - 900 | .06 | 320 |
| Compressed Liquid | 20 | saturation | 25 | 300 |
| R-12 Two-Phase Mixture | -40 | 112 | .6417 | 1.792 |
| Superheated Vapor | saturation | 80 - 220 | .6 | 16 |
| R-134a Two-Phase Mixture | -40 | 100 | .5164 | 39.742 |
| Superheated Vapor | saturation | 90 - 200 | .6 | 16 |
| Air | 0 | 2250 |  |  |
| Nitrogen (N2) | 0 | 3250 |  |  |
| Oxygen (O2) | 0 | 3250 |  |  |
| Water Vapor (H2O) | 0 | 3250 |  |  |
| Carbon Monoxide (CO) | 0 | 3250 |  |  |
| Carbon Dioxide (CO2) | 0 | 3250 |  |  |
| Hydrogen (H2) | 0 | 3250 |  |  |

## Available Units

When entering the values of properties in Thermo-Pro, the menu at the bottom of the screen will show the available units.  Each property has its own set of units which are commonly used and useful.  When scrolling from property to property the menu keys are automatically updated to display the correct set of units for each property.

The following table outlines all available units.

| Property | Units Available |
| --- | --- |
| Temperature | °C, °F, K, °R, |
| Pressure | bar, psi, MPa, atm, torr, inHg |
| Specific Volume | µ §, µ §, µ §, µ §, µ §, µ § |
| Internal Energy | µ §, µ §, µEMBED Equation.2 \s \* mergeformatµ §§ |
| Enthalpy | µ §, µ §, µEMBED Equation.2 \s \* mergeformatµ §§ |
| Entropy | µ §, µEMBED Equation.2 \s \* mergeformatµ §§, µEMBED Equation.2 \s \* mergeformatµEMBED Equation.2 \s \* mergeformatµ §§§ |
| Quality | no units available |

| # Programmable Commands | 2 |
| --- | --- |

Thermo-Pro contains programmable commands which can be used to perform lookups without having to use the graphical user interface.  These commands exist for each substance included in the database and for each valid combination of properties that may be used.  All the programmable commands are summarized at the end of this chapter.

These programmable commands will be useful for those who which to write their own interface for Thermo-Pro or those who wish to look up properties from their own programs.  The commands are also useful for plotting the variation of properties which respect to another property.

## Arguments

These programmable commands take as arguments two numbers from the stack.  The number in stack level two will be used for the value of the first property in the function name and the number in stack level one will be used for the value of the second property.

These numbers may have any units attached, even those that are not internally supported using the graphical user interface.  Appropriate unit conversions will be performed automatically.  If units are not supplied with the numbers, default Si units will be assumed.

This is an example of a correct stack configuration when calling a programmable command which requires temperature and internal energy.  The units could have been left off.

## Results

The results of the programmable commands are put on the stack as numbers with tags and units.  The results will include all of the parameters that would normally be found using the graphical user interface.  The tags will show which property each number represents and the units will be the default SI units.

Example: Using a programmable function (T&hH2O)

If the programmable function cannot locate a set of properties that matches its inputs, a "Bad Argument Value" error will occur.

Example: Using bad arguments with T&uH2O

## Plotting

SHOW PLOT EXAMPLES HERE - ASK RICK...

## Summary of Programmable Commands

| Substance | Programmable Commands Available |  |  |  |
| --- | --- | --- | --- | --- |
| Water | T&vH2O | P&vH2O | T&uH2O | P&uH2O |
|  | T&hH2O | P&hH2O | T&sH2O | P&sH2O |
|  | T&xH2O | P&xH2O | T&PH2O |  |
| Refrigerant 12 | T&vR12 | P&vR12 | T&uR12 | P&uR12 |
|  | T&hR12 | P&hR12 | T&sR12 | P&sR12 |
|  | T&xR12 | P&xR12 | T&PR12 |  |
| Refrigerant 134a | T&vR134 | P&vR134 | T&uR134 | P&uR134 |
|  | T&hR134 | P&hR134 | T&sR134 | P&sR134 |
|  | T&xR134 | P&xR134 | T&PR134 |  |
| Air | TAIR | hAIR | uAIR | sAIR |
|  | prAIR | vrAIR |  |  |
| Nitrogen | TN2 | hN2 | uN2 | sN2 |
| Oxygen | TO2 | hO2 | uO2 | sO2 |
| Water Vapor | TH2O | hH2O | uH2O | sH2O |
| Carbon Monoxide | TCO | hCO | uCO | sCO |
| Carbon Dioxide | TCO2 | hCO2 | uCO2 | sCO2 |
| Hydrogen | TH2 | hT2 | uH2 | sH2 |

| # The Interactive Environment | 3 |
| --- | --- |

Thermo-Pro uses a graphical user interface which enables the user to easily look up thermodynamic properties.  It is very user friendly and greatly speeds up the lookup process.

## Display Description

The Thermo-Pro is very simple, yet functional.  The display for the fluids and ideal gases are almost identical.  The ideal gas interface is missing the pressure, specific volume, and quality properties.  The display for air, however, has pr and vr instead of pressure and specific volume.

All three types of displays are shown below.

Display for H2O, R-12, and R-134a

Display for the ideal gas air

Display for the other ideal gases

## Key Bindings

All the HP48's keys become redefined when using the Thermo-Pro graphical user interface.  However, the keys behave differently depending upon whether they are pressed during the normal mode of the interface or when there is an edit line present.

All the valid keystrokes are shown on the tables below.  If a keypress is not on these tables, then it is invalid.  An error beep will signify this fact when an invalid keystroke is pressed.

The following table shows the key definitions when Thermo-Pro is in normal mode.  This is the mode Thermo-Pro will always be in when an edit line is not present.

| Normal Key Definitions |  |  |
| --- | --- | --- |
| Keystroke | Description |  |
| up | Moves the highlight box up one property |  |
| down | Moves the highlight box down one property |  |
| enter | Makes the current property the stable property |  |
| del | Clears the value and stability of the current property |  |
| clear | Clears all properties and returns the box to temperature |  |
| left-units | Convert all units to the default SI units |  |
| right-units | Convert all units to the default English units |  |
| space | Pushes all property values to the stack with tags |  |
| left-space | Pushes all property values to the stack with units and tags |  |
| right-space |  |  |
| on | Leaves the graphical user interface |  |
| left-on | Turns the calculator off |  |

The following table summarizes the definition of the HP48's keys when an edit line is present.  An edit line becomes present whenever one of the number keys is pressed and disappears when either ENTER is pressed or <= is used to delete the entire line.

| Edit Line Key Bindings |  |  |
| --- | --- | --- |
| Keystroke | Description |  |
| any digit | Places the corresponding digit onto the edit line |  |
| backspace | Erases last digit entered & if no digits are left, exits edit line |  |
| up | Attempts lookup if another property is stable & moves up |  |
| down | Attempts lookup if another property is stable & moves down |  |
| enter | Attempts lookup if another property is stable & quits edit line |  |
| clear | Clears all properties except current one but stays on edit line |  |
| left-units | Convert all units to the default SI units |  |
| right-units | Convert all units to the default English units |  |
| space | Pushes all property values to the stack with tags |  |
| left-space | Pushes all property values to the stack with units and tags |  |
| right-space |  |  |
| on | Restores the property to its previous value & clears edit line |  |
| left-on | Turns the calculator off |  |

Default Conditions

Entering Properties

Changing Properties

Units Conversions

Saving Results

| # Examples | 4 |
| --- | --- |

Example:   Given that the temperature of H2O as a saturated liquid is 250°C, calculate the P,v,u,h,s, and quality of the water at this state.

First of all, since the water is a saturated liquid its quality will be zero.

Now Thermo-Pro can be used to determine the rest of the properties.

Enter the temperature of the water:

250 enter

Enter the quality of the water.

down 5 times 0

Press enter

All the values wanted have been calculated by Thermo-Pro.

Example:   Given that the temperature of a sample of H2O has a temperature of 250°C and an internal energy of 1150 kJ/kg , calculate the sample's P, v, h, s, quality, and state.  Also calculate these properties with an internal energy of 1300 kJ/kg .

Enter the temperature of the H2O

Enter the internal energy of the sample

Press Enter

We have calculated the values for the sample at the first internal energy and have found it to be a two-phase mixture, since it has a quality.  The next step is to calculate the values at the new internal energy

Simply type the new value over the old one.

Press Enter to see the new results

Example:   Find T,u,h,quality, and the state of a sample of H2O with a pressure of 80 bars and a specific volume of .05 µ §.  Find the same properties if the sample has a pressure of 50 atms.

Enter the pressure of the sample.

Next, enter the specific volume.

Now that we have determined all the wanted properties, including that the state is Superheated Vapor and the quality is non-existant, it is time to change the pressure to 50 atms.

Make specific volume the stable property. A mark will appear by it.

Change the pressure value as well as its units.

Recalculate the properties.

Once again, Thermo-Pro has easily calculated the new properties.

Example:   Find T,u,v,x, and the state of a sample of H2O with a pressure of 20 MPa and an entropy of 600 µ §.

| # Thermodynamic Cycles | 5 |
| --- | --- |

WE NEED TO WRITE THIS CODE -> STRAIGHTFORWARD