GarHP v1.0

==========

Garmin GPS application by Arnold Moy (c)1997.

CONTENTS

========

1. Introduction

1.1 Copyright and Disclaimer

1.2 The GarHP Library

2. Installing and Removing the GarHP Library

2.1 Installing the Library

2.2 Bytes and Checksum

2.3 Removing the Library

3. Usage

3.1 Commands

3.2 Messages

4. History

5. Resources and References

1. Introduction

---------------

1.1  Copyright and Disclaimer

-----------------------------

All files in the GarHP package are copyrighted(c) by Arnold Moy

unless otherwise noted.

The software contained in this package is provided as free of charge

and therefore contains no warranty of any kind.  Should the Software

prove defective, you and not the author assume the entire cost of any

service and repair.  This disclaimer of warranty constitutes an

essential part of the agreement.

1.2 The GarHP Library

----------------------

Receive and send waypoint, proximity, track and route data between a

HP48 and Garmin GPS receiver.  This version does not support newer

Garmin receivers that use icons.

2. Installing and Removing the GarHP Library

---------------------------------------------

The GarHP library requires at least 1.7K to install.

2.1 Installing

--------------

a) send the library to the HP48

b) put a copy of the library on the stack

c) enter the port number to store the library (0-33 GX, 0-2 SX)

d) at this point, press [STO]

e) perform a warmstart with [ON][C] or turn on and off the HP48

f) erase the variable containing the library.  eg. 'GARHP10.LIB' PURGE

2.3 Bytes and Checksum

----------------------

Placing the library on the stack and executing the BYTES command:

Bytes: 1651.5

Checksum: # FEE4h

2.4  Removing the Library

-------------------------

a) :0: 1061 DETACH

b) :0: 1061 PURGE

Depending on which port number the GarHP library was stored, :0: in the

above example can be replaced by the appropriate port number.

3. Usage

--------

You will need to construct a cable to connect your GPS to the HP48. No

communication parameters need to be set on the HP side, but make sure

your GPS receiver's interface is set to GRMN/GRMN.

3.1 Commands

------------

Receiving

---------

Make sure your GPS receiver is connected and the interface is set to

GRMN/GRMN, then execute one of the following commands to download data

to the HP. The data received will be stored as a Library Data object.

When the transfer has completed, store the Library Data using a

meaningful name, so you can retrieve it later.

RWAY - receive waypoints

RPRX - receive proximity waypoints

RTRK - receive track

RRTE - receive route

Transmitting

------------

Make sure your GPS receiver is connected and the interface is set to

GRMN/GRMN, then execute TRAN which requires data previously received

using the RWAY or RPRX or RTRK or RRTE, on stack level 1.

TRAN - transmit data to the GPS receiver.

a) For example,  recall the data to the stack:

--------------------------

4:

3:

2:

1:           Library Data

|   |   |   |   |   |   |

b) Then execute TRAN

3.2 Messages

------------

Receiving Waypoint - receiving waypoint data

Receiving Proximity - receiving proximity data

Receiving Track - receiving track data

Receiving Route - receiving route data

Finished Receiving - transfer was completed

Receive Aborted - transfer aborted by user

No Records - no records to receive

No Memory Left - not enough memory to receive data

Comm Problem - a communication error occurred and this usually indicates

the interface on the GPS receiver is not set to GRMN/GRMN

Timed Out - the connection timed out and this usually indicates your

receiver is not connected to the HP48.

Invalid Record - a record with a bad checksum was encountered

4. History

----------

GarHP v1.0

- first release

5. Resources and References

---------------------------

HPTools - PC based System RPL and Assembly Language compiler

ftp://hpcvbbs.external.hp.com/dist/

Jazz - HP48 based System RPL and Assembly Language development tools

http://www.hut.fi/~mheiskan/

Arnold Moy - email: amoy@geog.ubc.ca

WWW: http://www.geog.ubc.ca/~amoy/