multi-task on your HP48SX

by Lilian PIGALLIO

November 1994

You need: a HP48SX with a RAM card not in SYSRAM mode and

without 'WRITE PROTECT'

The multi-task take 2 'Code' from the RPL STACK

When you are in one of the 2 asm progs, D1 is on level 3.

RPL STACK:      ..... (others parameters, options)

4: 2nd parameter of one asm prog (option)

3: 1st parameter of one asm prog (option)  D1 is here

2: Code

1: Code

The multi-task,the parametres and the 2 Codes can't be in the RAM needed card.

You can't use   - sub-routines in ROM with an address < 1000h

- TIMER 2 and his control

- keyboard buffer ans sub_routines

Now the 2 asm progs run at the same time !!!!!!

You can stop them when you want, just pressing ON.

Multi-task will finish when the 2 asm progs will finish.

You can read parameters in the RPL-STACK using D1 like an ordinaly asm prog.

C=DAT1  A

D0=C

%read 1st parameter value

D1=D1+  5

C=DAT1  A

D0=C

%read 2nd parameter value

%etc...

If you want that some registers keep their value in RPL, you must use this

method:

%.....

/LOAD_REG

D=D+1   A

D1=D1+  5

/SAVE_REG

/LOAD_REG       %restore D0

/RTN_RPL        % A=DAT0 A, D0=D0+ 5, PC=(A)

@"

If you want run a piece of you prog without scheduling use ST 15:

ST=0    15

%now, only this prog is active

ST=1    15

%scheduling

Look at P1, with and without scheduling in the BEEP sub-routine

*************************************************************************

Sub-routines that run with multi-task

-SAVE_REG

-LOAD_REG

-D0TOS

-D1TOS

-LOAD_REG_EXIT

-all calculus sub-routines

-BEEP

-STI (ALLINTR)

and these that don't work

-CLI (DISINTR)

-LOAD_REG_TRUE_EXIT    = LOAD_REG_EXIT

-LOAD_REG_FALSE_EXIT   = LOAD_REG_EXIT

-DROP_EXIT             = EXIT

-the calculus CRC is false

-ERROR

-STOP

If you want to help me:

- try to play a music at the same time that other display prog

- make a list of all ROM sub-routines that work (and not)

- give me a lot of suggestion and critics.

Is it a good idea for the next version?:

no address limit for ROM sub-routine (here >1000h)

keyboard buffer active with his sub-routine

It is a good chois to use ST 15 ????

Do you need others special instuctionsfor programming:

Kill_all (or kill_other_process)

wait_process ???

In all cases, if you make a prog for multi-task, it is better if it can work

without multi-task (example: music and graphics).

Have fun with it.

Lilian PIGALLIO