Path: cdc486.cdc.polimi.it!ghost.dsi.unimi.it!univ-lyon1.fr!jussieu.fr!univ-lille1.fr!zaphod.crihan.fr!warwick!doc.ic.ac.uk!agate!usenet.ins.cwru.edu!news.ysu.edu!psuvm!dar110

Organization: Penn State University

Date: Thu, 16 Dec 1993 03:47:31 EST

From: <DAR110@psuvm.psu.edu>

Message-ID: <93350.034731DAR110@psuvm.psu.edu>

Newsgroups: comp.sys.hp48

Subject: Tricorder converted to AsmFlash!

Lines: 85

AsmFlash Converts Another Popular Program!  "TriCorder"

Steven Ourada  "Tricorder"  IR Proximity Detector Copyrite 1992

Converted over to AsmFlash v6.1 by dar110@psuvm.psu.edu (DavidR)

Here is the AsmFlash (don't you just love this powerful utility!)

version of Steven Ouradas' "Tricorder".  Fortunately he provided the

(alternate mnemonic set) assembly language (and UserRPL) source, which

I have converted back to HP-Mnemonic format, then to AsmFlash...

This program here works pretty much the same way as Steves.

(Please consult his program for technical details).

Simply assemble and run the program.  Note a "low-freq" purr when

nothing obstructs the HP48 IR area.  Put an object, or light source

(even heat in a dark room!) in front of the IR area, and listen to

the responce!  Wow!

%%HP: T(1)A(D)F(.);     ; Header for HP48

<<                      ;

"                       ; Begin AsmFlash Program String

GOSBVL 0679B            ; =SAVPTR

*MAIN                   ; Main Program Loop

D1= 0011A               ; HP:IRC IR Control Register (bit 3 is RO)

A=DAT1 1                ; Read low nibble from D1    "move.a #$11a,d1"

ABIT=0 0                ; Clear bit 0 of that nibble "bclr #0,a"

DAT1=A 1                ; Write modified nib back    "move,1 (d1),a"

; comment reads "clear IR detector bit"

D0= 0011C               ; NS:LCR Led Control Register (draining ???)

A=DAT0 1                ; Read low nibble from D0    "move.a #$11c,d0"

ABIT=1 3                ; Set bit 3 of that nibble   "bset #3,a"

C=A B                   ; Copy Byte field of A into C

CBIT=0 3                ; Clear bit 3 of that nibble "bclr #3,c"

DAT0=A 1                ; Write modified nib back    "move.1 a,(d0)"

; comment reads "turn IR emitter on"

A=DAT1 1                ; Read  one nibble "read IR detector" ($0011A)

DAT0=C 1                ; Write nibble in C back to $0011C  "move.1 c,(d0)"

; comment reads "turn IR emitter off (on ^20us)

?ABIT=1 0               ; If bit_0 of $0011A is set, sound it!

; "bbs #0,a,reflec" branch_if_bit_set,bit,reg,label

GOYES REF               ; Where to go if reflection suspected...

GOTO MAIN               ; otherwise just loop back and keep checking...

*REF                    ; AsmFlash label (positive action to take)

;

; Some of you may recognize this routine.  Thanks Doug Cannon!

;

P= 0                    ; Register-access control

LC 800                  ; Load C with three nibbles (bitmap value;X field)

OUT=C                   ; Output_register (accepts 3 nibs; X field)

LC 00012                ; Load C with 'frequency'

*D1                     ; AsmFlash label w/in REF

C=C-1 A                 ; Decrement addr field of C (dec freq)

GONC D1                 ; Go relative on No Carry to label D1 (above)

C=0 A                   ; Clear Addr field of C

OUT=C                   ; Write $000 to Output_reg (toggle Speaker)

LC 00012                ; Load C with 'frequency'

*D2                     ; AsmFlash label w/in REF

C=C-1 A                 ; Dec 'freq'

GONC D2                 ; go relative on No Carry to label D2 (above)

GOTO MAIN               ; All done, exit REF routine, repeat Main Loop...

@"

>>

section 1 of uuencode 5.21 of file triaf.bin    by R.E.M.

begin 644 triaf.bin

M)25(4#H@5"@Q*4$H1"E&*"XI.PT*JPT*(@T*1T]30E9,(#`V-SE"#0HJ34%)

M3B`-"D0Q/2`P,#$Q00T*03U$050Q(#$-"D%"250],"`P#0I$050Q/4$@,0T*

M1#`](#`P,3%##0I!/41!5#`@,0T*04))5#TQ(#,-"D,]02!"#0I#0DE4/3`@

M,PT*1$%4,#U!(#$-"D$]1$%4,2`Q#0I$050P/4,@,0T*/T%"250],2`P#0I'

M3UE%4R!2148-"D=/5$\@34%)3@T**E)%1@T*4#T@,`T*3$,@.#`P#0I/550]

M0PT*3$,@,#`P,3(-"BI$,0T*0SU#+3$@00T*1T].0R!$,0T*0STP($$-"D]5

M5#U##0I,0R`P,#`Q,@T**D0R#0I#/4,M,2!!#0I'3TY#($0R#0I'3U1/($U!

+24X-"D`B#0J[#0J[

`

end

sum -r/size 16269/478 section (from "begin" to "end")

sum -r/size 28834/326 entire input file

Hope you enjoy this program.  Should you know of any other relatively,

small sized program which has Assembly-source, please pass it along to

me, I'll see what I can do to convert it over to AsmFlash!  Who needs

SysRpl :) !!