HP49 DATA

Ó HEUSON SOFTWARE

To transfer the programs from the disk to your HP49, you need a computer and a Serial Interface Kit. Transfer the zip.file to your computer and extract it with winzip7.0 (http://www.winzip.de). Use the included program Kermit.exe for transfer of the programfiles to your HP49. Read the file Kermit49.txt. Transfer the library hpdata.lib and the examples adress.db and particle.db to your HP49 HOME directory. Press the key hpdata.lib and [2] [STO]. Now perform a warmstart by pressing simultaneously [ON][C]. The library hpdata.lib is attached and you have access to the commands by pressing

[->][LIB][HPDATA].

HP49:

[APPS] [I/O functions][OK][transfer][OK].

Now key in the transfer parameter:

Port: wire, Type: Kermit, Name: () Fmt: Bin,

Xlat: Newl, Chk: 3, Baud:9600, Parity: None, OvrW: ()

PC:

start Kermit.exe by doubleclick  in EXPLORER or FILEMANAGER.

Filetransfer HP49=>PC:

PC: At the DOS-prompt key in: receive

HP49: Put the name of the file into stack 1.

You can also send a list of files 1: {NAME1 NAME2 ..}.

Now type SEND. The file 'NAME' now will be transfered.

Filetransfer PC=>HP49:

PC: At the DOS-prompt type:  send NAME

HP49: Type RECV

The file 'NAME' now will be transferred to the HP49

You can also type [APPS] [I/O functions] [transfer] and then the menuekeys RECV or SEND, or you can send or receive with the filer on the HP49. Save the transferred files in port 2 with the filer.

Hint: If the message "this port operates through the BIOS" appears, then finish Kermit and start it again !

HEUSON SOFTWARE

D-87493 Lauben, Zugspitzstr. 4, Germany

phone/fax : +49-8374-8299

E-mail: heuson-software@freenet.de

Homepage: http://www.freenet.de/heuson-software/

HPDATA

databaseprogram for den HP49.

Database	databaseprogram	1: 'NAME' (1:_)	1:'NAME'

cursor left/right: previous/next datas	Prog gestartet

cursor up/down: previous/next page

STO: store database, 12 [ ] goto to set 12

NXT: next menu keys, ENTER end.

menu keys: add, del, edit, srch, list, stk

goto, sort, cut, copy, paste, open.

S«D	string Û database	1:".."	1:{".."}

(datas seperated by 2 newlines)

Help	Hilfe	1:_	1:_ , help

ADRESS.DB  example address-database	1:_	1:{".."}

A databaseprogram for the HP49. You can use it for adresses or arbitrary sets of data. The program itself has 4500 B and is nearby selfexplaining. It has a list and a form screen and you can add, delete, edit, search, sort, cut, copy and paste datasets. Two examples : adress and particle-data are implemented in the Directory.

Database	databaseprogram. 'name' -> start; _ -> start.

If you press database when the stack is empty you see the message

"create new database or select old one key in name". Now press the

menu key adress.db and ENTER and you see the first dataset of adress.db.

cursor left/right : previous/next dataset

cursor up/down : previous/next page of the actual dataset or of the list screen

STO : store database

ON, ENTER : quit program without saving changes.

NXT : next page of the menu. pressing NXT again, you are back on the first page.

4, 12 any key -> goto dataset 4,12 fast jump to any dataset (also possible with goto)

Menukeys :

ADD : add new dataset at actual position

DEL : purge actual dataset

EDIT : edit actual dataset

SRCH : search dataset, key in search string.

LIST : switches between list- and data-screen.

I listscreen the first row of the datascreen is displayed. Every dataset has an number and the actual dataset is marked. All functions are in available in listscreen, but slower than in datascreen.

NXT => next Menukeys:

GOTO : go to dataset number n: n -> ..

SORT : sort data

CUT : cut datasets from m to n

COPY : copy datasets from m to n,

PASTE : paste copied or cutted datasets at position n.

OPEN : open new database without saving the old.

You can paste datasets into a new file, which you have copied or cutted in a old file.

ADRESS.DB	example adress-database, PARTICLE.DB example database with particledata.

S«D	transforms string into database, the single datasets are separated by 2 Newlines.

You can transfer ASCII-data from PC -> HP and transform them in a database with S«D