Moulds for HP49G & HP48xx Cable Plug

Calculator to PC Serial Interface

I salvaged a cable from an old non-functioning mouse. The end of the cable that connects to the serial port on the PC is moulded with a DB9, DB25 female plug or a PS2 connector. The cable is cut from the mouse end and the wires are exposed. Identify the wires that connect to the serial plug as shown in Fig 1. You will require the following 3 wires only:

|  | DB 9 | DB25 |
| --- | --- | --- |
| Transmit (TX) | 2 | 3 |
| Receive (RX) | 3 | 2 |
| Ground (GND) | 5 | 7 |

- If the shield is connected to the metal casing of the above plugs, the shield can be soldered to pin 1 of the calculator plug. Discard all other excess wire, as they are not needed or used.

- The plug that connects into the HP49G calculator is a dual in line socket 2x5 way with 2mm spaced holes/connectors. The dimension of this socket is 10mm long x 4mm high.

- The plug that connects into the HP48xx calculator is a single in line socket 1x4 way with 2mm spaced holes/connectors. The dimension of this socket is 8mm long x 2mm high.

- The cable wires are soldered to the relevant connectors on the socket as shown in Fig 1 for the HP49G and Fig 2 for the HP48xx calculators.

- The moulds are manufactured from aluminum (although any metal can be used) to the dimensions shown in Fig 3 (HP49G) and Fig 4 (HP48xx). You require 2 mould pieces (top and bottom). Before using the moulds, spray a releasing agent on the cavity surface of the mould so that the moulds are easy to separate after resin has been injected into the cavity and set.

- Place the socket with the cable into the mould, ensuring correct orientation of the socket and that the front of the socket is flush with the end of the mould. Clamp the moulds together. Inject fibre glass resin or other similar product into the mould cavity (back of the plug by the cable) and leave to set.

- If the resin should run out by any of the joints, seal the joints with plasticine, sticky tape or something similar and try again.

- Once the resin has set and hardened, separate the moulds and pry out the plug from the mould. Try and insert this plug into the calculator gently without using to much force the first time you insert the plug into the calculator. If the plug goes in without any problems then you are done. If you struggle to fit the plug into the calculator, you might have to file the plug to fit into the calculator recess.

Top side of calculator (HP49G)

You might have to file chamfers at the bottom of the plug that goes into the calculator and should look similar to the sketch below:

Top side of calculator (HP48xx)

Fig 1 Cable Connections (HP49G)

Solder Side

(Back)

Top

Front

1

5

10

6

7

Back (DB25 Female)

1

13

14

25

Shield

Calculator Plug

Pin 1 - Shield

Pin 2 - TX

Pin 3 - RX

Pin 4 - GND

Solder wire from pin 1 to DB25 metal casing.

Pin 2 (DB25) is RX

Pin 3 (DB25) is TX

Pin 7 (DB25) is GND

Solder Side

(Back)

Shield

Top

Front

1

5

10

6

Back (DB9 Female)

9

5

1

6

Solder wire from pin 1 to DB9 metal casing.

Pin 2 (DB9) is TX

Pin 3 (DB9) is RX

Pin 5 (DB9) is GND

Fig 2 Cable Connections (HP48xx)

Top

Front

Solder Side

(Back)

1

7

Back (DB25 Female)

1

13

14

25

Shield

4

Calculator Plug

Pin 1 - Shield

Pin 2 - TX

Pin 3 - RX

Pin 4 - GND

Solder wire from pin 1 to DB25 metal casing.

Pin 2 (DB25) is RX

Pin 3 (DB25) is TX

Pin 7 (DB25) is GND

Top

Front

4

Solder Side

(Back)

Shield

1

Back (DB9 Female)

9

5

1

6

Solder wire from pin 1 to DB9 metal casing.

Pin 2 (DB9) is TX

Pin 3 (DB9) is RX

Pin 5 (DB9) is GND

Fig 3: Mould Manufacturing Details (HP49G)

10.0

30.0

3.0

6.0

3.0

21.0

18

18.0

3.0

Dowel

9.0

13.0

6.0

1.5

10.0

FRONT

3.2

20.0

5.0

2.0

NOTE: Groove with 1.5mm ball nose cutter x 1.5mm deep on top mould only.

All dimensions are in mm.

Material: Aluminum

Qty: 2 mould halves

NOT TO SCALE

NOTE:

This area to be mill out to dimensions as shown.

Fig 4: Mould Manufacturing Details (HP48xx)

10.0

30.0

3.0

6.0

3.0

21.0

18

18.0

3.0

Dowel

9.0

11.0

6.0

1.5

8.0

FRONT

2.2

20.0

4.0

1.0

NOTE: Groove with 1.5mm ball nose cutter x 1.5mm deep on top mould only.

All dimensions are in mm.

Material: Aluminum

Qty: 2 mould halves

NOT TO SCALE

NOTE:

This area to be mill out to dimensions as shown.

References

Source for various configurations of cable connections for the HP49G calculator.

E.Carta  - cable49g.zip - File found at http://www.hpcalc.org/hp49/docs/misc/cable49g.zip

Cable connections for the HP48xx calculator

Return-Path: <garry.hayne@knuut.de>

Message-ID: <000501befbb0$15445c10$9601a8c0@shiva>

From: "Garry Hayne" <garry.hayne@knuut.de>

To: <eric@hpcalc.org>

Subject: Homemade PC to HP49 Cable

Date: Fri, 10 Sep 1999 19:15:28 +0200

MIME-Version: 1.0

X-MSMail-Priority: Normal

X-Mailer: Microsoft Outlook Express 5.00.2014.211

X-MimeOLE: Produced By Microsoft MimeOLE V5.00.2014.211

Status: RO

Hello

If you think this might be helpful please publish as needed:

Here is an easy way to make a PC to HP49 cable if you don't need

the HP48 adapter shipped with the calculator.

what you need:

The HP 48 Adapter.

4 pins from an old motherboard (the pins used for jumpers), the pins

from a motherboard fit fine, if you cannot get hold of some, use

pieces of wire, experiment!

A soldering iron with a fine point

A 9 or 25 pin serial plug with some cable (an old serial mouse is perfect)

If you are using pins from a motherboard, first desolder them and

remove the plastic base, watch your fingers!

insert the pins with the de-soldered end into the HP48 Adapter:

-------------

|               -->pin

|HP         -->pin

|LOGO   -->pin

|              -->pin

-------------

apply a little! heat to each pin, enough to melt the solder remaining

on the pin, but not long enough to melt the plastic

if you are using wire, first apply a little solder to the ends and then

insert them

as shown, then solder as above.

now solder the wires from the serial plug:

Adapter              Serial Plug (9 pin)       Serial plug 25 pin

--------

|     -->pin.........SHIELD....................SHIELD

|HP   -->pin.........Pin 3 (Out TX)............Pin 2 (Out TX)

|LOGO -->pin.........Pin 2 (In  RX)............Pin 3 (In RX)

|     -->pin.........Pin 5 (GND)...............Pin 7 (GND)

--------

Isolate the connection with tape (each pin).

Now connect this cable to one end of the HP49->HP49 cable supplied with the

calculator

plug the other end in your hp49 and away you go.

I have just updated my rom from 1.05 to 1.1 using this cable. Try it, it's

not all that

difficult, or wait until you get one from HP;) as usual this is your

responsibility, I accept

no responsibility for failure.

Garry Hayne

__________________________________________________________________

Article 1929 of comp.sys.handhelds:

Path: en.ecn.purdue.edu!noose.ecn.purdue.edu!mentor.cc.purdue.edu!purdue!tut.cis.ohio-state.edu!zaphod.mps.ohio-state.edu!uakari.primate.wisc.edu!aplcen!unmvax!pprg.unm.edu!topgun!mustang!nntp-server.caltech.edu!piglet!madler

From: madler@piglet.caltech.edu (Mark Adler)

Newsgroups: comp.sys.handhelds

Subject: Re: HP-48 Connectors wanted

Message-ID: <1990Oct21.215046.10775@nntp-server.caltech.edu>

Date: 21 Oct 90 21:50:46 GMT

References: <271fa1da.10b3@petunia.CalPoly.EDU>

Sender: news@nntp-server.caltech.edu

Organization: California Institute of Technology, Pasadena

Lines: 22

I have some information on connectors for the HP-48, but I have not found a

source yet.  Here's the info ...

Mfr:  DuPont Electronics

Specs:  Connectors are part of the "Minitek System".

Description "Crimped-wire receptacle housing headers".

Holes are on 2mm (0.079 inch) centers.

Posts are 0.002 inch (0.05 mm) square.

Posts are 0.002 inch (0.05 mm

Parts:  For each cable, you will need

1 ea  #69305-004  Single-row housing, 4-pin header

4 ea  #77138-001  Crimp terminals for 20-30 ga wire

Newwark couldn't even find them to order them.  If anyone has better luck,

please let me know.

Mark Adler

madler@piglet.caltech.edu

__________________________________________________________________

Source for DIL or SIL 2mm sockets

Connector sockets was purchased from RS Components. You require the connector with a 2mm pitch.

In their CD-ROM catalogue the connector information can be found as follows:

Choose:

CONNECTORS - MULTIPOLE PCB

RS PCB Headers and Sockets

Select "Virtual Page" to view.

Look under "Single Double Top Entry Sockets

The part numbers are:

4way Single Row - 		Part No: 251-8193

5way Double Row - 		Part No: 251-8222

The above was not available in South Africa. What I manage to find was the following:

20way Double Row - 	Part No: 131-9995

I cut pieces off this length make the socket I required. The final dimensions should be 10 mm long.

20way Single Row - 	Part No: 131-9894