# The development library

## Table of contents

The development library	1

1	Table of contents	2

2	Introduction	3

3	The tools of the development library	4

3.1	APEEK	4

3.2	PEEK	4

3.3	POKE	4

3.4	A	4

3.5	A	4

3.6	RAM	4

3.7	AH	4

3.8	HA	5

3.9	CD	5

3.10	CD	5

3.11	H	5

3.12	H	5

3.13	SH	5

3.14	HS	5

3.15	SREV	5

3.16	MAKESTR	6

3.17	SERIAL	6

3.18	S2	6

3.19	XLIB~	6

3.20	CRC	6

3.21	S~N	6

3.22	R~SB	7

3.23	SB~B	7

3.24	LR~R	7

3.25	LC~C	7

3.26	COMP	7

3.27	ALG	7

3.28	PRG	7

3.29	LST	8

4	CRLIB	9

4.1	Extension program	9

5	ASM	11

5.1	Introduction	11

5.2	Saturn ASM mode	17

5.3	ARM mode	26

5.4	System RPL mode	32

5.5	Examples of program using the MASD compiler	34

6	ASM	38

7	ARM	39

8	The Entry point library: extable	40

8.1	nop	40

8.2	GETNAME	40

8.3	GETADR	40

8.4	GETNAMES	40

## Introduction

Built in the HP49+ stands a set of function not accessible to the user by default. This library contains lots of low level development tools mainly aimed at system RPL and assembly development.

In order to enable this library, you must attach it (256 ATTACH).

Note: You can also set the flag –86. This will cause the library to attach itself on the next warmstart.

Note: When the library is attached, it appears in the APPS menu.

Note: The tools and programs in this library are extremely powerful, and misusing them may cause memory lost.

## The tools of the development library

### APEEK

Address PEEK command: Read the address stored at an address.

Example: #80711h APEEK returns the address of the home directory.

Level 1		->	Level 1

Binary integer	->	Binary integer

### PEEK

Memory read: reads nibbles from a specified address in memory.

Note: Due to bank switching, the data read from address #40000h to #7FFFFh may not be accurate.

Level 2			Level 1					->	Level 1

Binary integer (address)	Binary Integer (number of nibbles to read) 	->	String

### POKE

Memory write command: Writes nibbles in memory.

Note: you can not write data in the Flash ROM using this command.

Note: Writing data in memory randomly will cause memory lost.

Level 1					Level 2			->

Binary integer (Address where to write)	String (Data to write) 	->

### PEEKARM

Memory read: reads nibbles from a specified address in memory in the ARM address space.

Level 2			Level 1					->	Level 1

Binary integer (address)	Binary Integer (number of byte to read) 	->	String

### POKEARM

Memory write command: Writes bytes in ARM memory address space.

Note: you can not write data in the Flash ROM using this command.

Note: Writing data in memory randomly will cause memory lost.

Level 1					Level 2				->	Level 1

Binary integer (Address where to write)	String (Data to write in hex) 	->

### A

Address out: Returns the object stored at a specific address.

Level 1		->	Level 1

Binary integer	->	Object

### A

Get Address: Returns the address of an object.

Level 1		->	Level 1

Object		->	Binary integer

### RAM

Improved NEWOB: This command makes a copy of an object in RAM, wherever the object is.

This commands allows you to copy a ROM object in RAM.

Level 1		->	Level 1

Object		->	Object located in RAM

### AH

Address to string: Returns the hex representation of an address (you can then use this with the POKE command).

The hex representation of an address is a 5 character string where the address is written backwards.

Level 1		->	Level 1

Binary integer	->	string

### HA

String to address: Returns the address represented by a 5 character string.

The hex representation of an address is a 5 character string where the address is written backwards.

Level 1		->	Level 1

String		->	binary integer

### CD

Code to hex: Returns the hex representation of a code (Assembly program) object.

Level 1		->	Level 1

Code		->	string

### CD

hex to Code: Returns the code (Assembly program) object represented by an hex string.

A hex string is a string that only contains the characters ‘0’ to ‘9’ and ‘A’ to ‘F’.

Level 1		->	Level 1

String		->	Code

### H

Object to hex: Returns the hex representation of an object.

Level 1		->	Level 1

Object		->	string

### H

hex to object : Returns the object represented by a hex string.

A hex string is a string that only contains the characters ‘0’ to ‘9’ and ‘A’ to ‘F’.

Note: if the string does not represent a valid object, this can corrupt your memory;

Level 1		->	Level 1

String		->	Object

### SH

String to hex: Returns the hex representation of the characters of a string.

Example: “A” SH  “14”

Level 1		->	Level 1

String		->	String

### HS

hex to String : Returns the string which data are represented by a hex string.

A hex string is a string that only contains the characters ‘0’ to ‘9’ and ‘A’ to ‘F’.

Example: “14” HS  “A”

Level 1		->	Level 1

String		->	String

### SREV

String reverse: gives the mirror image of a string.

Example: “14” HS  “A”

Level 1		->	Level 1

String		->	String

### MAKESTR

Create a string of the given size.

Example: 10 MAKESTR -> “ABCDEFG<cr>AB”

Level 1		->	Level 1

Real		->	String

### SERIAL

Retrieve the calculator serial number

Level 1		->	Level 1

->	String

### S2

Decompile an object in system RPL mode.

Example: << >> S2 -> “!NO CODE !RPL :: x<< x>> ; <cr>@”

Level 1		->	Level 1

Object		->	String

### XLIB~

Convert reels to an XLIB.

Level 2	Level 1	->	Level1

Real	Real	->	Xlib

Binary	Real	->	Xlib

Real	Binary	->	Xlib

Binary	Binary	->	Xlib

### CRC

CRC computation: gives the CRC of a library or a string.

This command gives you the CRC of the data in a library object or string (the CRC computation starts on the size of the object and finishes 4 nibbles before the end of the object)

Level 1		->	Level 1

String/Library	->	System integer

### S~N

String to name conversions: This command converts strings to names and names to strings.

This command allows you to create invalid names.

Note: Do not purge or move the null directory in home. Do not modify data in this directory.

Level 1		->	Level 1

String		->	Global name

Global name	->	String

### R~SB

Real to System binary conversions: This command allows to convert system binary to real and real to system binary.

Level 1		->	Level 1

Real		->	System binary

integer		->	System binary

System binary	->	Real

### SB~B

Binary integer to System binary conversions: This command allows to convert system binary to binary integer and binary integer to system binary.

Level 1		->	Level 1

Binary integer	->	System binary

System binary	->	Binary integer

### LR~R

Long real to real : This command allows to convert long real to real and real to long real.

Level 1		->	Level 1

Long real	->	Real

Real		->	Long real

### LC~C

Long complex to complex: This command allows to convert long complex to complex and complex to long complex.

Level 1		->	Level 1

Long complex	->	Complex

Complex	->	Long complex

### COMP

Composite out: This is equivalent to the RPL LIST command, but it also works on Program and Symbolic objects.

Level 1			->	Level n+1..2	Level 1

List/Program/Symbolic	->	Objects		n (real)

### ALG

Create symbolic: This is equivalent to the RPL LIST command, but it creates a symbolic object.

Note: this command will also convert a program or a list in a symbolic object.

Level n+1..2	Level 1			->	Level 1

List/Program/Symbolic	->	Symbolic

Objects		real (n)			->	Symbolic

### PRG

Create program: This is equivalent to the RPL LIST command, but it creates a program object.

Note: this command will also convert a symbolic or a list in a program object.

Level n+1..2	Level 1			->	Level 1

List/Program/Symbolic	->	Program

Objects		real (n)			->	Program

### LST

Create symbolic: This is equivalent to the RPL LIST command, but it can also convert a program or symbolic in a list.

Level n+1..2	Level 1			->	Level 1

List/Program/Symbolic	->	List

Objects		real (n)			->	List

## CRLIB

Create library command.

A library is one of the most complex object in the HP49. One of the basic uses of a library is to group all the files of an application.

In order to create a library, you must store in a directory all the variables that will be part of this library. Then, you must store configuration information in some special variables.

The $TITLE variable must contain a character string defining the title of the library. This string must be less than 256 characters long. The first five characters will be used for the name that is shown in the library menu.

The $ROMID variable must contain the library number or your library. This number must be in the range 769 to 1791. In order to avoid conflicts, you should go to www.hpcalc.org to check whether the number is already in use.

This variable may contain either a real or an integer.

The $CONFIG variable contains the library configuration object which is run at warmstart. The basic action that this program should perform is to attach the library to the home directory. Placing a real or an integer in the $CONFIG variable will cause the CRLIB command to generate a default CONFIG object. This Program must leave the stack intact and is not allowed to produce errors.

The $VISIBLE variable contains a list of all the variables in the current directory that you want to have visible in the library menu.

The $HIDDEN variable contains a list of all the variables in the current directory that you want to have invisible in the library. They are generally subprograms of your application.

The $EXTPRG variable contains the name of the extension program of the library. This program must be either a visible or an hidden object of the library. See the Extension program for more information.

Then, once you have specified the required variables, you can type CRLIB to create the library.

->	Level 1

->	Library

### Extension program

It is possible to enhance some of the statistics menu using a user library. The HP49 does not provide every possible functions in every area, but let you customise the built in menu in order to add your functions 'as if they were built in'.

Example: customise the main statistic menu.

Go in RPL mode (MODE, +/-, ENTER ) and attach the development library (256 ATTACH).

In a directory, create the following variables:

$ROMID	1324

$CONFIG	1

$TITLE		"Statistic enhancements"

$VISIBLE	{ ABOUT }

$HIDDEN	{ MessageHandler }

$EXTPRG	'MessageHandler'

ABOUT		"This library is a statistic enhancement example"

MessageHandler

<<

IF DUP 1 R~SB ==

THEN

SWAP

{ { "7.New entry" << "My Stats" 1 DISP 7 * FREEZE >> } } +

SWAP

END

>>

Create the library (CRLIB) and store it in an extension port (0 STO)

Now, run the statistic menu (Shift 5)!

How does it work?

Each time the stat menu pops up, the HP49 executes every Extension Program of the library in the system. This extension Program takes on the stack a message number (and let it on the stack!). Each message number has a specific meaning, as described below.

Here are the I/O expected for the Extension program for different menus:

APPS menu

Input: { { "String" Action } ... } ZERO

Output: Modified list ZERO

Main Statistic menu

Input: { { "String" Action } ... } ONE

Output: Modified list ONE

Hypothesis statistic menu

Input: { { "String" Action } ... } TWO

Output: Modified list TWO

Confidence interval statistic menu

Input: { { "String" Action } ... } THREE

Output: Modified list THREE

Finance menu

Input: { { "String" Action } ... } FOUR

Output: Modified list FOUR

Numeric solver menu

Input: { { "String" Action } ... } FIVE

Output: Modified list FIVE

## ASM

The Machine Language and system RPL Compiler (Masd)

### Introduction

#### Warnings

The operating system can not control what a low level program is doing, therefore, any programming error is likely to cause the calculator to crash (with eventual memory lost). A careful developer will always save his source code in the internal flash rom or port 1 for protection before trying his programs,

This document does not intend to be a programming course, it just presents the syntax of the compiler. Ample resources are available on the web (www.hpcalc.org) to learn how to program the Saturn CPU in assembler, how to program in system RPL or how to program in ARM assembly.

With the instruction of the new ARM based series of calculator, some new things have been included that are not backward compatible with previous calculators. The careful programmer should be weary of this.

#### Starting Masd

To compile a program, put the source code on the level 1 of the stack and type  (the development library must be attached) or use the  menu of the Development library.

If you have a new version of MASD packaged as a library 259, the command to type is  (note the lowercase).

#### Modes

Masd can be used to compile program in 3 different languages: Saturn ASM, ARM ASM and System RPL. Although some things are common to all modes, some are not. As a programmer, you should always know what mode you are in.

Compilation directives instruction are used to switch from one mode to another:

 (switch to Saturn ASM mode, referred in the rest of this document as the Saturn mode)

 (switch to System RPL mode)

 (switches to ARM ASM mode)

In addition, in RPL mode,







Switches from RPL mode to Saturn mode (and generates an assembly program object)

#### Syntax

Masd expects a character string (called source) on the stack level 1.

A source is a set of instructions, comments, and separation characters and ends with a carriage return and an  character.

Masd is case sensitive, so be careful, as «  » and « » are two different things for MASD.

Separation characters are those with an ASCII number below 32. They include spaces, tabs, line feed and carriage return.

In Saturn mode, some instructions need a parameter. Separation characters between an instruction and the parameter are spaces, tabs, and points. Therefore  can be used instead of  .

In ARM mode, parameters for the instruction are separated by spaces and comas.

In Saturn or ARM mode, comments can be placed everywhere and begin with  or  and finish at the end of the current line.

In RPL mode, comments are delimited by ‘‘ ‘’ as isolated characters and can be multi line. A line that starts with a ‘’ on the first character will also be considered a comment.

Directives change the way Masd interprets your source. Theses directives begin with a  and will be explained later.

#### Errors

If Masd detects one or more syntax error, it will push a list describing all errors on the stack. The  command can help you make sense of that list, point you on the errors and let you correct them.

Masd will report a maximum of 16 errors before stopping compilation.

The  command takes 2 objects as arguments:

The original source code (level 2)

The error list generated by MASD (level 1)

Normally, you should compile using a process similar to:  (this is what the  command does BTW). Most peoples will just type the  command followed, if error by the command.

##### Format of the error list:

It’s a list of at most 16 sub-lists.

Each sub-list contains 3 system-binary and 1 global-name.

The first system binary is an error message number.

The second is an extra system binary used to indicate how ‘too long’ a jump is.

The third one is the position in the source where the error is.

The global name is either a NULLNAME if the error was in the main source or the filename of the buggy source.

##### Error messages

|  | The file is not a valid source or macro. (must end with a @) |
| --- | --- |
|  | You can not do this operation as you are limited to a certain amount of them (for example, you can not have more than 64 simultaneous skips) |
|  | Unknown instruction |
|  | Incorrect field |
|  | An integer between 0 and 15 is expected |
|  | An integer between 1 and 16 is expected |
|  | An integer between 1 and 8 is expected |
|  | A label is expected |
|  | An hexadecimal number is expected |
|  | An decimal number is expected |
|  | This object can not be located |
|  | This name is already in use |
|  | A { character was expected |
|  | A } character was expected (this can happen if you do not close all the open skips for example) |
|  | A [ or ] character was expected |
|  | This can not be done |
|  | This expression is invalid |
|  | This jump is above the limit of the instruction (use a different type of jump) |
|  | You can not do this thing here because you are creating a matrix object |
|  | You can not do this operation in a DEFINE |
|  | No comments. |
|  | In ARM mode, constants must be representable on 8 bit with an even number of rotation |

#### Links

Links are secondary source files that MASD can be directed to compile (equivalent to the {$I} directive in PASCAL and #include in C). As there is no linking phase with MASD (like in C), a multi source project will usually have the form of a main source file that contains a certain number of links.

An example of main source would be:













When a link call is encountered, Masd suspends compilation of the current source, compiles the new source and then continues compiling the first one.

Program and data in the final object will be in the order in which MASD encounter the links.

Syntax in ASM and ARM mode:

			links the file called FileName.

Syntax in RPL mode:

	links the file called FileName.

Note 1: A link can call other links

Note 2: You can not use more than 64 links in your project

Note 3: To know how Masd looks for files, see the File search section

Note 4: Links are useful to cut projects in independent parts to allow fast and easy access to source code

Note 6: It is beneficial to place all constants definition at the beginning of the compilation process as this will speed up compilation and give more flexibility

#### Labels

A label is a marker in the program. The principal use of labels is to determine jump destinations.

A label is a set of less than 64 characters different from space, ‘’, ‘‘, ‘’ and ‘’. A label begins with a star ‘’ and ends with a separation character.

Syntax in ASM and ARM mode:

		is the BigLoop label declaration.

Syntax in RPL mode:

	is the BigLoop label declaration.

Be careful about upper and lower cases!

Three types of labels can be used:

- Global labels

A global label is a label that can be used everywhere in the project, like global variables in Pascal or C.

- Local labels

A Local lab is a label that is only accessible in a local section like local variables in Pascal or C.

A local section starts at the beginning of a source, after a global label or after a link (see link section)..

A local section finishes at the end of a source, before a link or before a global label.

A local label is identified by a ‘’ as the first character.

- Link labels

A link label is a label that exists only in the link where it is declared, like a private clause in Object Pascal.

A link label is identified by a ‘’ as the first character.

Note 1: In projects, using less global labels is better because a global label takes longer to compile and because it gives a better program structure. A good habit is to use global labels to cut the program in subroutines, and to use local labels inside these subroutines.

Note 2: The command line editor is able to find labels in a source. See the  selection in the command line

 menu.

Note 3: labels in system RPL should only be used by people who know what they are doing. They are only used for fixed address program (absolute mode) which is pretty advanced programming.

Note 4: Labels can not be given the same name as constants.

#### “extable”

“extable” is an external library that contains a list of constants.

This list can be used by the Masd as a basic list of constants and is especially useful to the System RPL programmer as most entry points are defined there (like TURNMENUOFF for example).

In addition, it also contains a set of supported constants and ASM entry points for the ASM programmer.

Please read the extable section in this document to find more information about this library.

#### Constants

Constants are a way for the user to associate a value to an alphanumerical name. This is extremely useful as it makes programs much easier to read and makes them more portable. One of the most popular ways to use constants is to represent memory address for storage of variables.

For example, instead of typing  every time it is needed, it is better to declare  at the beginning of the project and then to type  when needed (it is more portable, more readable and less likely to cause errors).

You can create a constant in ASM or ARM mode by doing:

 or

 or



In RPL mode, the only valid way to define a constant is:



ExpressionHex is either an hexadecimal number or an expression (starting with a char that can not be confused with the start an hex number (..,..). An expression starting with a hexadecimal number can be typed with a leading , an expression starting with a decimal number can be typed with a leading # character. For an expression starting with a constant that starts with a .. or .. character, you should put the constant in brackets.

Note 1: A constant cannot be given the same name as a label.

Note 2: The name of a constant follows the same rules as the name of a label.

Note 3: A constant value is stored on 16 nibbles.

Note 4: having constants starting with something that can be interpreted as a hex number, or an ARM register is not a good idea as the compiler might get confused. For example:  will generate an error on  as the compiler will interpret the mov as a mov from SP to R4.

Masd introduces a ‘constant pointer’ called CP which helps to define constants. CP is defined by:



CP is defined on 5 nibbles, its default value is 80100 (an area of memory that can be used freely by programmers).



Declares a constant with the current CP value and then increase CP by Increment.

Note 1: in ASM and ARM mode,  is also valid

Note 2: Increment is a hexadecimal value, to use a decimal value, put a leading #.

For example, if CP equals to $10



Defines a Foo constant with a value of $10 and then change the value of CP to $15.

Note:

Several constants can be defined at once using CP.



Defines N constants  with a value of CP+x*Inc and then changes the CP value to CP+N*Inc.

By default, Inc is a decimal number or an expression that can be immediately evaluated.

These features are extremely useful to define area of memory for storage of ASM program variables.

Note 1: If the entry point library (see related section) is installed on your calculator, all the values in the constant library will be available in your programs the same way than constants are.

Note 2: you can define a constant in your program to override the value of an entry in the equation library.

#### Expressions

An expression is a mathematical operation that is calculated at compilation time.

Terms of this operation are hexadecimal or decimal values, constants or labels.

An expression stops on a separation character or a ‘]’.











are correct expressions (provided that the entry point library is installed).

Notes:

- A hexadecimal value must begin with a .

- A decimal value may begin with a  or directly a number.

- A  or  equals the offset of the current instruction in the program (This value has no meaning in itself, but may be used to calculate the distance between a label and the current instruction). In absolute mode, this represents the final address of the instruction.

- The value of a label is the offset of the label in the program (This value has no meaning in itself, but may be used to calculate the distance between a label and the current instruction). In absolute mode, this represents the final address of the instruction.

- Entries from the EXTABLE may be used. As the EXTABLE does not have the label names limitations with operators, in ambiguous case ( may either be an addition , or an entry ‘‘), add  around the word: .

- Calculations are done on 64 bits.

- X divide by 0 = $FFFFFFFFFFFFFFFF.

- In order to avoid wasting memory, Masd tries to compile Expressions as soon as it see them. If Masd is not able to compile an expression directly, it’s compiled at the end of the compilation. In order to use less memory, it’s a good idea to define your constants at the beginning of the sources so Masd can compile expression using the constants directly.

- The only operator symbols not allowed in labels are +, -, * and /, therefore, if you want to use a symbol operator after a label, you must put the symbol between “ in order to ‘limit’ the symbol. Meaningless Example: .

- A label/constant with strange char may be ‘protected’ between “ chars.

- The evaluation stack of MASD allows you to have around 10 pending computations (parenthesis, operator priority).

- Masd only works with integers, you can represent signed value using standard 2’s complement, but be careful as all operators are unsigned.

Masd recognises the following operators

| Operator | priority | Notes |
| --- | --- | --- |
|  | 7 | Left Shift |
|  | 7 | Right shift  |
|  | 6 | Modulo (remainder of division) X%0=0 |
|  | 5 | Multiplication |
|  | 5 | Division X/0=$FFFFFFFFFFFFFFFF |
|  | 4 | Addition |
|  | 4 | Subtraction |
|  | 3 | Is smaller (true=1, false = 0) |
|  | 3 | Is greater (true=1, false = 0) |
|  | 3 | Is smaller or equal (true=1, false = 0) |
|  | 3 | Is greater or equal (true=1, false = 0) |
|  | 3 | Is equal (true = 1, false = 0) |
|  | 3 | Is different (true = 1, false = 0) |
|  | 2 | Logical and |
|  | 1 | Logical Or |
|  | 1 | Logical Xor |

Note: throughout this documentation, you will see talks about expressions that can be “immediately” evaluated. This refers to any expression that contains only number and labels/constants that have already been declared.

#### Macros and includes

If data are to be included in a project, they can be entered in hex in a source file, using .

But a simpler way is to include data from an external file, which is called a macro. The macro file must be a character string, a graphic, a code object or a list.

- In case of a string or a code, Masd includes only the data part (after the length) of the object

- In case of a graphic, only the graphic data will be included (no length, no dimensions)

- In case of a list, only the first object of the list will be included following the previous rules

The syntax in ASM or ARM mode is:



Note: To know how Masd looks for the FileName file, see the following section.

You can also include a complete object (prologue included) using  or .

In ASM or ARM mode, use  or  followed by a filename to include an object, in RPL mode, use .

#### Filename conventions

Masd sometimes needs to find a file in the HP 49 memory.

The file can be found either by specifying the file name and location, or only the file name to be search in the directory search list.

The initial directory search list contains the current directory, and all parents directory up to the HOME directory.

You can add a directory in the directory search list using  where RepName identifies a directory name using filename rules.

To specify a full path, use

 to specify HOMEDIR as the root.

 where x is a port number, to specify a port as root (note, you can not use 3 (SD card) here.

This root is followed by a list of directories, ending with the name of the file.

 specifies the BRA file in the BAR directory, stored in the FOO backup of the port 2.

 specifies the YOU file in the ME directory, in the HOMEDIR.

Note 2: You cannot have more than 16 entries in the directory search path.

#### Compilation directive

The following instruction modifies the way Masd reacts and compile things. They are valid in all modes:

|  | Add the specified directory in the search path list. |
| --- | --- |
|  | Masd will not generate a $02DCC prologue but will directly output the data. If the generated file is not a valid object, an error will be generated. |
|  | Masd will generate code when  or  are found in the source. |
|  | Masd will not generate code when  or  are found in the source. |
|  | Switch to 1-16 mode. |
|  | Switch to 0-15 mode. |
|  | Switch to RPL mode. |
|  | Switch to ASM mode. |
|  | Switch to ARM mode. |
|  | Clear the a compilation flag. |
|  | Set the a compilation flag. |
|  | Compile the end of the line if flag a is set. |
|  | Compile the end of the line if flag a is clear. |
|  | Switch to absolute mode. The program begins at the address Addr. Note: Masd always consider the prolog $02DCC and code length to be the beginning of the program even if  is set. |
|  | If in absolute mode, add whites nibbles to continue at the specified address. If not possible, errors. |
|  | In absolute mode, cause an error if the directive is not on an even address. |
|  | Masd will generate a source defining all constants and labels used in the program instead of the program. |
|  | Cause Masd to calculate all previous expressions. |
|  | Display/update compilation statistics |
|  | Causes MASD to generate debugging information (see next section for more information) |
|  | See local variable documentation in RPL mode |
|  | See local variable documentation in RPL mode |

##### The !DBGINF directive

If you put the  directive into a MASD source, the assembler not only generates your compiled object, but it also returns a string (on level 1) full of debug information. The structure of this string is as

follows:

5  DOCSTR

5  Length

5  Number of links (source files)

n*[

2  Number of characters

.. Name of link file

]

5  Number of symbols (labels and constants)

n*[

2  Number of characters

.. Name of symbol

1  Type: 9=Label 2=Constant

for labels:    5  Address of label

for constants: 16 Value of constant

]

5  Number of source->code associations

n*[

5  Offset in code (this list is sorted by offset)

2  Number of link this instruction comes from

5  Character offset in link where this instruction starts

]

Notes:

- If the source string is unnamed, i.e. in TEMPOB, the number of links is 00001 and the number of characters is 00, immediately followed by the symbol table.

- The label symbol table is, according to Cyrille, supposed to be an *offset* table. However it seems that the current (1.19-6) MASD has got a bug which makes it put *addresses* into this table. The "associations" table correctly contains offsets.

This instruction is intended for the case where someone decides to create a source level debugger.

### Saturn ASM mode

This section is only applicable to the Saturn ASM mode.

#### CPU architecture

This section has for only aim to familiarise experienced ASM programmers to the Saturn architecture, not to teach anyone to program in Saturn ASM.

The Saturn CPU has 12 main registers:

A, B, C, D, R0, R1, R2, R3 and R4 are 64 bits register (see description bellow),

D0 and D1 are 20 bits pointers (you can only access memory through them, the Saturn is little endian),

PC, 20 bit program counter.

In addition, there are 16 flags ST0 to ST15 (12-15 being reserved for the system) 1 bit register accessible separately, a carry that is set when operation overflow or tests are validated and can be tested using the GOC (Go On Carry) and GONC (Go On No Carry) jump instruction, a decimal/hexadecimal mode (SETHEX and SETDEC) that affects the way + and – instructions on the A, B, C and D register works (default Is HEX), and a 8 level return stack for GOSUBs (and RTN).

##### 64 bits register

Most operations on 64 bits register will act on a specific “field”. A field is a division in a 64 bit register.

If this represents the 16 nibbles of a 64 bit register, the fields cover the register as follows:

| F | E | D | C | B | A | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | P |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | WP |  |  |  |  |  |  |  |  |  |  |  |
| S | M |  |  |  |  |  |  |  |  |  |  |  | XS | B |  |
|  |  |  |  |  |  |  |  |  |  |  | A |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |

The P field location depends of the value of the 4 bit P register (ie: you can move it), and so does the WP field.

Please look at the instruction set to see what instructions are available to the programmer.

I usually write “Rf” to indicate a register (uppercase) and a field (lowercase) as in .

In addition, in the new simulated Saturn architecture, 7 new fields F1 to F7 have been introduced.

You can define the field mask by using the  where n is a number between 1 and 7 to define the field Fn using the mask in Cw as in this example:









A is now equal to:



ie: the instruction on F fields equate to:

reg1= (reg1 & ~ mask) | ((reg1 & mask) operation (reg2 & mask))

These new fields are available for all instruction that previously used the so called ’f’ encoding and includes the following instruction:

Reg=Reg&Reg.f, Reg=Reg!Reg.f, DATx=Reg.f, Reg=DATx.f, Reg=Reg+Cte.f, Reg=Reg-Cte.f, RegSRB.f, RReg=Reg.f, Reg=RReg.f and RegRRegEX.f.

##### Other notes

You should read documentations on the internal structure of RPL objects (www.hpcalc.org has good documentation)

D0, D1, Ba and Da are used by the system (next RPL instruction pointer, RPL stack pointer (@@object on level 1 of the stack), start of free memory and free memory in 5 nibble blocks). The SAVE instruction will save these registers in dedicated memory areas, the LOADRPL instruction will restore them and continue the execution in the system.

Please consult documentation on memory map for more information.

##### New instructions

In addition to the F fields, the following new instructions have been created:

r=s.f, r=r+s.f, r=r-s.f, r=r*s.f, r=r/s.f, r=r%s.f (modulo) r=-s-1.f (logical not), r=-s.f (mathematical not), r=r<s.f (left shift), r=r>s.f (right shift), r=r^s.f (logical xor).

r=1.f (alias for r=r/r.f) has also been created.

Note 1: any combination of the A, B, C and D registers can be used (notted r and s here)

Note 2: all field (including F1-F7 fields) are valid

Note 3: Masd will always choose the shortest version of the instruction (ie:  will use the standard  encoding AND affect the carry)

Note 4: the carry is not affected by these instructions.

The following other new instructions have been added (see description in the ASM syntax section):

|  |  |  |  |
| --- | --- | --- | --- |
|  |  |  |  |
|  |  |  |  |
|  |  |  |  |
|  |  |  |  |
|  |  |  |  |
|  |  |  |  |
|  |  |  |  |
|  |  |  |  |

#### Skips

Skips are a first step from ML to a third generation language, even if they are only another way to write ASM instructions.

Skips are wonderful as they allow you to:

- structure your program

- avoid using goto’s

- make programs and piece of code that can be easily copied and past (because there is no label)

The foundation of Skips is the Block structure.

A block is enclosed in  and , and can be nested within another block.

The following instructions deal with blocks.

| SKIPS instructions | Equivalents |
| --- | --- |
|  ... | Defines a block (generates no code) |
|  ...  |  ...  |
|  ...  |  ...  |
|  ...  |  ...  |
| ...  |  ...  |
| ...  |  ...  |
|  ...  | GONC .S ...  |
| Test ...  | Test GOYES .S ...  |
| Test  ...  | Test GOYES .S ...  |
| Test  ...  | /Test GOYES .S ...  |
| Test  ...  | /Test GOYES .S ...  |
|  ...  | GOSUB .S ...  |
|  ...  | GOSUBL .S ...  |
|  ...  |  ...  (to create a character string) |
|  ...  |  ...  (to create a code object) |
|  ...  | S ...  (to create a ‘prolog – length’ object) |

/Test is the opposite of Test. For example if Test is , /Test is . The test instructions dealing with the hardware register (, , , , , , , ,  and ) cannot be inverted.

Once blocks are defined, special instructions can be used in them. These instructions called EXIT and UP allow to jump to the end or to the beginning of a block.

| These instructions | are equivalent to |
| --- | --- |
|           |           |

Note: in Saturn mode do not make confusion between EXIT and UP instructions, which are GOTOs, and EXIT and UP after a test, which are GOYES’s.

EXIT and UP can jump to the beginning or to the end of an upper-level block by specifying the number of blocks to exit, after the UP or EXIT instructions.

| These instructions | Are equivalent to |
| --- | --- |
|           |           |

Note 1: EXIT1 is equivalent to EXIT, and UP1 is equivalent to UP.

Note 2: the same rules apply in ARM mode: EXITGE3 for example is a BGE for the exit label 3 blocks down

Using SKELSE, SKEC, SKENC, SKLSE instructions, two blocks create an IFNOT-THEN-ELSE structure.

| These instructions | Are equivalent to | Or in high-level language |
| --- | --- | --- |
|            |            |            |

Note: SKELSE places a GOTO between the 2 bocks, SKEC places a GOC, SKENC a GONC and SKLSE places nothing.

Notes:

s are compiled directly when encountered while  and block opening are compiled later on. You can not have more than 64 pending EXITs and block opening simultaneously.

#### Tests

A test instruction () may be followed by:

- A ,  or  instruction

- A -> { or  { instruction. In this case, the test is inverted and a skip block is open.

- A  or  instruction.

- A  or   instruction. In this case, a skip block is open.

- A , , , , or . In this case, the test is inverted and a proper jump instruction is generated (ie:  is compiled as .

- A  or .

#### SATURN instructions syntax

In this section:

x is a decimal number between 1 and 16. An expression can be used if its value can be determined at the first encounter.

h is a hexadecimal digit.

a is a decimal number ranging from 1 to 16 or a 0 to 15 number depending of the current mode (0-15 or 1-16). An expression can be used, if it’s value can be determined at the first encounter.

f is a field A, B, X, XS, P, WP, M, S, F1, F2, F3, F4, F5, F6 or F7.

Reg is a working register A, B, C or D.

RReg is a scratch register R0, R1, R2, R3 or R4.

Exp is an expression.

Cst is a decimal constant. An expression can be used if its value can be determined at the first encounter.

DReg is a pointer register D0 or D1.

Data is memory data pointed by D0 or D1. It means  or .

Note: For instructions that use two working registers, instruction using the pairs A-B, B-C, C-D and A-C are smaller and faster (if the  fields are not used).

For instructions like Reg1=Reg1… you can write only Reg1… Example:  is the same as .

| Syntax | Example |  | Notes |
| --- | --- | --- | --- |
| Reg=0.f |  |  | Sets the specific field of the register to 0 |
| Reg=1.f |  |  | Sets the specific field of the register to 1 |
| LC hhh..hhh LA hhh..hhh |   |  | The number of nibbles loaded in the register is the number of characters necessary to write the value. So LC #12 will be equivalent to LC 00C. Note: the less significant nibble is loaded in the nibble P (as in the value of the register P) of the register, the next one into nibble p+1 mod 16, and etcetera. |
| LCASC(x) chrs LAASC(x) chrs |   |  | Loads the hexadecimal value of x characters into C. x must be between 1 and 8. See note on LC instruction |
| LC(x) Exp LA(x) Exp |  |  | Loads the result of an expression into C or A, using x nibbles. See note on LC instruction |
| Reg1=Reg2.f |  |  | Copies the value of a specific field of a register into the same field of another register |
| Reg1Reg2EX.f |  |  | Exchanges the value of 2 registers on the given field. Note: this is not valid for the  fields |
| Reg1=Reg1+Reg2.f Reg1+Reg2.f |   |  | Adds the value of the specific field of one register to the other register. Note: If Reg1 and Reg2 are the same, this is a multiply by 2 instruction Note: This instruction is affected by the DEC/HEX mode only if the field is not a F field and the registers are AB, BC, CD or AC. |
| Reg1=Reg1-Reg2.f Reg1-Reg2.f |   |  | The following instructions are also available (but not on the  fields):     see note on Reg1=Reg1+Reg2.f |
| Reg=Reg+Cst.f Reg+Cst.f Reg=Reg-Cst.f Reg-Cst.f |     |  | Note 1: The Saturn processor is not able to add a constant greater than 16 to a register. If cst is greater than 16, Masd will generate as many instructions as needed. Note 2: Even if adding constants to a register is very useful, large values should be avoided because this generates a large program. Prefer another solution like LC(5) Cte A+C.A Note 3: Adding a constant greater than 1 to a P, WP, XS or S field is a bugged SATURN instruction (problem with carry propagation). Use these instructions with care. Note 4: After adding a constant greater than 16 to a register, the carry should not be tested (because you do not know if the last generated instruction generated the carry or not) Note 5: You can put an expression instead of the constant (Masd must be able to evaluate the expression right away). If the expression is negative, Masd will invert the addition in a subtraction and vice versa. Note 6: Be careful when using subtraction, it’s easy to be misled. A-5-6.A is equivalent to A+1.A, not A-11.A because the instruction is: A-(5-6).A Note 7: If using  fields, be careful if non nibble bounded masks are used. |
| RegSR.f |  |  | Shift register right by 4 bit on the specified field, set SB if bits are lost. Note: this instruction is not available on the  fields |
| RegSL.f |  |  | Shift register left by 4 bit on the specified field, set carry if bits are lost. Note: this instruction is not available on the  fields |
| Reg1=Reg1<Reg2.f Reg1<Reg2.f |  |  | Shift register left by n bits (as defined by the value of Reg2) on the specified field |
| Reg1=Reg1>Reg2.f Reg1>Reg2.f |  |  | Shift register right by n bits (as defined by the value of Reg2) on the specified field |
| RegSRB.f |  |  | Shift register right by 1 bit on the specified field, set SB if bits are lost. |
| RegSRC |  |  | Circular right shift by 1 nibble |
| RegSLC |  |  | Circular left shift by 1 nibble |
| Reg1=Reg1&Reg2.f Reg1&Reg2.f |   |  | Logical and on the specified field |
| Reg1=Reg1!Reg2.f Reg1!Reg2.f |   |  | Logical or on the specified field |
| Reg1=Reg1^Reg2.f Reg1^Reg2.f |   |  | Logical xor on the specified field |
| Reg1=-Reg1.f |  |  | Mathematical not on the specified field |
| Reg1=-Reg1-1.f Reg1=~Reg1.f |   |  | Logical not on the specified field |
| RReg=Reg.f |  |  | Sets the specified field of RReg to the value of the specified field of Reg Only A and C are valid for Reg. If f is W, the shorter encoding of the instruction is used |
| Reg=RReg.f |  |  | Sets the specified field of Reg to the value of the specified field of RReg Only A and C are valid for Reg. If f is W, the shorter encoding of the instruction is used |
| RegRRegEX.f |  |  | Exchanged the value of the specified field of RReg with the value of the specified field of Reg Only A and C are valid for Reg. If f is W, the shorter encoding of the instruction is used |
| Data=Reg.f Data=Reg.x |   |  | Write the content of the specified field of the specified register in the memory location pointed by Data register (POKE) Reg can only be A or C |
| Reg= Data.f Reg Data.x |   |  | Read the content of the memory location pointed by Data register in the specified field of the REG register (PEEK) Reg can only be A or C |
| DReg=hh DReg=hhhh DReg=hhhhh DReg=(2)Exp DReg=(4)Exp DReg=(5)Exp |       |  | Change the first 2, 4 or all nibbles of the Data register with the given value |
| Dreg=Reg |  |  | Reg can only be A or C |
| Dreg=RegS |  |  | Sets the first 4 nibbles of Dreg with the 4 first nibble of Reg Reg can only be A or C |
| RegDRegEX |  |  | Reg can only be A or C |
| RegDRexXS |  |  | Exchange the first 4 nibbles of Dreg with the 4 first nibble of Reg Reg can only be A or C |
| DReg=DReg+Cst DReg+Cst DReg=DReg-Cst DReg-Cst |     |  | Note 1: The Saturn processor is not able to add a constant greater than 16 to a register but if cst is greater than 16, Masd will generate as many instructions as needed. Note 2: Even if adding constants to a register is very useful, big constants should be avoided because this will slow down execution, and generate a big program. Note 3: After adding a constant greater than 16, the carry should not be tested. Note 4: You can put an expression instead of the constant (Masd must be able to evaluate the expression right away). If the expression is negative, Masd will invert the addition in a subtraction and vice versa. Note 5: Be careful when using subtraction, it’s easy to be misled. D0-5-6.A is equivalent to D0+1.A, not D0-11.A |
| Please read the section on test above for information on what MUST follow a test instruction. f can NOT be a  field. |  |  |  |
| ?Reg1=Reg2.f |  |  |  |
| ?Reg1#Reg2.f |  |  | The HP special character can also be used |
| ?Reg=0.f |  |  |  |
| ?Reg#0.f |  |  | The HP special character can also be used |
| ?Reg1<Reg2.f |  |  |  |
| ?Reg1>Reg2.f |  |  |  |
| ?Reg1<=Reg2.f |  |  | The HP <= character can be used |
| ?Reg1>=Reg2.f |  |  | The HP >= character can be used |
| ?RegBIT=0.a ?RegBIT=1.a |  |   | Test if a specific bit of A or C register is 0 or 1 Reg must be A or C |
|  |  |         | Sets Aa or Ca to the address of the next instruction Set PC to the value contained in Aa or Ca Exchange the value of PC with register Aa or Ca Sets PC to the value read at the address contained in Aa or Ca |
|  |  |           | SB, XM, SR and MP are 4 bits in the HST register. They can be set to 0 by the specific instruction and tested. SB is set to 1 by RegSR and RegSRB instruction, XM by RTNSXM instruction and SR and MP should always be 0 (hardware related stuff). HST=a sets all bits set to 1 in a to 0 in the HST register. ?HST=a test that all bits set to 1 in a are 0 in the HST register |
|  |  |           | See above. This is only valid in emulated Saturn |
|  |  |        | The HP special character can be used instead of # |
|  |  |                | GOTO is limited to 1kb jumps GOTOL can jump over 16KB of code this jump to a specific address GOSUB is limited to 1kb jumps GOSUBL is limited to &6kb jumps GOSBVL jumps to a specific address GO if Carry set (limited to 64byte) GO if no carry (limited to 64 bytes) Equivalent to SKNC { GOTO label } Equivalent to SKC { GOTO label } |
|  |  |         | Return from subroutine (GOSUB call) RTN + XM=1 RTN + set carry RTN + clear carry RTN if carry set RTN if carry not set Return from interrupt Return if test true (see test section) |
|  |  |   | Pop value from RSTK in Ca Push value from Ca in RSTK |
|  |  |     | Set the first 2 nibbles of the OUT register to the value of Cb Set the OUT register to the value of C4 Copy the IN register in Ax or Cx (bugged instruction, do not use if you do not know what you are doing) |
|  |  |       | Set the CPU in DECIMAL or HEXADECIMAL mode Deconfigure/Configure memory modules deconfigure ALL memory modules STOP the CPU waiting for an interrupt Enable/disable keyboard interrupts Reset interrupt system |
|  |  |  | Equivalent to . (& is the address of the instruction) |
|  |  |  | Equivalent to  (& is the address of the instruction) |
|  |  |   | Includes hexadecimal data in the program. Example: . |
|  |  |  | Includes hexadecimal data in reverse order. Example:  is equivalent to . |
|  |  |    | Places the value of Exp in the code, on x nibbles. |
|  |  |   | Includes ASCII data. The end of the string is the next ¢ or carriage return. Example: ¢Hello¢. To output a ¢ character, put it twice. To put a char from its number, use \xx where xx is an hex number. To put a \, put the \ chr twice. |
|  |  |         | Same as  with x=5, 4, 3 or 2. Useful to create a jump table. |
|  |  |  | Equivalent to  |
|  |  |  | Equivalent to  |
|  |  |  | Equivalent to  |
|  |  |  | Equivalent to  |
|  |  |  | Equivalent to  |
|  |  |  | Equivalent to  |
|  |  |  | Equivalent to  |
|  |  |  | Equivalent to  |
|  |  |  | Equivalent to  |
|  |  |  | Equivalent to  |
|  |  |  | Equivalent to  |
|  |  |  | Equivalent to  |
|  |  |  | Equivalent to  |
|  |  |  | Equivalent to  |
|  |  |   | Equivalent to  |
|  |  |   | Equivalent to  |
|  |  |  | Equivalent to  (only if debug is on) |
|  |  |  | Equivalent to  (only if debug is on) |
|  |  |  | Equivalent to  |
|  |  |  | Equivalent to  |
|  |  |  | Equivalent to  |
|  |  |  | Equivalent to  |
|  |  |  | Equivalent to  |
|  |  |  | Equivalent to  |
|  |  |  | Equivalent to  |
|  |  |  | Equivalent to  |
|  |  |  | Equivalent to  |
|  |  |  | Set carry if native function xx is undefined, clear it if defined. |
|  |  |  | Sets bits in the HST register (XM=1, SB=1, SR=1 and MP=1 are also available). Note: the program  will set ST0 to 0 if the calculator is non emulated and to 1 if it is emulated. |
|  |  |  | Test for HST bits. See HST=1.x comments |
|  |  |  | See section 5.2.1 |
|  |  |  | Turns the calculator off. |
|  |  |  | Simulates a LOOP (A=DAT0.A D0+5 PC=(A)). |
|  |  |  | (C[A]) kbd peeks with immediate rtn CS if keydn. Also - Sets DOUSEALARM flag if [ON][9] sequence. Entry: P=0, HEX Mode, C[A]: #kbd peeks (loop count) |
|  |  |  | Abstract: Creates a hole in the tempob area of the specified size + 6 (5 for the link field and 1 for marker nibble). Sets the link field of the "hole" to size+6 and adjusts AVMEM, RSKTOP and TEMPTOP. Entry Conditions: RPL variables in system RAM C(A) contains desired size of hole Exit Conditions: carry clear, RPL variables in system RAM D1 -> link field of hole, D0 -> object position B(A), C(A)= desired size+6 Error Exits: Returns with carry set when there's not enough memory to create a hole of size+6. |
|  |  |  | Entry: C[A]: d ;d=Beep duration (msec) D[A]: f ;f=Beep frequency (hz) P=0 Exit: CARRY:0 |
|  |  |  | Enables the remote control mode (ON+R). |
|  |  |  | Copy serial number to address pointed to by D1 in Saturn memory. |
|  |  |  | Purpose: Send byte to IR printer Entry: A[B]: Byte Exit: CC, P=0, Byte Sent Alters: P:0, CARRY:0, SETHEX. |
|  |  |  | Abstract: Used to move block of memory to higher address. No check is made to ensure that the source and destination do not overlap. Code is moved from from high to low addresses. Entry Conditions: D0 -> end of source + 1 D1 -> end of destination + 1 C(A) = number of nibs to move (unsigned) Exit Conditions: HEX mode, P=0, carry clear D0 -> start of source D1 -> start of destination |
|  |  |  | Abstract: Used to move block of memory to lower address. No check is made to ensure that the source and destination do not overlap. Code is moved from lower to higher addresses. Entry Conditions: D0 -> start of source D1 -> start of destination C(A) = number of nibs to move (unsigned) Exit Conditions: P=0, carry clear D0 -> end of source + 1 D1 -> end of destination + 1 |
|  |  |  | Call a function at global dword address C[0-7]&~3. The function takes should be of the form: U32 f(U32 pc, Chipset* c) { /* put your code here */ return pc; } |
|  |  |  | Call a function at Saturn address C.A&~7. The function should have the following format: U32 f(U32 pc, Chipset* c) { /* put your code here */ return pc; } In RAM asm, this means that as you enter the function, pc is in R0, @Chipset is in R1 and the return address is in LP. R2 and R3 are free to use, and R0 should normally not be modified except if you want to change the PC when exiting the function. |
|  |  |  | Stops the remote control (ON+S). |
|  |  |  | Wait for (C[A]/183) ms. |
|  |  |  | Return how many columns the screen contains in Ca |
|  |  |  | Sets the RTC time from C[W] in ticks. |
|  |  |  | Set number of lines of disp0 from C[B], refresh display. |
|  |  |  | Set offset of display inside disp0 in bytes from C[X]&7FF. |
|  |  |  | Return how many lines the screen contains. In Ca |
|  |  |  | Unconfigure the 4KB block containing the top 16-line header. This will refresh the header on the display. |
|  |  |  | Emulates gettime function in ROM, and also updates the 8192Hz timer accordingly. Purpose:Get current time: (=NEXTIRQ)-Timer2 Return CS iff time appears corrupt. Entry: Timer2 Running Exit: Timer2 Running CC - A:NEXTIRQ (ticks) C:Time (ticks) D:Timer2 (sgn extended ticks) P:0, HEX CS - Same as the non-error case but the time system is corrupt because of one of: (1) TIMESUM # CRC(NEXTIRQ) -- CheckSum Error (2) TIMER2 was not running on entry. (3) Time not in range: [BegofTime, EndofTime) |
|  |  |  | Carry=1 on HP48, 0 otherwise. |
|  |  |  | Configure a 4KB block containing the top 16-line header. C.A = Start address of the block (must be multiple of 4KB). If already configured, unconfig, refresh and re-config. |
|  |  |  | Carry=1 on HP49, 0 otherwise. |
|  |  |  | Reset the calculator (including the OS). This code doesn't return, the calculator restarts at 00000000. |
|  |  |  | Force to refresh the header on the display. |
|  |  |  | - 003AA: AUTO_USER_TEST - 003A3: MANU_USER_TEST - 0039C: MANUFACTURE_TEST - Other: signed index, OS-specific, see OS_API.doc. |
|  |  |  | SD Card functions (depending on P, see HP's vgeraccess for more details. |
|  |  |  | Return port number depending on tag name. Entry: D1: name (size+chars) Exit: A[A]: port number (0-3) D1: after name Carry: clear if ok, set if wrong name |

### ARM mode

#### ARM architecture

For all user intents and purposes the ARM CPU has 16 32 bit registers noted R0 to R15 (R15 is also the program counter, R14 is the link register (ie: a BL (GOSUB) instruction copies the return address in R14 before jumping, a Return From Subroutine is performed by doing MOV PC, LR), and R13 is the Stack pointer).

Each instruction can be conditionally executed depending on the value of 5 flags.

Each instruction can be told to modify or not modify these 5 flags (add the S suffix to the instruction).

Please read the ARM ARM (ARM Architecture and Reference Manual for more information).

Please look at the ARMSAT Saturn instruction and the ARM mode documentation to see the instruction set and the rules of calling ARM code from Saturn code.

#### Skips

Skips are a first step from ML to a third generation language, even if they are only another way to write ASM instructions.

Skips are wonderful as they allow you to:

- structure your program

- avoid using goto’s

- make programs and piece of code that can be easily copied and past (because there is no label)

The basement of Skips is the Block structure.

A block is enclosed in  and , and can be nested within another block.

The following instructions deal with blocks.

| SKIPS instructions | Equivalents |
| --- | --- |
|  ... | Defines a block (generates no code) |
|  ...  |  ...  |
|  ...  |  ...  |

Once blocks are defined, special instructions can be used in them. These instructions called EXIT and UP allow to jump to the end or to the beginning of a block.

| These instructions | are equivalent to |
| --- | --- |
|     |     |

EXIT and UP can jump to the beginning or to the end of an upper-level block by specifying the number of blocks to exit, after the UP or EXIT instructions.

| These instructions | Are equivalent to |
| --- | --- |
|           |           |

Note 1: EXIT1 is equivalent to EXIT, and UP1 is equivalent to UP.

#### Instruction set

Note: for instruction names, the case does not matter.

Register names are:

R0, R1, R2, R3, R4, R5, R6, R7, R8, R9, R10, R11, R12, R13 (or SP), R14 (or LP or LR) and R15 (or PC).

Setting the S flag on an instruction causes the instruction to modify the flags.

Every instruction is evaluated ONLY of the attached condition is true. By default, the instruction is always evaluated.

Separation between arguments can be either ‘,’ or spaces.

| Operation | Assembler | Action | S flags |
| --- | --- | --- | --- |
| Copy and shift |  | d:= <Oprnd> | NZCR |
| Not |  | d:= ~<Oprnd> | NZCR |
| Add |  | d:= Rn + <Oprnd> | NZCVR |
| Add w carry |  | d:= Rn + <Oprnd> + Carry | NZCVR |
| Sub |  | d:= Rn - <Oprnd> | NZCVR |
| Sub w carry |  | d:= Rn - <Oprnd> - Not(Carry) | NZCVR |
| Reverse Sub |  | d:= <Oprnd> - Rn | NZCVR |
| Rev sub w carry |  | d:= <Oprnd> - Rn – Not(Carry) | NZCVR |
| Multiply |  | d:= Rm * Rs | NZR |
| Multiply Add |  | d:= (Rm * Rs) + Rn | NZR |
| Compare |  | flags:= Rn - <Oprnd> | NZCV |
| Cmp Negative |  | flags:= Rn + <Oprnd> | NZCV |
| Test |  | flags:= Rn And <Oprnd> | NZC |
| Tst equivalence |  | flags:= Rn Xor <Oprnd> | NZC |
| And |  | Rd:= Rn And <Oprnd> | NZC |
| Xor |  | Rd:= Rn Xor <Oprnd> | NZC |
|  |  | Rd:= Rn Xor <Oprnd> | NZC |
| Or |  | Rd:= Rn Or <Oprnd> | NZC |
| BitClear (ÑAnd) |  | Rd:= Rn And Not <Oprnd> | NZC |
| Branch |  | R15/PC:= address |  |
| Gosub |  | R14:=R15/PC, R15/PC:= address |  |
| Load Int |   | Rd:= [address] Rd:= data at label. The label address is calculated relative to the PC. This does not work with constants |  |
| Load Byte |   | Rd:= [byte at address] 0 extended Rd:= data at label. The label address is calculated relative to the PC. This does not work with constants |  |
| Multiple load |  | Stack operations (Pop) |  |
| Inc Before |  | ! sets the W bit (updates the base |  |
| Inc After |  | register after the transfer) |  |
| Dec Before |  |  |  |
| Dec After |  |  |  |
| Store Int |   | [address]:= Rd data at label:= Rd. The label address is calculated relative to the PC. This does not work with constants |  |
| Store Byte |   | [address]:= byte value from Rd data at label:= Rd. The label address is calculated relative to the PC. This does not work with constants |  |
| Multiple Store |  | Stack operations (Push) |  |
| Inc Before |  | ! Sets the W bit (updates the base |  |
| Inc After |  | register after the transfer |  |
| De Before |  |  |  |
| De After |  |  |  |
| multiplication |  | rd=r1*r2 |  |
|  |  | rd=r1*r2+r3 |  |
|  |  | Signed mul rd1=low r1*r2, rd2=high r1*r2 |  |
|  |  | Signed mul add rd1+=low r1*r2, rd2+=high r1*r2 |  |
|  |  | rd1=low r1*r2, rd2=high r1*r2 |  |
|  |  | mul add rd1+=low r1*r2, rd2+=high r1*r2 |  |
|  | Creates a label |  |  |
|  | See $ in ASM mode |  |  |
|  | See ASM mode |  |  |

Where cond can be any of:

|  |  |   |   |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| equal | Non equal | Carry set, unsigned >= | Carry clear < | Negative | Positive or 0 | Overflow | No overflow | Unsigned > | Unsigned <= | >= | <= | > | <= |

Oprnd can be of the form:

| Immediate value | Cte Note, cte is encoded on 8 bits + a rotation right encoded on 4 bits. This means that not every value is possible. |
| --- | --- |
| Logical shift left |   |
| Logical shift right |   |
| Arithmetic shift right |   |
| Rotate right |   |
| Register |  |
| Logical shift left |   |
| Logical shift right |   |
| Arithmetic shift right |   |
| Rotate right |   |

In all cases, Cte must be a decimal value or an expression that can be evaluated immediately.

A_mode can be:

|  | Value of rn + or - constant |
| --- | --- |
|  | Value of rn + or – value of rm |
|   | Value of rn + or – value of rm shifted left |
|   | Value of rn + or – value of rm shifted right |
|   | Value of rn + or – value of rm shifted arithmetically right |
|   | Value of rn + or – value of rm rotated right |
|  | Value of rn + or – constante Rn is updated with that value |
|  | Value of rn + or – value of rm Rn is updated with that value |
|   | Value of rn + or – value of rm shifted left Rn is updated with that value |
|   | Value of rn + or – value of rm shifted right Rn is updated with that value |
|   | Value of rn + or – value of rm shifted arithmetically right Rn is updated with that value |
|   | Value of rn + or – value of rm rotated right Rn is updated with that value |
|  | The value used is the value of rn, but rn is then updated with Value of rn + or – constante |
|  | The value used is the value of rn, but rn is then updated with Value of rn + or – value of rm |
|   | The value used is the value of rn, but rn is then updated with Value of rn + or – value of rm shifted left |
|   | The value used is the value of rn, but rn is then updated with Value of rn + or – value of rm shifted right |
|   | The value used is the value of rn, but rn is then updated with Value of rn + or – value of rm shifted arithmetically right |
|   | Value of rn + or – value of rm rotated right Rn is updated with that value |

#### ARMSAT instruction

When using the  instruction, the Saturn pc is in register r0 and the address chipset structure that contains the state of the Saturn CPU is in r1.

That structure has the following elements at the following offsets:

| 0 | P_U32 read_map[256+1]; read_map[x] points on the 2Kb of Saturn address space at Saturn address x<<12 |
| --- | --- |
| 1028 | P_U32 write_map[256+1]; read_map[x] points on the 2Kb of Saturn address space at Saturn address x<<12 for write purpose (write_map[x]=0 if x points on some non readable memory) |
| 2056 | enum ModulePriority top_map[256+1]; // Type of block on top, to know if new configured block takes over |
| 2316 | REG A; |
| 2324 | REG B; |
| 2332 | REG C; |
| 2340 | REG D; |
| 2348 | REG R0; |
| 2356 | REG R1; |
| 2364 | REG R2; |
| 2372 | REG R3; |
| 2380 | REG R4; |
| 2388 | U32 D0 |
| 2392 | U32 D1; |
| 2396 | U32 P, P4, P4_32; // P4 = 4*P, P4_32 = 4*P-32, use setP() to modify P. |
| 2408 | U32 ST; |
| 2412 | U32 HST; |
| 2416 | U32 carry; // 0 or !0 |
| 3420 | BOOL dec; // 0->hex or 1->dec |
|  | U32 RSTK[NB_RSTK]; |
|  | U32 RSTK_i; // Index for next push. |
|  | REG FIELD[32]; // Field masks. |
|  | U32 FIELD_START[32]; // Lowest nibble of the field. |
|  | U32 FIELD_LENGTH[32]; // Length of the field. |

Therefore,  allows you to read the lower 32 bits of the Saturn register A.





allows you to read the first 8 nibbles at Saturn address 01008

The following file can be used to declare your saturn chipset structure.



























































### System RPL mode

Masd can also compile SysRPL programs (you should read the book “An Introduction to System RPL” before trying to write system RPL programs).

The  directive will switch MASD in RPL mode.

Note: if the Flag –92 is Set, Masd starts in  and  mode.

#### Instructions

In RPL mode, Masd interprets instructions/tokens in the following order.

##### Reals and system binary

If the instruction is a decimal number, a system binary is created (MASD will try, if possible to use the internally defined system binary)

It that number has a decimal point (in the middle, or starts with the decimal point), a real number is created.

##### Unnamed local variables

If the instruction is a recall or a set of a local variable defined by  the correct instruction is generated.

A local environment is created using:

	with N<23

These variables have names during compile time, but they are implemented as unnamed local variables, which are faster to access than named local variables.

A local variable is recalled by typing its name or with an optional ending .

Data can be stored in a local variable by typing its name, with a leading or ending  or a leading .

Note 1: Local variable are available until the next local definition.

Note 2: The local environment is not closed automatically, use  or other provided words.

Example:

 will become:

 if there is only one variable)

And:











Program example

|      |      |
| --- | --- |

Note that it is your responsibility to destroy the local environment using ABND and that Masd does not handle multiple level of definition of local variables, nor it is destroying the current environment, even if ABND is used.

Variables defined this way will be valid until a new set of variables are defined.

##### Defines

If the instruction matches a define, the correct code is inserted (see the  instruction)

##### Labels

If the instruction matches the name of a constant or a label, the value of the said constant or label is inserted (if you insert a label, be sure to know what you are doing and to be in absolute mode).

##### extable

If the instruction matches an entry in the extable (see appropriate section at the end of this document) the value associated with this entry is used.



Will produce



Note: Using an external table is much faster than using constants. On the other hand, constants are project dependant, which is not the case of an external table.

##### Tokens

Then, the following instruction are tested:

|  | Program prologue $02D9D |
| --- | --- |
|  or  | List, Program or Algebraic end $0312B |
|  | List prologue $02A74 |
|  | List end $0312B |
|  | Algebraic matrix object |
|  | Algebraic prologue $02AB8 |
|  | Unit prolog $02ADA |
|  | flash pointer from constant |
|  | flash pointer from value |
|  | System Binary of cst value, given in hexadecimal. If there is no spaces between the  and the cst, MASD will try, if possible to use the internally defined system binary |
|  | Address.  generates . |
|  | Extended pointer with given hexadecimal values for the address and switch address |
|  | XLIB object from constant |
|  | XLIB object from value |
|  | Real number |
|  | Long real number |
|  | Complex number |
|  | Long complex number |
|  | Character string. Special characters can be included by typing \ and the ASCII value on two hexadecimal characters. \ can be inserted by typing \\ |
|  | Integer |
|  | Global name (see  for info on character encoding) |
|  | Local name (see  for info on character encoding) |
|  | Tagged object |
|  | XLIB identified by its name. If it is a standard HP48 command (like ), the address is used instead of an XLIB object. |
|  | Binary integer ($02A4E), Size is in hexadecimal and Data is a set of hexadecimal characters. Example:  |
|  |  ($02B1E). |
|  | Library data ($02B88). |
|  | Backup ($02B62). |
|  | Library ($02B40). |
|  | Extended3 ($02BEE). |
|  | Array ($029E8). |
|  | Linked Array ($02A0A). |
|  | Minifont object |
|  | Array object |
|   | Array object, they can have 1 or 2 dimension. All objects in the array must be of same type |
|  | If RplName is a RPL instruction, compiles the RPL instruction (or xlib depending) |
|  | Character object. See rules on  for more information |
|  | Creates a label at this position. Use carefully |
|  | Equivalent to DEFINE name ROMPTR2 xlibname |
|  | Equivalent to DEFINE name FPTR2 fptrname |
|  | Code object ($02DCC). |
|    | Include a code object, change to ASM mode and closed the code object on the next ENDCODE. |
|     | Includes directly hexadecimal data (no prolog). |
|  | Includes the content of the file FileName. |
|  | Includes the source of the file FileName to be compiled (Like ' in ASM mode). |
|  | Defines a label (like * in ASM mode). |
|  | Defines a constant (Like  in ASM mode). |
|  | Defines a constant (Like  in ASM mode). |
|  | Associate the data compiled between the name and the end of the line with the name. After that, if the name is used again, the associated data is placed in the compiled file |
|      | Creates a directory containing the objects in the given variables. |

### Examples of program using the MASD compiler



















































































































				

						

				

























						

					

					



						

	

			



			



					

					

					



			

				



				

				

				

					

				

			

			

				

			

				

					



			





	





			





	

						





				

					



		

				

					



		

						

			

				



	

					

				



					

			

					



	



	

			

			



					

			

		



	

			

						

				

	

					

					

						



## ASM

The disassembler converts Saturn assembly into a source string.

The syntax used is Masd syntax, in mode 0-15.

Each line contains an address and an instruction.

If the system flag -71 is set (with -71 SF), addresses are not shown, except for the destinations of jumps. In this case, the resulting source may be then reassembled if needed.

ASM-> can either disassemble a CODE object or the memory area between 2 given addresses (as binary integer)

Example:

| -71 CF2 (default) | -71 SF2 |
| --- | --- |
| AE734 GOSBVL 0679B AE73B LC 01000 AE742 C-A A AE744 GONC AE742 AE747 GOVLNG 138B9 | GOSBVL 0679B LC 01000 *AE742 C-A A GONC AE742 GOVLNG 138B9 |

| Level 2 | Level 1 | -> | Level 1 |
| --- | --- | --- | --- |
| Binary integer (start address of the memory area to disassemble) | Binary integer end address | -> | String (disassemble between the 2 address) |
|  | Code object | -> | String |

## ARM

The disassembler converts ARM assembly into a source string.

Each line contains an address and an instruction.

If the system flag -71 is set (with -71 SF), addresses are not shown, except for the destinations of jumps. In this case, the resulting source may be then reassembled if needed.

ARM-> can either disassemble a CODE object (which does not make much sense at this point in time) or the memory area between 2 given Saturn addresses (as binary integer)

Example:

| -71 CF2 (default) | -71 SF2 |
| --- | --- |
| 874FF LDMGEIA R0 ! { R5 R6 R7 LP PC } 87507 STMDB R5 { R5 R12 PC } 8750F BL 87527 87517 BLGE 87527 8751F BGE 87527 87527 ADD R0 R0 R0 8752F MOV R0 R1 87537 TST R4 R5 | LDMGEIA R0 ! { R5 R6 R7 LP PC } STMDB R5 { R5 R12 PC } BL 876E3 BLGE 876E3 BGE 876E3 *876E3 ADD R0 R0 R0 MOV R0 R1 TST R4 R5 @ |

| Level 2 | Level 1 | -> | Level 1 |
| --- | --- | --- | --- |
| Binary integer (start address of the memory area to disassemble) | Binary integer end address | -> | String (disassemble between the 2 address) |
|  | Code object | -> | String |

## The Entry point library: extable

The entry points library is an external library (you can get it from the HP web site) that contains a table of entry points names and address. This is used by the MASD compiler to get the value of system RPL entry points or assembler constants (like TURNMENUOFF for example).

This library should be stored in port 0, 1 or 2.

If you want to program in system-RPL, you must install this library.

This library also contains 4 functions:

### nop

This function is here for internal purpose and should not be used.

Running this function does nothing

### GETNAME

Lookup for the name of an entry from its address.

Example: #054AFh GETNAME -> “INNERCOMP”

Note: as multiple entries might have the same address, GETNAME is not a bijective (one-to-one) function

Level 1		->	Level 1

Binary integer	->	String

### GETADR

Lookup for the address of an entry.

Example: “INNERCOMP” GETADR -> #054Afh

Level 1		->	Level 1

String		->	Binary integer

### GETNAMES

Find all the entry which name starts by a specific string.

Note: giving a null string as an input will return a list of all the entry points.

Level 1		->	Level 1

String		->	List of entry names.