GPSurvey

Programs to Aid in Preliminary Survey with GPS Devices

GpSurvey is a series of programs created for the HP 50G calculator that can be used with other HP 49 series calculators, and the HP 48GX.

I wrote the program as an aid to help me perform preliminary site surveys for existing and proposed pipe line alignments. This program is most useful when used to determine positions, lengths and in some cases elevations of civil infrastructure within a large area or along a long alignment, (e.g. pipelines, roads, trails, basins, large industrial plants, etc.).

The accuracy of the data output depends heavily on the accuracy of the GPS device used. In my case I used a Magellan 310 to develop the program and for initial testing, and later I used the program with a professional GPS device available at work. The difference in accuracy between the two devices was not significant for a preliminary site evaluation. However, environmental factors must be considered when performing a survey with a GPS device, proximity to buildings and large water surfaces can reflect and distort the signal, and trees can affect your accuracy as well. I recommend, if you haven’t done so already, that you research the limitation of GPS devices for your better understanding of their potential uses and misuses.

The connection between the HP 50G (49G or 48G) series calculator is done through a custom serial cable available through HPCalc.org, the following is a link to their commerce site: http://commerce.hpcalc.org/

The use of a null-modem may be necessary to allow the two devices to connect; this is also available at the above website.

Instructions for Program Use

- Once you download GPSurvey on your calculator and open the folder in your device you will have the following soft-key menu.

  - “Marks” is the folder that contains the dataset stored during the survey.

  - “Data” is a rudimentary help file that will give you basic information on the programs, for example it will tell you that the GPS signal should be NMEA standard and GPGGA format.

  - “Reset” resets the dataset to zero (erases all data from the “Marks”folder) so you can begin a new survey. You will need to press “Reset” before starting any new survey unless you are trying to resume a previous survey from where you left off.

  - “GPS>” Is the main data acquisition software, press this soft-key over a geographical point or near a landmark you are trying to use as a survey point. The calculator will display a message on the screen that will indicate if it is acquiring satellite data. Note: the process of acquiring satellite data may take a few minutes for each survey point, depending on the strength of the satellite signal. It is strongly recommended that you check each satellite reading as it is displayed on the calculators stack, you may find that the data was corrupted due to a week or distorted signal. To delete the last reading made, press the “Redo<-“soft-key and press the “GPS>” soft-key again to survey your current position (survey point) again. The “Redo<-“soft-key is explained below.

  - “hp50>” this program will calculate distances (via the Haversine method), slopes based on the data collected in the “Marks” folder (global Lat-Long coordinates and elevations). This program must be run after you finished surveying or in the middle of the survey if you wish to see your progress.

  - “Redo<-“is a command that will allow you to delete previous survey points. It will erase the last survey point read when pressed once and if pressed more than once, it will continue deleting the previous consecutive survey points form the “Marks” folder.

- If you press NXT on the keyboard you should see the following screen:

  - “Test\/” is a folder that contains the “Test>” program and a series of commands that can allow you to test the communication between the calculator and the GPS by visually seeing what the GPS is sending to the calculator. I use this program and the commands when I believe that the data sent to me may be corrupted for some reason, (e.g. if you are near a military base for example, the signal may be distorted).

  - “UTM>” will transform the Longitude Latitude coordinates received by the GPS to Universal Transverse Mercator (UTM) coordinate. Please note that the algorithm used in this calculation is not 100 percent accurate and it is very dependant of the geodetic parameters you input. The method used in this algorithm is that prescribed by the Jet Propulsion Lab USAICS 1987, the ALGO_PUB_0132 Report. Per this report any northing at Latitude south of the equator must be added by 1E7 (N=1E7+N), and any easting at Longitude east or west of the meridian must be corrected by 5E5+E or 5E5-E respectively. The program does not perform this calculation, it will only provide you with the raw coordinate data.

  - “Coor>” provides you with a plan view of the coordinates and trajectory you followed in the survey. You can use this graphic to check the data input based on the shape of your trajectory in the survey. This program will give you the option to plot Lat-Lon or UTM coordinates, if you wish to plot UTM coordinates, you first must transform the coordinates to UTM by using the “UTM>” program.

  - “Prof>” allows you to graph a profile of the survey points. Before using this program you must first have ran the “hp50>” program.

- If you press NXT again you will see the following screen. All these files are simple backups of the executable files in case one of the executable files is deleted accidentally.

- Your output data should look similar to this:

The results are displayed as lists in the order of points surveyed.

Plan view of the horizontal alignment for the survey.

Profile of the vertical alignment for the survey; note that the “flags” indicate points where data was taken.

The source code is open in case you would like to employ the algorithms in your own program or customize the input or output.

I hope you find the program useful.

Best regards,

Bryan R. Osborne, PE

Black50g@gmail.com