HP49G  REINFORCED CONCRETE DESIGN  Ver. 1.0

By : ALEX C. NACIONALES

10th St., Alta Tierra Village

Iloilo City, Philippines

Email: anacionales@insightts.net

acnacionales@yahoo.com

To load the Program , transfer the file PRGM to HP49G

Special Instructions:

CAS MODE : Numeric must be unchecked and Approx must be checked

This program is free but the author will appreciate donations sent to the above address if you find the  program useful.

Program  Manual

PRGM - Program Directory

Program Contents

- RC Beam Design

- RC Doubly Reinforced Beam Analysis

- Rectangular Column Analysis

- Spiral Column Analysis

- Footing Design

- Slab Design

1)   RC Beam Design

Designs Singly Reinforced RC Beams using Ultimate Strength Design

To start the program from directory that contains PRGM,

press the following   softkeys :

PRGM DESN  USD  BEAM  RUN    or   PRGM BEAMS

Step1  - Enter the following data

Mu = Ultimate Moment Required

B   =  Assumed Beam Width

P   = Steel ratio

Note: Value in D  is result of previous calculation

Use arrow keys to move to different fields

– Press OK  or  ENTER

Step 2 – Enter the following data

Fc’ = Ultimate Strength of Concrete in  MegaPascals (MPa)

Fy  =  Yield Strength of Steel in  MegaPascals (MPa)

Es  =  Modulus of Elasticity of Steel in  MegaPascals (MPa)

Ec  =  Modulus of Elasticity of Concrete in  MegaPascals (MPa)

- Press OK  or  ENTER

Step 3  - Computed D(effective depth) is displayed

Enter a value of D to use or use the computed value

Enter a bar size in millimeter

Note: Do not change the values of B and P in this Step

- Press OK or Enter

Result displays the following:

Qty and Size of Rebar

Area of Steel provided

Actual steel ratio

2) RC Doubly Reinforced  Beam Analysis

Computes Ultimate Moment Strength of Doubly Reinforced Beam

To start the program from directory that contains PRGM,

press the following   softkeys :

PRGM DESN  USD  BEAM  DRBM  RUN   or  PRGM  DBEAM

Step1  - Enter the following data

B        =   Beam Width in meters

H        =   Beam Depth in meters

QTY   =   Quantity of  tension side reinforcing steel bars(RSB)

RSB    =    Size of tension side reinforcing steel bars (RSB) in millimeters

D’       =   distance from centroid of  RSB to face of beam

QTY ‘  =   Quantity of  compression side reinforcing steel bars(RSB)

RSB’   =    Size of compression side reinforcing steel bars (RSB) in

millimeters

– Press OK  or  ENTER

Step 2 – Enter the following data

Fc’ = Ultimate Strength of Concrete in  MegaPascals (MPa)

Fy  =  Yield Strength of Steel in  MegaPascals (MPa)

Es  =  Modulus of Elasticity of Steel in  MegaPascals (MPa)

Ec  =  Modulus of Elasticity of Concrete in  MegaPascals (MPa)

- Press OK  or  ENTER

Result displays Mu,  Ultimate Moment Strength of Doubly Reinforced Beam

3.) Reinforced Concrete Rectangular Column Analysis

Checks adequacy of Short Rectangular Columns, in single axis and biaxial bending

To start the program from directory that contains PRGM,

press the following   softkeys

PRGM DESN  USD  CLMN  RCTN   RUN   or  PRGM  RCOLM

Step1  - Enter the following data

Pu       =   Ultimate Axial Load

Mux    =    Ultimate Moment in x-axis

Muy  =    Ultimate Moment in y-axis (Enter 0 for Single Axis Analyis)

- Press OK  or  ENTER

Step2  - Enter the following data

b           =   Column Width in meters

h           =   Column  Depth in meters

QTYx   =   Quantity of x-axis reinforcing steel bars(RSB)

RSBx    =    Size of  x-axis reinforcing steel bars (RSB) in millimeters

d’          =   distance from center of  RSB to face of  column

QTY y  =   Quantity of  y-axis reinforcing steel bars(RSB)

(Enter 0 for Single Axis Analysis)

RSBy   =  Size of  y-axis  reinforcing steel bars (RSB) in millimeters

(Enter 0 for Single Axis Analysis)

Step 3 – Enter the following data

Fc’ = Ultimate Strength of Concrete in  MegaPascals (MPa)

Fy  =  Yield Strength of Steel in  MegaPascals (MPa)

Es  =  Modulus of Elasticity of Steel in  MegaPascals (MPa)

Ec  =  Modulus of Elasticity of Concrete in  MegaPascals (MPa)

- Press OK  or  ENTER

Result checks adequacy of columns

If Computed Pu/Pu  is equal or more than 1.0 then column is OK

4) Reinforced Concrete Spiral Column Analysis

Checks adequacy of  Short Spiral Columns, in single axis and biaxial bending

To start the program from directory that contains PRGM,

press the following   softkeys

PRGM DESN  USD  CLMN  SPRL  RUN   or  PRGM SCOLM

Step1  - Enter the following data

Pu       =   Ultimate Axial Load

Mux    =    Ultimate Moment in x-axis

Muy   =    Ultimate Moment in y-axis (Enter 0 for Single Axis Analyis)

- Press OK  or  ENTER

Step2  - Enter the following data

D           =   Column Width in meters

D’          =   distance from center of  RSB to face of  column

QTY      =   Quantity  reinforcing steel bars(RSB)

RSB    =    Size of reinforcing steel bars (RSB) in millimeters

Step 3 – Enter the following data

Fc’ = Ultimate Strength of Concrete in  MegaPascals (MPa)

Fy  =  Yield Strength of Steel in  MegaPascals (MPa)

Es  =  Modulus of Elasticity of Steel in  MegaPascals (MPa)

Ec  =  Modulus of Elasticity of Concrete in  MegaPascals (MPa)

- Press OK  or  ENTER

Result checks adequacy of columns

If Computed Pu/Pu  is equal or more than 1.0 then column is OK

5) Reinforced Concrete Spread Footing Design

Designs RC spread footings with axial and bending loads

To start the program from directory that contains PRGM,

press the following   softkeys

PRGM DESN  USD  FTNG   RUN    or  PRGM  FOOT

Step1  - Enter the following data

P         =  Unfactored Axial load  in KN

Pu       =   Ultimate Axial load in KN

M       =    Unfactored moment load  KN-m

Mu     =    Ultimate moment load in KN-m

qa      =    Allowable soil bearing pressure in KPa

T       =    Assumed footing thickness in meters

y        =   Assumed soil weight in KPa

- Press OK  or  ENTER

Step 3 – Enter the following data

Fc’ = Ultimate Strength of Concrete in  MegaPascals (MPa)

Fy  =  Yield Strength of Steel in  MegaPascals (MPa)

Es  =  Modulus of Elasticity of Steel in  MegaPascals (MPa)

Ec  =  Modulus of Elasticity of Concrete in  MegaPascals (MPa)

- Press OK  or  ENTER

Step 4  -  Program displays computed minimum footing width

Enter footing width to use

Enter column width

Step 5  - The program checks footing depth for punching shear and base shear

The program then displays min. steel ratio, Press ENTER

The program checks effective footing depth for moment

Effective footing depth = T – 0.10m

Step 6   -  Enter Size of RSB to use

Press OK  or  ENTER

Step 7   -   Program displays required number of RSB

And Area of steel provided

- Reinforced Concrete Slab Design

Designs Two-Way beam supported slabs using

Working Stress Design- ACI code 1963  Method 2

To start the program from directory that contains PRGM,

press the following   softkeys

PRGM DESN WSD SLAB RUN   or  PRGM SLABS