SECTN STRESS V. 1.00

Library No. 1235

Following the determination of the characteristic properties by means of the excellent  program

“Sectn48" (Library No. 1600) this  program calculates for a cross section submitted to a longitudinal force N and one or two bending moments Mx and My the stresses in every point of the input matrix.

The sign definition is shown in picture 1)      				       +My

A longitudinal force N	is positive if acting as tractive force.			       +y

+x     +Mx

Picture 1)

Installation:

Transfer both libraries to the calculator and place them on the stack in succession.

Place the port number you wish to store the libraries in on the stack and press “STO”

Warmstart the calculator by pressing “ON” and “C” simultaneously or execute

“:port:library-No.”  “ATTACH”

Purge the variables containing the libraries.

Running the program:

First of all you have to pay attention to the corresponding units:

if you enter the coordinates in [cm], you have to input the moments in [kNcm] !

The results then will be in [kN/cm²]

Press “SECTN STRESS V1.00” in the library. Then the  displays looks like this:

Put the matrix of  coordinates  [ [ x1  y1 ] [x2  y2] [ ….] [xn  yn] ]  ( with n = 1; end point = first

point, see [1] ) on the stack and press “l1600”. Now you see the plot of the cross section.

Press “ON” or “ENTER” and see this display:

Now press “CONT” and the display will look like this:

Enter the loads: longitudinal force ( negative = pressure force )   and press  “N”.

It is assumed that N acts in the center of gravity, otherwise you have to enter the target point by the    softkey(s)  x-N and/or y-N ( referred to “0”-point of coordinates ).

If existing, enter moment Mx (acting around x-axis, positive as shown ) and press “Mx”

If My exists, enter value and press “My”, otherwise press only this softkey without any input, as this step is necessary to continue the program and see the results in level 1 – level 3 in the stack:

LEVEL 3: shows the angle between the x-axis and the neutral axis, i.e. the line where the stresses

Sigma,i = 0. The angle is positive if the line is rotated counterclockwise to the x-axis.

LEVEL 2: shows a { n  5 }-sized array containing:

column 1 : the x-coordinates of the cross section

column 2 : the y-coordinates of the cross section

column 3 : the stresses in the corresponding point

column 4 and 5: if the neutral axis is situated between the actual and the preceding point

i.e. if there is a change of sign in stresses , both columns show the coordinates (x and y ),

where the neutral axis intersects the contour.

LEVEL 1: shows [ [maximum stress ] [minimum stress] ], i.e.  greatest positive (tensile) and negative (compressive) stress of all values in level 2, column 3.

Before analysing the values in level 2 you have here the opportunity to check immediately whether maximum stress meets any limitations.

Now you may retrieve a plot of the contour with the actual location of the neutral axis by pressing

the last softkey “SigmaPLOT”.

Note that this plot only shows an approximate position of this axis. The less the ratio |M/N| the more inaccurate the location of the neutral axis! Nevertheless the numerical results in level 2 are always accurate !

Press  “ENTER” to clear the plot. You may no enter other load combinations by entering values via

the corresponding softkeys.  Every new input is handled separately ,i. e. the preceding loads are set to “0”.

Press “NEXT” to see this display:

The key “xy>AI” shows again all characteristic properties of the actual cross section.

“EXIT” leaves the program and deletes all variables and the subdirectory “I,A>Sigma”.

Example:

The stresses for example 1 ( in chapter 7 of [1] ) have to be calculated for :

N = -104 kN ,  Mx = -28500 kNcm , My = 7600 kNcm

Enter the array of  example 1  multiplied by 0,1 ( as the moments are in kNcm !):

[[0   0] [28   0 ] [28   4] [16   4] [16   26] [28   26] [28   30] [0   30] [0   26] [12   26] [12   4] [0   4] [0    0] ] and  press  “l1600”.

You will see the plot of an I-section,  press “ENTER” to see the characteristic properties (refer to [1] )

then “CONT”  and enter 104  +/-   “N”  , 28500  +/-   “Mx”  and 7600  “My” .

Then the display looks like this:

Sigma, max = 17,13 kN/cm² , Sigma, min = -17,80 kN/cm². Press “      “ (right key), then “    “ (down key) to scroll through the result in level 2. You will find four points where the neutral axis intersects the contour (28,00 ; 3,96) , (27,94 ; 4,00) , (16,00 ; 13,00) and ( 12,00 ; 16,02 ).

Press  “ON” and “     “ again, then “     “ ( up key ). Line 3 shows “Sigma,angle : -37,01” which is the

angle between neutral and x-axis in degrees.

Press “ON” and  “SigmaPLOT” . The result is:

Press ”ENTER” , “NEXT” and “EXIT” to end the program.

COPYRIGHT AND DISCLAIMER OF WARRENTY:

All parts of this  programm except Sectn48 (library-No. 1600, copyrighted  by  C.  Lugtmeier, see [1]  ) are copyrighted by Claus Dachselt . It is provided as a free release for non-commercial purpose “as is”  without warranty or responsibility of any kind. In no event will the copyright holder be liable for damages arising out of the use or inability to use this program.

Claus Dachselt

e-mail:  clan.dachselt@t-online.de

References:

[1] : Manual of program “Sectn48” ( Library-No. 1600 ) by Caspar Lugtmeier