Manual

##### SED48

Version 1.0

C. Lugtmeier © 2000

Contents

Copyright and disclaimer of warranty	3

Description	3

SED48 library menu	4

Commands	4

SEDM	4

ALLP	4

SELP	4

SST	4

SST	5

MTYP	5

STYP	5

PROP	5

VALS	5

DBS	5

DBS	5

CFGRsed	5

ABOUTsed	5

Command reference	6

ABOUTsed	6

ALLP	6

CFGRsed	6

DBS	6

DBS	7

PROP	7

MTYP	7

SEDM	7

SELP	8

SST	8

SST	9

STYP	9

VALS	9

Remarks	9

Database structure	10

General	10

Compression	10

Examples	12

Database	12

Steel sections	12

Standard beams	12

Sheet piles	12

Programming	13

User functions	13

Browsing	13

Installation	15

Characteristics	15

Author	15

# Copyright and disclaimer of warranty

All the files of the SED48 library are copyrighted © by C. Lugtmeier. SED48 is distributed in the hope that it will be useful, but the copyright holder provides the program “as is” without warranty of any kind, either expressed or implied, including, but not limited to, the implied warranties of merchantability and fitness for a particular purpose. In no event will the copyright holder be liable to you for damages, including any general, special, incidental or consequential damages arising out of the use or inability to use the program.

This version of SED48 is a Giftware release. You may use it as long as you like, but only for non-commercial purposes and only as a private person. Permission to copy the whole, unmodified, SED48 library is granted provided that the copies are not made or distributed for resale (excepting nominal copying fees) and provided that you include on each copy this copyright notice and disclaimer of warranty.

# Description

The SED48 library is an application for the Hewlett Packard 48 series calculators. It is designed to be used as a database for Structural and Mechanical Engineering. The database is optimized for defining steel sections, standard beams, etc. However, please note that it is not limited to these uses. The database structure is so that it can be used for a lot of different types of data.

SED48 is completely written in SysRPL.

# SED48 library menu

## Commands

The SED48 library menu consists of the following commands:

- SEDM

- ALLP

- SELP

- SST

- SST

- MTYP

- STYP

- PROP

- VALS

- DBS

- DBS

- CFGRsed

- ABOUTsed

The stack diagrams of these commands are listed in the section command reference. Below the use of these commands is explained.

### SEDM

This command displays an easy to use interface to the most commonly used commands: ALLP, STYP, SST and SST. See the description of these commands for further details on their use.

The menu assembles a menukey for each Main_type available in the database. These menukeys provide four actions, two in the unshifted plane and two in the shifted plane:

- unshifted		ALLP	(with %Sub_type as argument on level 1)

- unshifted		STYP	(with ‘?’ as argument on level 1)

- left-shifted		SST

- right-shifted	SST

Please note that the use of these commands through this menu only does not require the Main_type as an argument! Since there is a menukey for each Main_type this is not necessary. The Main_type argument is provided internally by the menu definition.

### ALLP

This command is used to extract all Properties of the selected Main_type and Sub_type that are defined in the current database. Depending on the status of userflag 45 and 46 the output is given tagged or untagged and in a list or not.

### SELP

This command is used to extract selected Properties of the selected Main_type and Sub_type that are defined in the current database. Depending on the status of userflag 45 and 46 the output is given tagged or untagged and in a list or not.

### SST

This command is used to extract a Sub_Type of a selected Main_type that meets the search criterium. For instance, you could search a Sub_type of a Main_type that has an area of at least (equal to or bigger than) 2500.

### SST

This command is used to extract a Sub_Type of a selected Main_type that meets the search criterium. For instance, you could search a Sub_type of a Main_type that has a height of at most (equal to or smaller than) 500.

### MTYP

This command is used to extract all Main_types that are defined in the current database. Depending on the status of 46 the output is given in a list or not.

### STYP

This command is used to extract all Sub_types of the selected Main_type that are defined in the current database. Depending on the status of userflag 45 and 46 the output is given tagged or untagged and in a list or not.

### PROP

This command is used to extract all Properties of the selected Main_type that are defined in the current database. Depending on the status of userflag 45 and 46 the output is given tagged or untagged and in a list or not.

### VALS

This command is used to extract all Values of the selected Main_type that are defined in the current database. Depending on the status of userflag 45 the output is given tagged or untagged.

### DBS

This command is used to store a database or to delete the current database (both with confirmation).

### DBS

This command is used to recall the current database.

### CFGRsed

This command provides an easy to use interface to manipulate userflag 45 and 46. Depending on the state of these flags given output is tagged or untagged and in a list or not.

| Userflag | 45 | 46 |
| --- | --- | --- |
| Clear | tagged results | results in list |
| Set | untagged results | results on stack |

### ABOUTsed

This command displays a simple “about” message.

# Command reference

## ABOUTsed

This command displays a small “about” message.

| IN |  | OUT |  |
| --- | --- | --- | --- |
| level |  | level |  |
| 1 | - | 1 | - |

## ALLP

This command yields all property values of the selected Main_type & Sub_type that are registered in the database.

| IN |  | OUT |  |
| --- | --- | --- | --- |
| level |  | level |  |
| 2 1 | “Main_type” %Sub_type | 1 1 n n-1 2 1 n n-1 2 1 | with flag 45 clear & 46 clear: Main_type_Sub_type_TAG: {Property TAG: %Values} with flag 45 set & 46 clear: {%Values} with flag 45 clear & 46 set: Property_TAG: %Value_1 Property_TAG: %Value _2 Property_TAG: %Value_n-1 Property_TAG: %Value _n with flag 45 set & 46 set: %Value_1 %Value_2 %Value_n-1 %Value_n |

## CFGRsed

This command displays the database configuration menu.

| IN |  | OUT |  |
| --- | --- | --- | --- |
| level |  | level |  |
| 1 | - | 1 | - |

## DBS

This command is used to store a new database or to delete the current database. You will be asked to confirm the action.

| IN |  | OUT |  |
| --- | --- | --- | --- |
| level |  | level |  |
| 1 1 | {Database} or: %0 | 1 | - |

## DBS

This command is used to recall the database to the stack.

| IN |  | OUT |  |
| --- | --- | --- | --- |
| level |  | level |  |
| 1 | - | 1 | {Database} |

## PROP

This command yields the defined Properties of the selected Main_type.

| IN |  | OUT |  |
| --- | --- | --- | --- |
| level |  | level |  |
| 1 | “Main_type” | 1 1 n n-1 2 1 | with flag 45 clear & 46 clear: Main_type_TAG: {Properties} with flag 45 set & 46 clear: {Properties} with flag 45 set & 46 set: Property_1 Property _2 Property _n-1 Property _n |

## MTYP

This commands yields all Main_types of the current database.

| IN |  | OUT |  |
| --- | --- | --- | --- |
| level |  | level |  |
| 1 | - | 1 n n-1 2 1 | with flag 46 clear: {“Main_types”} with flag 46 set: “Main_type_1” “Main_type_2” “Main_type_n-1” “Main_type_n” |

## SEDM

This command displays the SED48 menu interface to the ALLP, SST and SST commands.

| IN |  | OUT |  |
| --- | --- | --- | --- |
| level |  | level |  |
| 1 | - | 1 | - |

## SELP

This command yields selected property values of the selected Main_type & Sub_type that are registered in the database.

| IN |  | OUT |  |
| --- | --- | --- | --- |
| level |  | level |  |
| 3 2 1 3 2 1 | “Main_type” %Sub_type {Selected_properties} or: “Main_type” %Sub_type Selected_property | 1 1 n n-1 2 1 n n-1 2 1 | with flag 45 clear & 46 clear: Main_type_Sub_type_TAG: {Selected_property_TAG: %Values} with flag 45 set & 46 clear: {%Values} with flag 45 clear & 46 set: Selected_Property_TAG: %Value_1 Selected_Property_TAG: %Value_2 Selected_Property_TAG: %Value_n-1 Selected_Property_TAG: %Value_n with flag 45 set & 46 set: %Value_1 %Value_2 %Value_n-1 %Value_n |

## SST

This commands yields a Sub_type of the selected Main_type that is equal to or bigger than the Property_value that is desired.

| IN |  | OUT |  |
| --- | --- | --- | --- |
| level |  | level |  |
| 3 2 1 | “Main_type” Selected_property %Minimum_value | 3 2 1 3 2 1 | with flag 45 clear: “Main_type” %Sub_type Property_TAG: %Value with flag 45 set: “Main_type” %Sub_type %Value |

Note: The SST command searches from the top row to the bottom row of the database and returns the first value that matches the search criterium.

## SST

This commands yields a Sub_type of the selected Main_type that is equal to or smaller than the Property_value that is desired.

| IN |  | OUT |  |
| --- | --- | --- | --- |
| level |  | level |  |
| 3 2 1 | “Main_type” Selected_property %Maximum_value | 3 2 1 3 2 1 | with flag 45 clear: “Main_type” %Sub_type Property_TAG: %Value with flag 45 set: “Main_type” %Sub_type %Value |

Note: The SST command searches from the bottom row to the top row of the datbase and returns the first value that matches the search criterium.

## STYP

This command yields all Sub_types of the selected Main_type.

| IN |  | OUT |  |
| --- | --- | --- | --- |
| level |  | level |  |
| 1 | “Main_type” | 1 1 n n-1 2 1 | with flag 45 clear & 46 clear: Main_type_TAG: {%Sub_types} with flag 45 set & 46 clear: {%Sub_types} with flag 45 set & 46 set: %Sub_type_1 %Sub_type_2 %Sub_type_n-1 %Sub_type_n |

## VALS

This command yields the defined %Values of the selected Main_type.

| IN |  | OUT |  |
| --- | --- | --- | --- |
| level |  | level |  |
| 1 | “Main_type” | 1 1 | with flag 45 clear: Main_type_TAG: [%Values] with flag 45 set: [%Values] |

## Remarks

48G series owners can also use the parallel list processing capabilities of the G series to extend the usability of the commands above.

# Database structure

## General

The database consists of a list of lists. Each of these lists contains a list as first element and a matrix as second element (see also examples).

- Database

{

Main_type_list_1

Main_type_list_2

Main_type_list_n-1

Main_type_list_n

}

- Main_type_list

{

Property_list

Values_matrix

}

- Property_list

{

Main_type_string

Property_1

Property_2

Property_n-1

Property_n

}

- Values_matrix

[[ Subtype_1   Value_1 Value_2 Value_n-1 Value_n ]

[ Subtype_2   Value_1 Value_2 Value_n-1 Value_n ]

[ Subtype_n-1 Value_1 Value_2 Value_n-1 Value_n ]

[ Subtype_n   Value_1 Value_2 Value_n-1 Value_n ]]

## Compression

In order to save some memory it is a good idea to use a compressed database. SED48 supports the use of any compression technique as long as the following applies:

- The %Values matrix of each Main_type should be compressed in such a way that it is replaced by a program that, when this is EVALuated, yields the uncompressed %Values matrix.

- The thus compressed database can then be stored with the DBS command.

- The decompression of the %Values matrix should not error!

- Preferably the decompression of the %Values matrix should not use commands that provide argument checking. Note that this rules out the use of UserRPL! If you do use commands that provide argument checking errors that occur while searching the database will not be attributed to the correct commands! If you are proficient in SysRPL you should use the EvalNoCK command to avoid this.

An example of a SysRPL program that accomplishes such a task while using the BZ compressor looks like this:

::

CK1NoBlame

::					(check presence of BZ xlib)

“BZ”

palparse

NOTcsdrpfls

DUPTYPEROMP?

NOTcsdrpfls

DUP1LAMBIND

ROMPTR@

NOTcsdrpfls

DROPTRUE

;

NOTcase

::

“Error: BZ Not Found”	(BZ not available as xlib)

DO$EXIT

;

1GETLAM				(get xlib)

EVAL					(perform compression)

‘ ‘					(place quotation mark on stack)

1GETABND				(get xlib)

‘ EvalNoCK			(place EvalNoCK pointer on stack)

FOUR ::N				(create composite)

;

This program is include in the DBS.DIR directory in the package in which SED48 came. In the same directory are two UserRPL programs, called COMPR and DECOMPR. As can be guessed from their names these commands can be used to perform compression or decompression on the complete database. These programs call the BZP program which is listed above. This all in the assumption of the BZ command being available on your calculator

# Examples

## Database

### Steel sections

{

{

{ “HEA” “b” “h” “A” “Iy” }

[[ 100  96 100 2120  3490000 ]

[ 200 190 200 5380 36920000 ]]

}

{

{ “HEB” “b” “h” “A” “Iy” }

[[ 100 100 100 2600  4500000 ]

[ 200 200 200 7810 56960000 ]]

}

}

Also allowed is:

{

{

{ “HEA” b h A Iy }

[[ 100  96 100 2120  3490000 ]

[ 200 190 200 5380 36920000 ]]

}

{

{ “HEB” h A “Iz” }

[[ 100 100 2600  1670000 ]

[ 200 200 7810 20030000 ]]

}

}

This means that you can have different properties (if desired) for any Main type! You can also use string and ID property definitions in the same list.

### Standard beams

Used just like steel sections!

### Sheet piles

Used just like steel sections!

## Programming

### User functions

Below an example is given of how to declare a user-function in order to use the SED library command SELP in equations.

This is done for the property “A” (here supposedly the cross-sectional area of a steel section).

‘Area’

<<  maintype subtype		@ creation of local variables

<< 45 SF 46 SF				@ no tags, no lists

maintype

subtype

“A”					@ property “A”

IFERR

SELP

THEN

IF

-55 FC?				@ last arguments system flag

THEN

3 DROPN				@ drop last arguments

END

0					@ return 0 if SELP errors

END

>>

>>

Now you can use the function Area in your equations like this:

‘Weight=Area(MT,ST)*0.000785’

You could load this equation into the solver and define MT and ST to your liking. Other properties could be declared exactly the same.

### Browsing

If you own a 48G series calculator you could use the choose-box engine to browse through your database. Below an example is given for a User-RPL program BR.PRG which accomplishes such a task. This program is also included in the BR.DIR directory in the package in which SED48 came.

‘BR.PRG’

<< 0 0 0 0 1 1 RCLF



mt st l1 l2 c1 c2 f		@ creation of local variables

<< 45 CF 46 CF				@ tags and lists

DO

“Main Type” MTYP

DUP ‘l1’ STO

c1 CHOOSE				@ main type choose box

IF

THEN

l1 OVER POS

‘c1’ STO			@ store choose position

‘mt’ STO

DO

mt STYP

OBJ SWAP

DUP ‘l2’ STO

c2 CHOOSE			@ sub type choose box

IF

THEN

l2 OVER POS

‘c2’ STO			@ store choose position

‘st’ STO

mt st ALLP

OBJ SWAP

1 CHOOSE			@ properties choose box

IF

THEN

DROP

mt st ALLP		@ properties to stack

1 1			@ quit browser interface

ELSE

0				@ back to sub type choose box

END

ELSE

0 1				@ back to main type choose box

END

UNTIL

END

ELSE

1

END

1 ‘c2’ STO			@ reset sub type choose postion

UNTIL

END f STOF

>>

>>

Note:

Users who have the Metakernel installed should replace the CHOOSE command with the CHOOSE2 command.

# Installation

The SED48 library can be installed in any available port on the calculator. However, it works fastest from a non-covered port (port 0 or 1). To install the SED48 library on your calculator do the following:

- Transfer the library to your calculator and place the library on the stack

- Place the port number you wish to store the library in on the stack

- Press the STO button

- Warmstart the calculator (press ON and C simultaneously) or shut it off and then turn it on again

- Purge the variable which still contains the library

If any of this gives you problems you should read the HP manual!

# Characteristics

| SED48 | Description |
| --- | --- |
| Version | 1.0 |
| Library number | 1500 (decimal) |
| Size (bytes) | 3527 (G) 3557 (S) |
| Checksum | #29806d (G) #61591d (S) |
| Systemflags used | none |
| Userflags used | 45,46 |
| Language used | Sys-RPL |

# Author

C. Lugtmeier

The Netherlands

c.lugtmeier@hccnet.nl

http://home.hccnet.nl/c.lugtmeier

As a rom pointer! A BZ program in a directory or HOME is not allowed.

The Main type HAS to be a string though.

Actually, the properties can be in any format, like arrays, lists etc. Most of the time this is not meaningful though.

Note: when entered directly from the commandline you cannot define the Main type as a string since the user function will not accept this. If this should be possible you can add the sequence:  STR DUP SIZE 1 – 2 SWAP SUB  after  maintype  and then enter the Main type as an ID (global variable, with quotation marks).