SPRING CONSTANTS EQUIVALENT TO RIGIDITY OF BEAM  v1.2

Author : Claus Dachselt   ( clan.dachselt@t-online.de )

Library No. :   1232

The following program determines for a one-field beam  the equivalent spring-constant values

( vertical spring c-v [MN/m] or rotational spring c-m [MNm/rad] ) at any point “a” of the length given the module of elasticity E [MN/m²] and the moment of inertia  I [cm^4].

The following supports are allowed:

- hinges/clamps at both ends

- clamped at one end and hinge at the opposite end

- clamped at the left end only ( cantilever beam)

Installation:

Transfer the library (#1232) to the calculator und put it on the stack

Enter a port number (“2”  recommended) and press STO.

Then warmstart your calculator or enter : “:2:1232” “ ATTACH”

Example:                                     Which are the equivalent spring constants at this point for

the given beam?

a = 3,1

L =  7,5

HEB 280 ; E = 210000 MN/m²

A.   Calculating the vertical spring constant:

A1.)  Press   “ | C    | ”

A2.) Input the following data into the corresponding fields:

E     = 210 000 ; I = 19270 [cm^4] ( = I of HEB 280 ) ; a = 3,1 [m]; l = 7,5 [m];

left   = 1 ( support condition left end: 0 / 1 = hinged / clamped ) ;

right = 0 ( right end support : 0 / 1 /-1 = hinged/clamped/not supported (cantilever beam)

A3.) Press softkey “OK” . Result : C-v [MN/m]  : 13,20454

B.  Calculating the rotational spring constant:

B1.)  Press   “ | C | ”

B2.)  As program saves last input ( A2. ) you may skip to B3.)

B3.) Press softkey “OK” . Result : C-m [MNm]  : 59,45156

V1.2:  error in computing rotational spring ( both ends being clamped ) corrected.

WARNING and DISCLAIMER: This program is provided as freeware without any kind of warranty, SO USE AT YOUR OWN RISK