- THREE-EDGES-SUPPORTED TWO-WAY SLAB     V. 3.1

- Author: Claus Dachselt						   Library-No. 1238

The following program determines for a two-way slab supported on three edges ( top edge without support ) the bending moments  if  the  lengths Lx and Ly [ m ] and one or more of the following loads

( sketch 1)  ) are given:

- area load q [ kN/m² ]  ,

- triangle-shaped area load q,Tri [kN/m²] ( q = q,Tri on bottom, q = 0 on top edge) ,

- load q,R  [ kN/m ] along the free edge  .

The supports on left, right, and bottom edge may be fixed (clamped) and/or hinged in any order.  The ratio  = Ly/Lx must lie within 0.25 and 1.5!

The calculation is based on sixth-degree polynomials  interpolating the tabled values in [1] .

qR

mx,r

mx,er      mxy1	          mxy1      mx,er

y                                                 q,Tri      Ly                                                    mx,m                      				    q                                                 mx,em              my,m                    mx,em

x                                                                1)   		       mxy2	         mxy2     	    2)

my,em						    Lx

The output of moments depends on the conditions of support: the moments mxer, mxem and myem

only are displayed if the corresponding edge is fixed ( i.e. those moments are unequal  “0” ) , see

sketch 2) .  The torsion moments ( “Drillmomente”) in the corners, mxy1 and mxy2, both occur if the left or right edges are simply supported ( mxy1) or if left (respectively right) and bottom edges are hinged (mxy2). As they both cause tension stresses on top and bottom of slab, they are indicated with the

“  “ - sign in the results.

Furthermore the ratio  = ly/lx  and informations on the support conditions and load input are shown.

- EXECUTION:

Transfer the library ( 1238 ) to the calculator and put it on the stack.

Enter the port ( “2” recommended ) you wish to store the library in on the stack and press “STO”.

- Warmstart the calculator by pressing “ON” and “C” simultaneously or execute  :port:1238  ATTACH.

- EXAMPLE:                                                                                        q,R = 0.5 kN/m

For the  system  of  sketch  3) all moments  have

to  be calculated.											Ly =										            q = 5.7 kN/m²           3.4																			     q,Tri = 22.7 kN/m²																			        3) 																									Lx = 4.0

Go to LIB and press the THREE-EDGES SLAB  -  key.

Then start the program with “slab3”   ( picture  4)  ).

As all three supports are simply supported by default, in our case softkeys “right”  and  “bottm”

( picture 5 ) have to be pressed to set these supports being fixed for bending moments as indicated in sketch 3).

Now you may change the default settings according to German Regulation DIN 1055-100 (dead load: ,g=1.35; imposed load: ,q=1.5) of the load factors. To do so enter ,g , “SPC” or “ENTER” ,  ,q

and press  “g     q”.

Then you have to key in the lengths:  4.0   “lx”   and   3.4   “ly” .

4)                                    5)

In the next display (picture 6)  enter the loads:  5.7  “q”  , 22.7  “q,Tri” and  0.5  “ q,R     ”. If there is

no edge load (q,R = 0 ), you have to press “q,R     ”  without any input to continue.

There is the option to enter the values separated in dead weight  q,g ( q,R,g, , qTri,g respectively) and imposed load q,p (q,R,p  ,  qTri,p ) , i. e.  q,g “ENTER” q,p. The values for the calculation of the moments

are determined considering the above mentioned safety factors by :

q  (q,R) (qTri) = ,g * q,g ( q,R,g ) ( qTri,g )    + ,q * q,p (q,R,p ) ( qTri,p )

In the present case the results look like this ( picture  7)  ):

6)                                 7)

New loads may be entered by softkeys  “q”  ,  “q,Tri”  or  “q,R    ”, respectively.

For other support conditions ( same lx, ly ! ) use softkey  “supp”.  That sets all edges to the default status ( simply supported !) Display of sketch  5) appears.

Press the appropriate keys for those edge(s) being clamped and after that  softkey  “lx” without any input.  Program then continues with screen 6) to enter loads.

For a new system press “new”.

To end calculation use key “exit”. That clears the subdirectory “SLDat” , restores flag status and

returns to “HOME”.

To print the results on an infrared printer 82240B press  “NXT”   “print”.

- RESTRICTIONS AND LIMITATIONS:

0.25 <= Ly/Lx <= 1.5  .    If  Ly/Lx  is beyond  these limits  a warning will be shown !

All moments are determined by sixth-degree interpolation polynomials and therefore show slight deviations compared to the tabled values in literature [1] .  All systems calculated up to now showed differences mostly less than 1.0 %  ( for small values in few cases up to 2% ) to the tabled values in [1] which is sufficient for practical purpose!

Note that the index of the moment  ( x and y, respectively)  indicate the direction of stress that is caused by the corresponding moment !

- COPYRIGHT AND DISCLAIMER OF WARRANTY:

All parts of this program are copyrighted by Claus Dachselt. It is provided as a free release for non-commercial purpose “as is” without warranty or responsibility of any kind. In no event  will the copyright holder be liable for damages arising out of the use or inability to use this program.

- VERSIONS   (three-edges-supported two-way slab):

- 3.1 : Inaccuracy in the display of explanations removed, variable load factors added  (04/2009).

- 3.0 : Calculation for triangle-shaped load added. Output of lengths and loads added. Print option included.

- 2.0:  Improved accuracy by now using sixth-order polynomials. Output of corner moments mxy1 and mxy2 added.  Additional option for  load input included.

- 1.1:  Improved polynomials for edge load q,R

- LITERATURE:

[1]  Stiglat / Wippel :  “Platten“  3rd edition ,  Verlag Ernst & Sohn ( Ernst & Sohn Publishers )

Claus Dachselt 									February  2009

e-mail: clan.dachselt@t-online.de