# Repeating Key Library

Synopsis:	Provides UserRPL access to the SystemRPL REPEATER and REPEATERCH.  Also accesses SystemRPL  commands which pause program execution until a keystroke is performed.  Finally, a command to update the menu.

Application:	HP48 series (S and G), HP49 (source code provided)

Type:	Auto-attaching library.  Library number is 1535.  562.5 bytes.  Note, since the library auto-attaches, access to these commands is available by typing their names and pressing ENTER.

## Stack Diagrams

Library name:	RepeatKey

Stack (before):		2):	program,  global program name, local program name

1):	real number (of keyboard key to cause repeat action)

Stack (after):		?? – user defines action

Description:	RepeatKey  executes a program taken from stack level two as long as a user-specified key is held down.  A slight pause between program executions is built-in.

Cautions:	Program to repeat must not alter stack!  For example:

<< DROP>>

15

RepeatKey

is not allowed and could cause a memory clear.

Library name:	RepeatKeyCH

This is identical to the above except there is no pause between program executions.

Library name:	GetKey

Stack (before):		this command does not require stack arguments

Stack (after):		real number value of key pressed (see description)

Description:	The calculator waits in a low-power state until a key is pressed.  Key values are returned as a real number, the number 1 returned for the top-row, leftmost menu key (A); the number 2 for the next key (B); and so on.  For example, the ENTER key returns 25, the TAN returns 21; the 3 key returns 43.  A key value is determined by counting keys from left to right and down the keyboard.  Three exceptions to this scheme are:

Key		Value

Alpha-on	128

Left-shift	  64

Right-shift	192

A useful benefit of GetKey is the action when the ON (Cancel) key is pressed.  UserRPL commands that pause for key presses and subsequently return a key value do not behave in this manner.  A program using UserRPL WAIT acts the same as GetKey until ON is pressed.  WAIT is terminated and the value –1 is returned.  This is useful to halt longer programs or clear operations.  But, if an attempt is made to use a screen prompt such as “Press any key to continue,” extra code is necessary to prevent the ON ending all program execution.  GetKey simply returns the value 45, therefore ideal for these applications.

Cautions:	Since no action takes place it’s not clear when the calculator is waiting for a key press.  The hourglass is not displayed, no messages appear, etc.  Once a key is pressed there’s feedback, but not before.  This does not cause any harm but may make for an odd question (“I press the GETK library key and nothing happens.  If I press it again, it returns 3 to the stack.”)  It was working exactly as intended, its waiting!

Left alone for long periods – through the normal HP48 inactivity timeout -  GetKey allows the calculator to shutoff.  The waiting continues after the calculator is turned back on.  In reality, this is quite useful.

Alarms are reported during GetKey.  Once acknowledged, key waiting continues.

Library name:	GetKeyRC

Stack (before):		this command does not require stack arguments

Stack (after):	real number value of key:  row_column.plane, exactly the same as returned by UserRPL commands.

Description:	Almost identical in action to GetKey except the value returned for a key press is the same as the value returned by the UserRPL WAIT command: the ENTER key returns 51.1; the TAN key returns 43.1; etc. This library command does not return a value if the alpha-on, left-shift, right-shift are pressed but does return a value for the ON key.  This makes it a better replacement for similar UserRPL commands.

Refer to GetKey for additional comments.

Library name:	DispMenu.1

Stack (before):		this command does not require stack arguments

Stack (after):		stack does not change but the current menu is displayed.

Description:	Menu updates are done in UserRPL executing programs only when a program pauses for keyboard input.  For example, a program designed to solve an involved computation could pause with a menu selection asking which function to apply and a withy screen message containing instructions.  UserRPL code works well in these applications, but once the program resumes – solving the equation – screens can display the programs progress but the menu is stuck where it was when input was taken.  Turning off the menu or changing its names would make a better status update for users.  An empty set {} and TMENU provides the update but it won’t display until the next pause for input.  Adding  DispMenu.1 after TMENU updates the screen immediately.

Cautions:	none known

Examples:

The following is a simple program to scan through a large list.  The list is assumed to contain many strings, such as names and phone numbers, but just as easily could contain numbers.  Instead of a list, a large array may need scanned.

<<	{ }   TMENU   DispMenu.1				/* show a blank menu immediately

DO

GetKey					/* wait for key

CASE

DUP 17 ==				/* the down cursor key

THEN

<< 	{ list of names }

INDEX		/* INDEX is a global variable

GET

3 DISP			/* display on row 3

‘INDEX’

INCR			/* increment value

DROP

>>

SWAP				/* swap key value to level 1

RepeatKeyCH			/* scan quickly

- /* signal DO to continue

END					/* end of case-then

DUP  11 ==				/* the up cursor key

THEN

<<	{ list of names }

INDEX

GET

3 DISP

‘INDEX’

DECR			/ * decrement value

DROP

>>

SWAP				/* swap key value

RepeatKeyCH

0				/* signal DO to continue

END

DUP 25 ==				/* enter key

THEN

DROP

{ list of names }		/* copy name to stack

INDEX

GET

0				/* signal DO to contiunue

END

45 ==					/* ON key pressed

THEN

- /* signal DO to end

END

1200 .05 BEEP		/* beep, not a valid key

0

END						/* end of case

UNTIL

END

>>

Note, this program is simplified to feature actions of library commands.  INDEX must contain a real number between 1 and the size of the list.  These safeguards are an exercise for the reader.

### Source Code

The complete source code is included as a separate download.