SUPER QUICK START CHEAT SHEET

Audio EQ Cookbook HP15c CE software pac

Pepin Torres, P.E. pepin[dot]torres[at]gmail[dot]com

Pre-requisites

Calculator in 15.2 mode (this program is huge, can’t be run in default mode.)

Registers

R9   : Sampling frequency in Hz

R.0 : Center/Cutoff frequency in Hz

R.1 : Q. If necessary, it can be calculated as a function of bandwidth via GSB B

R.2 : A. Can be calculated as function of Gain in dB and S via GSB A. If not needed, set to 1.0.

Run Main Biquad Coefficients Generation Program

GSB C

Ruturn Frequency Response at arbitrary frequency fx in Hz (Program C must be run first)

GSB D (place fx in x-register, desired filter type index (1-9) in y-register)

Memory Layout (after execution)

Matrix A [4 3] an coeffs

Matrix B [9 3] bn coeffs

Matrix C [3 2] z-n at fx

Matrix D [4 2] Σ  bnz-n

Matrix E [9 2] Σ anz-n

See user’s guide for definition of the output matrices and filter type indexing.

C, D and E are temporary and can be deallocated after to save space

Register states after execution of Prog C and D

Registers	Description

R9		Sampling Frequency Fs

R.0		Center/cutoff frequency f0

R.1		Quality Factor Q

R.2		Gain in linear form A

R.3		sin(ω0)

R.4		cos(ω0)

R.5		α

R.6		A-1

R.7		A+1

R.8		2·√A·α

R.9		Bandwidth

R6		fx/Fs

R7		Desired filter type row index for matrix A

R8		Desired filter type row index for matrix B