Interesting Serial Data Speeds

Asynchronous Speeds

50   Baudot

75   Baudot

110   TTY33; the old standby

134.5 IBM Selectrics

135   same as above

150

200

300   "it's so much faster than 110..."

600

Except for Bell 202A format (the 1200 bps one that no one used much),

speeds above 600 bps are communicated at 600 baud, with multiple bits

per symbol.

1 200   "boy, this is really fast..."

1 800

2 400   the speed modem that evey machine came with after

9 600 was available...

3 600

4 800

7 200

9 600   "at last, fast enough!"

14 400   but then came...

19 200   2 x 9 600

28 800   fastest base signal rate on a phone line

38 400   4 x 9 600

57 600   6 x 9 600

115 200   4 x 28 800

Synchronous Speeds

8 000   voice channel sample rate

56 000   US DS0

64 000   European DS0; ISDN B channel

128 000   2 x 64 000; 1/2 T1

144 000   ISDN 2B+D channel

192 000   3 x 64 000; 3/24 T1

230 400   AppleTalk LocalTalk

256 000   4 x 64 000; 1/6 T1

320 000   5 x 64 000; 5/24 T1

384 000   6 x 64 000; 1/4 T1

448 000   7 x 64 000; 7/24 T1

512 000   8 x 64 000; 1/3 T1

576 000   9 x 64 000; 3/8 T1

640 000   10 x 64 000; 5/12 T1

704 000   11 x 64 000; 11/24 T1

768 000   12 x 64 000; 1/2 T1

832 000   13 x 64 000; 13/24 T1

896 000   14 x 64 000; 7/12 T1

960 000   15 x 64 000; 5/8 T1

1 000 000   Corvus Omninet data rate

1 024 000   16 x 64 000; 2/3 T1

1 088 000   17 x 64 000; 17/24 T1

1 152 000   18 x 64 000; 3/4 T1

1 216 000   19 x 64 000; 19/24 T1

1 280 000   20 x 64 000; 5/6 T1

1 344 000   21 x 64 000; 7/8 T1; also 24 x 56 000

1 408 000   22 x 64 000; 11/12 T1

1 472 000   23 x 64 000; 23/24 T1

1 536 000   24 x 64 000; T1; IDSN Primary

1 544 000   DS1; T1 + 8 000 bps signalling

2 048 000   European DS1

3 000 000   original Ethernet data rate

4 000 000   4 Mbit token ring data rate

6 176 000   4 x DS1; DS2

10 000 000   Ethernet data rate

16 000 000   16 Mbit token ring data rate

43 232 000   7 x DS2; 28 x DS1; DS3

51 500 000   OC1

100 000 000   FDDI data rate

135 000 000   3 x DS3 + signalling

155 000 000   OC3

622 000 000   OC12