SPRING CONSTANTS EQUIVALENT TO RIGIDITY OF BEAM  v1.1

Name: SpringConst

The following program calculates for a one-field beam with modulus of elasticity E  and the moment of inertia  I  the equivalent spring-constant values ( vertical spring c-v and rotational spring c- ) at any point “a” of the beam’s length L.  The following supports are allowed:

1)   hinged/fixed at both ends (code-number for input:  0/1)

2)   fixed at one end and hinged at the opposite end ( cf. example )

3)   clamped at the left end only ( cantilever beam) ( Input 1 for left and -1 for right end ).

Any units for the input may be selected, but must be consistent to each other !

Installation:

Transfer “SpringConst”  to the calculator and select this file.

Example:                                    Which are the equivalent spring constants at this point for

the given beam?   ( Units in this example: [MN] and [m] )

L =  7,5

la = 3,1

I = 1.927E -4 m4; E = 210 000 MN/m2 MMMN/m2

lb = 4,4

1.)  Start  “Springcst()”

2.)  Input the following data into the corresponding fields ( EE stands for the “EE”-key ):

E = 210 000 [MN/m2]; I = 1.927 EE -4 [m4] ( = I of HEB 280 ) ; la = 3.1 [m]; lb = 4.4 [m];

left support condition  = 1 = fixed   (possible support condition left end: 0 / 1 = hinged /

fixed ); right supp. = 0 = hinged  (possible: 0/1/-1=hinged/fixed/not supported (cantilever

beam) )

(To enter negative exponents be sure to use the (+/-) – key! )

3.)    Result : E*I=40.467

L/R=fixed/hinged – a/b=3.1/4.4

C-v  : 13,2045416   (according to the selected units in [MN/m]

C-  : 59,451557     [MNm]

WARNING and DISCLAIMER: This program is provided as freeware without any kind of warranty, SO USE AT YOUR OWN RISK !

Claus Dachselt

clan.dachselt@t-online.de